| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Overall Network Architecture for IEEE 802.16n |  |
| Date Submitted | 2010-11-23 |  |
| Source(s) | Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon ETRI Jaehyuk Jang, Yeongmoon Son, Youngbin Chang, Kyungkyu Kim, Rakesh Taori Samsung Electronics | E-mail: ekkim@etri.re.kr scchang@etri.re.kr yb.chang@samsung.com ym1004.son@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “IEEE 802.16gman-10/0042,” in response to the agreement by the Gridman TG at session #70 for the 1st Teleconference |  |
| Abstract | This contribution provides proposed overall network architecture for the TGn |  |
| Purpose | To be discussed and adopted by TGn |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Overall Network Architecture for IEEE 802.16n

Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon

ETRI

Jaehyuk Jang, Yeongmoon Son, Youngbin Chang, Kyungkyu Kim, Rakesh Taori

Samsung Electronics

# Introduction

This document provides in response to the agreement by the Gridman TG at session #70 for teleconference for SARM to address the overall network architecture as an annex of SRD (i.e., IEEE 802.16gman-10/0038r4[1]) of TGn.

# Reference

- IEEE 802.16gman-10/0038r4, “802.16n System Requirements Documents,” November 2010.

# Proposed Text for Annex A. Overall Network Architecture of 802.16n System Requirement Document (SRD; IEEE 802.16gman-10/0038r4)

Note: RED color: the removal of existing IEEE C802.16gman-10/0060r1.

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

Note that editorial comments are highlighted in Yellow.

# Annex A. Overall Network Architecture

Link between devices

| HR-BS | HR-RS | HR-MS |  |
| --- | --- | --- | --- |
| No | Yes | Yes | HR-BS |
| No | No | No | ABS |
| No | No | No | BS |
| Yes | Yes (multi-hop) | Yes | HR-RS |
| No | No | No | ARS |
| No | No | No | RS |
| Yes | Yes | Yes | HR-MS |
| No | No | No | AMS |
| No | No | No | MS |

Comment: This table only refers to air interfaces between the specified functions.

Comment: if a device is capable of role change, this table shows the link after the role change.

Comment: Table does not consider backward compatibility – only 16n modes.

| HR-RS | ARS | RS |  |
| --- | --- | --- | --- |
|  |  |  | HR-BS |
|  |  |  | ABS |
|  |  |  | BS |
|  |  |  | HR-RS |
|  |  |  | ARS |
|  |  |  | RS |
|  |  |  | HR-MS |
|  |  |  | AMS |
|  |  |  | MS |

| HR-MS | AMS | MS |  |
| --- | --- | --- | --- |
|  |  |  | HR-BS |
|  |  |  | ABS |
|  |  |  | BS |
|  |  |  | HR-RS |
|  |  |  | ARS |
|  |  |  | RS |
|  |  |  | HR-MS |
|  |  |  | AMS |
|  |  |  | MS |

Table AA shows the IEEE 802.16n interfaces that are supported and those that are not required to be supported in the IEEE 802.16n. The interfaces between HR-stations (i.e., HR-BSs, HR-RSs, HR-MSs) indicate that an IEEE 802.16n protocol is to be used for supporting high reliability. The interfaces between two entities where one of them is HR-station and the other is non HR-stations (i.e., ABS, ARS, AMS, BS, MR-BS, RS, and MS) indicates that either IEEE 802.16m or IEEE 802.16-2009/802.16j protocol is to be used for backward compatibility. The interface involving between non HR-stations indicates the specific either IEEE 802.16m or IEEE 802.16-2009/802.16j protocol is used and is out of scope of IEEE 802.16n.

The usage of the interfaces described in Table AA is constrained as follows:

- An HR-MS may connect to aan HR-BS either directly, via an HR-RS, or via a forwarding HR-MS. At least two hops between an HR-BS and an HR-MS may be supported when the HR-MS is connected to an HR-BS via the HR-RS. an its the

- An AMS may connect to an HR-BS either directly or via an HR-RS. Furthermore, an AMS may connect to an HR-BS via one ARS. its the by

- An MS may connect to an HR-BS either directly or via an HR-RS. Furthermore, an MS may connect to an HR-BS via one or more RSs in multi-hop configuration. an its the by

Table AA – Interconnection between the entities and the protocol used.

(Note: This table only refers to air interfaces between specific network entities. If a device is capable of role change, this table shows the link after the role changes. “n/a” means the interface between two entities is not applicable to IEEE802.16 air interface.)

[Yellow highlighted interfaces are in scope of 16n protocol, blue highlighted interfaces are backward compatible with either 16m or 16-2009/16j, and the rest of the interfaces are out of scope of 16n.]

| HR-BS | HR-RS | HR-MS | ABS | ARS | AMS | BS | MR-BS | RS | MS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n/a | 16n | 16n | n/a | 16m | 16m | n/a | n/a | 16j | 16-2009 | HR-BS |
| 16n | 16n | 16n | 16m | n/a | 16m | n/a | 16j | 16j | 16-2009 | HR-RS |
| 16n | 16n | 16n | 16m | 16m | n/a | 16-2009 | 16-2009 | 16-2009 | n/a | HR-MS |
| n/a | 16m | 16m | n/a | 16m | 16m | n/a | n/a | 16j | 16-2009 | ABS |
| 16m | n/a | 16m | 16m | n/a | 16m | n/a | n/a | n/a | 16-2009 | ARS |
| 16m | 16m | n/a | 16m | 16m | n/a | 16-2009 | 16-2009 | 16-2009 | n/a | AMS |
| n/a | n/a | 16-2009 | n/a | n/a | 16-2009 | n/a | n/a | n/a | 16-2009 | BS |
| n/a | 16j | 16-2009 | n/a | n/a | 16-2009 | n/a | n/a | 16j | 16-2009 | MR-BS |
| 16j | 16j | 16-2009 | 16j | n/a | 16-2009 | n/a | 16j | 16j | 16-2009 | RS |
| 16-2009 | 16-2009 | n/a | 16-2009 | 16-2009 | n/a | 16-2009 | 16-2009 | 16-2009 | n/a | MS |

[-------------------------------------------------End of Text Proposal---------------------------------------------------]

Includes HR-MS acting as an HR-BS

Includes HR-MS/HR-BS acting as an HR-RS