| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal of General Requirements of 802.16n SRD |  |
| Date Submitted | 2011-01-12 |  |
| Source(s) | Ming-Tuo Zhou1, Liru Lu (Alina)1, Xin Zhang1, Masayuki Oodo2, Hiroshi Harada1,2 NICT 1: 20 Science Park Road, #01-09A/10 TeleTech Park, Singapore 117674 2: 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan. 239 0847 | E-mail: mingtuo@nict.com.sg; liru@nict.com.sg; zhangxin@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp |
| Re: | CfC for 802.16n SRD |  |
| Abstract | This provides proposed text of general requirements for 802.16n SRD. |  |
| Purpose | To be discussed and adapted by TGn for draft 802.16n SRD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Text Proposal of General Requirements of 802.16n SRD

Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Masayuki Oodo, Hiroshi Harada

NICT

# Introduction

In April 2010, Japan Ministry of Internal Affairs and Communications (MIC) specified 200 MHz band (170-205 MHz) for public broadband (PBB) use from July 2011 [1]. The 200-MHz PBB systems will be shared by multiple public agencies (users) such as fire departments, police agencies, local public authorities, and will provide multimedia communications. On contrast the current public systems are operated independently and can only provide voice communication since they are based on the narrow band systems. Figure 1 shows difference between the current and future public system in Japan.

Japan Association of Radio Industries and Businesses (ARIB) is responsible to develop a standard for the 200-MHz PBB systems and now a draft ARIB standard is prepared. The draft ARIB standard specifies the physical (PHY) layer and the media access control (MAC) layer of the radio equipment of “portable” base stations and mobile stations for the broadband wireless communication system using the 200 MHz band. It has two modes, Mode 1 and Mode 2. Mode 1 is subset of IEEE 802.16TM standard [2]. Mode 2 basically is based on IEEE 802.16TM standard, and changes some parameters.

The 200-MHz PBB systems can be viewed as subset of Public Protection and Disaster Relief (PPDR) systems, and clearly, they require higher-reliability. Furthermore, as the PBB systems will be based on 802.16 standards, they share much commonness with 802.16n-GRIDMAN systems. This stimulates need and interest to develop PHY and MAC protocols that are also applicable to the 200-MHz PBB system in 802.16n-GRIDMAN standard.

This document provides text proposal for 802.16n SRD based on commonness of 200-MHz PBB systems and 802.16n networks.

- Public wireless system of Japan before July 2011

- Public wireless system in Japan after July 2011

Fig. 1 Current and expected public wireless systems in Japan [3]

- VHF frequency allocation plan after July 2011 in Japan

- Frequency allocation of 200-MHz for PBB systems

Fig.2 Frequency allocation plan for VHF including 200-MHz Band in Japan after July 2011 [3].

# Frequency allocation of 200 MHz band in Japan

As shown in Fig. 2, the 200-MHz band spanning 170-205 MHz will be used for PBB application after July 2011 [1]. The 35MHz-wide band is divided into 6 channels, each of 5MHz width. This is consistent of the channel width of 5 MHz in 802.16 systems.

# Requirements with view of 200-MHz PBB systems

1. Operation frequency band

In order to be applied in 200-MHz PBB systems, operation in 170-205MHz is required for HR-stations.

2. Low Downlink to Uplink Ratio

In use of public safety and disaster recovery, in some cases the uplink requires much higher bandwidth than downlink for transmission. For example, the uplink is used to transmit real-time video to command center, and the downlink transmits only commands and orders to the accident/disaster field. This requires the HR-stations to support low downlink to uplink (DL/UL) ratio (D:U), to allow the uplink to acquire higher bandwidth than downlink to support higher uplink data transmission. In 802.16m, the lowest D:U is 3:5 [6], which may not be able to meet requirement for uplink video transmission in some cases. In WiMAX the lowest D:U is 26:21, which is not low enough.  IEEE 802.16n networks may require lower DL/UL ratio. For example, with 5 MHz channel width, a D:U as low as 9:38 can support a uplink transmission date rate of 7.4 Mbps (if use 16QAM-CC3/4 modulation-coding scheme), which is able to transmit certain kinds of HD video.

3. Interference mitigation

In both 200-MHz PBB systems and 802.16n networks, mobile base station is supported and then several cells may meet and have overlap in coverage. This requires self-coexistence of HR-stations to reduce interference among HR-stations associated to different cells.

# Text Proposal for draft 802.16n SRD

Note:

The text in BLACK color: the existing text

The text in BLUE color: the new text added to the existing text

--------------Start of Proposed Text---------------------------------------------------------------------

# 3. Definitions

|  | Other definitions |
| --- | --- |
| Self-coexistence | A state of acceptable co-channel and/or adjacent channel operation among HR-stations within the same geographical area |

## 5.4 Operating Frequencies

The HR-Network shall be specified to allow operation in all radio frequencies where 802.16 operates.

The HR-Network shall also be specified to allow operation in unlicensed and lightly licensed spectrum bands below 6 GHz with means and mechanisms to coexist with other radio access technologies (RATs).

As an example for frequencies used for PPDR, the following frequency bands have been identified by WRC 2003 (RESOLUTION 646 (WRC-03): Public Protection and Disaster Relief, The World Radiocommunication Conference (Geneva, 2003).)

For region 1

- 380-385 MHz

- 390-395 MHz

For region 2

- 746-806 MHz

- 806-869 MHz

- 4,940-4,990 MHz

For region 3

- 406.1-430 MHz

- 440-470 MHz

- 806-824 MHz

- 851-869 MHz

- 4,940-4990 MHz

- 5,850-5,925 MHz

One more example is 170-205 MHz that is specified by Japan Ministry of Internal Affairs and Communications (MIC) in 2010 for public broadband applications in Japan [1].

## 6.1.5 Coexistence requirements

6.1.5.1 Operation in unlicensed and lightly licensed bands

HR-Stations shall comply with regulators’ respective requirements for operation in unlicensed and lightly licensed spectrum.

6.1.5.2 Support for Multi-carrier operation in different licensing regimes

The HR-Network shall support multicarrier operation in licensed, unlicensed and lightly-licensed licensing environments.

An HR-MS that supports MC operation in different licensing environments shall be able of operating in all three types of spectrum at the same time.

6.1.5.3 Coexistence mechanism

HR-Network shall support self-coexistence mechanism [8].

6.2.3 Priority Access Service

The HR-Network MAC shall be able to support a priority access service for ETS (Emergency Telecommunications Services) and other priority applications

### 6.2.4 Uplink Heavy Data Service

The HR-Network MAC shall support uplink heavy data service for supporting uplink video steaming for surveillance, PPDR and other applications. To support this service, low downlink to uplink (DL/UL) ratio is required. For example, when channel bandwidth is 5 MHz, at least 9:38 (DL/UL ratio) in WirelessMAN-OFDMA based system or 2:6 (DL/UL ratio) in WirelessMAN-Advanced Air Interface based system is required [7].

7.2.2 Multicast connection setup time

Multicast connection setup time is the required time for HR-MS to establish a multicast connection with a group of users. The value of multicast connection setup time is only the transmission time on the MAC layer signaling procedures excluding upper layer signaling procedures. The maximum multicast connection setup time is 150 ms in a single hop.

--------------End of Proposed Text ---------------------------------------------------------------------

# References

- Japan Ministry of Internal Affairs and Communications (MIC) “The Ordinance Regulating Radio Equipment” (ORE） Article 49-31 (in Japanese)

- IEEE Std 802.16TM-2009, IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems

- Masayuki Oodo, Noriyuki Soma and Hiroshi Harada, “Radio Propagation experiments for broadband wireless communication system in the VHF band”, The 11th International Symposium on Wireless Personal Multimedia Communications (WPMC 2008), Lapland, Finland

- IEEE 802.16gmain-10_0038r4, “(Draft) 802.16n System Requirements Document,” 2010-11-10.

- Liru Lu, Xin Zhang, Ming-Tuo Zhou, Masayuki Oodo, Hiroshi Harada, “Japanese Public Safety Use Case Model for 802.16n”, IEEE C80216-10/0067.doc

- IEEE P802.16m/D9, IEEE DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems

- Ming-Tuo Zhou, Masayuki Oodo, Liru Lu (Alina), Xin Zhang, Hiroshi Harada, Uplink Heavy Data Service Requirement in 802.16n for surveillance, PPDR, and other Applications, IEEE C80216n-10/0074

- Ming-Tuo Zhou, Masayuki Oodo, Xin Zhang, Liru Lu, Hiroshi Harada, Self-Coexistence Mechanism Requirement in 802.16n, IEEE C80216n-10/0075