| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast Key Management for IEEE 802.16n HR-Network without Infrastructure |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Joseph Chee Ming Teo, Jaya Shankar, Yeow Wai Leong, Hoang Anh Tuan Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2292 E-mail: cmteo@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this document, we propose multicast key management procedures for Initial Group Formation, Node Join events and Node leave events to secure multicast communications in 802.16n HR-networks. We focus on the situation where there is no backbone infrastructure and all multicast group members possess a private/public key pair and public key certificate (X.509 certificate) signed by a certification authority (e.g. Manufacturer), that is, we assume that there is X.509 PKI infrastructure in place. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multicast Key Management for IEEE 802.16n HR-Network without Infrastructure

Joseph Chee Ming Teo, Jaya Shankar, Yeow Wai Leong, Hoang Anh Tuan

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

# The IEEE 802.16n System Requirements Document (SRD) [1] specifies shall provide the security architecture that provides a group of HR-MSs with authentication, authorization, encryption and integrity protection. The HR-Network shall provide multicast key management for the group of HR-MSs and the key shared within the group should be distributed securely and efficiently. The multicast communications should be able to take place with or without Base Station (HR-BS) in order to provide high reliability.

# To ensure that an attacker is not able to masquerade as a multicast member or eavesdrop in the multicast communications, multicast key management (MKM) protocols have to be designed for the 802.16n networks. Although multicast is already supported in existing IEEE 802.16 standards and a Multicast and Broadcast Rekeying Algorithm (MBRA) has already been proposed, however, it was pointed out in existing literature that MBRA is not scalable and also does not provide backward and forward secrecy, i.e. a new joining member is able to decrypt previous secure multicast communications sent before it joined (backward secrecy) and a leaving member can still decrypt future secure multicast communications after leaving the multicast group (forward secrecy).

# In this contribution, we focus on the situation where there is no backbone infrastructure and all multicast group members possess a private/public key pair and public key certificate (X.509 certificate) signed by a certification authority (e.g. Manufacturer), that is, we assume that there is X.509 PKI infrastructure in place. We propose multicast key management procedures for Initial Group Formation, Node Join events and Node leave events to secure multicast communications in 802.16n HR-networks.

# 2. Multicast Key Management

Figure 1 shows a group of multicast members that wishes to establish a secure multicast. The “controller” node HR-MSX/BS refers to either an appointed HR-MS node (HR-MSX) in the case where there is no Base Station (HR-BS) around or it can also refer to the Base Station node (HR-BS) in the case where an HR-BS is present. In this contribution, it is assumed that there is no backbone infrastructure and that all nodes possess a private/public key pair and public key certificate (X.509 certificate). In the case of Figure 1, an Initial Group Formation Procedure will be executed to establish a secure multicast key GTEK for secure multicast communications as well as pairwise shared keys MSKi that will be shared between the controller node and each HR-MSi in the multicast group.

Controller node can be either HR-BS or an appointed HR-MS (Denoted as HR-MSX if HR-BS is not present)

Figure 1: Initial Group Formation

Figure 2 below shows the multicast group where new node HR-MSj joins the multicast group. In this case, the existing multicast group key has to be changed in order to prevent the new node from reading previous secure multicast communications (backward secrecy). The Join Event Procedure will also establish a pairwise shared key between controller node and the new joining node HR-MSj.

Figure 2: Join Event

Figure 3 below shows the scenario where a multicast member HR-MSl leaves the multicast group. In this case, the multicast group key has to be changed to prevent the leaving multicast member from obtaining future secure multicast communications (forward secrecy).

Figure 2: Leave Event

As such, the Multicast Key management protocol in the IEEE 802.16n standards has to provide procedures to update the key whenever the multicast group is formed or whenever a user joins or leaves the multicast group.

# 3. Summary

Suppose a group of HR-MSs (High-Reliability-Mobile Stations) wishes to communicate securely with each other for multicast applications such as Push-to-Talk or multicast data transfer, then a multicast group has to be formed. In order to prevent malicious nodes from masquerading as a legitimate multicast member and send malicious message to disrupt the multicast communications, multicast key management (MKM) protocols has to be used to establish a multicast key for secure multicast communications in the group. The data encrypted with the multicast key will also be secure against eavesdropper trying to listen in to the secure multicast communications.

The MKM protocols also has to take into account backward and forward secrecy, i.e. i.e. a new joining member is able to decrypt previous secure multicast communications sent before it joined (backward secrecy) and a leaving member can still decrypt future secure multicast communications after leaving the multicast group (forward secrecy).

# 4. Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the AWD Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n AWD Document (C802.16x-xx/xxxx)]

# 17.2.10.2 Multicast Key Management

## 17.2.10.2.1 Multicast Key Management – PKI

The following procedures shall be used to 1) establish pairwise key MSKi between controller node and HR-MS nodes and the multicast key GTEK for a group of HR-MS nodes to establish secure multicast communications with each other, 2) change the multicast key whenever new nodes join the multicast group and establish pairwise key MSKj with the new nodes and 3) change the multicast key whenever nodes leave the multicast group. The HR nodes shall use PKI to perform authentication and multicast key management.

### 17.2.10.2.1.1 Initial Group Formation Procedure

The Initial Group Formation procedure takes place during initial multicast group formation to establish a pairwise key MSKi between the controller node HR-MSX/BS and multicast members HR-MSi as well as establish a multicast key GTEK for secure multicast communications amongst the HR-MS nodes in the multicast group. Figure X1 shows the flow diagram while Figure Y1 shows the flow chart for the Initial Group Formation Procedure.

The Initial Group Formation Procedure includes the following steps:

Step 1: …

Step 2:  …

### 17.2.10.2.1.2 Join Event Procedure

The Join Event Procedure takes place whenever there are nodes that join the multicast group and the multicast key has to be changed in order to prevent the new joining nodes from obtaining previous secure multicast communications. A pairwise key MSKj will also be established between controller node HR-MSX/BS and new joining node HR-MSj. Figure X2 shows the flow diagram while Figure Y2 shows the flow chart for the Join Event Procedure.

The Join Event Procedure includes the following steps:

Step 1: …

Step 2:  …

### 17.2.10.2.1.3 Leave Event Procedure

The Leave Event Procedure takes place whenever there are nodes leaving the multicast group and the multicast key has to be changed in order to prevent the leaving nodes from obtaining future secure multicast communications. Figure X3 shows the flow diagram while Figure Y3 shows the flow chart for the Leave Event Procedure.

The Leave Event Procedure includes the following steps:

Step 1: …

Step 2:  …

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.