| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A Proposal of Self-Coexistence Mechanism for 802.16n-GRIDMAN Stations |  |
| Date Submitted | 2011-03-16 |  |
| Source(s) | Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada NICT Yih-Guang Jan, Yang-Han Lee Tamkang University (TKU) Hsien-Wei Tseng De Lin Institute of Technology (DLIT) | E-mail: mingtuo@nict.com.sg; zhangxin@nict.com.sg; liru@nict.com.sg; hvdien@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp yihjan@yahoo.com hwtseng@mail.dlit.edu.tw |
| Re: | Call for contributions for the 802.16n Amendment Working Draft |  |
| Abstract | This provides a proposal of self-coexistence for interference mitigation in 802.16n networks |  |
| Purpose | To be discussed and adapted by TGn for draft 802.16n AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# A Proposal of Self-Coexistence Mechanism for 802.16n-GRIDMAN Stations

Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

NICT

Yih-Guang Jan, Yang-Han Lee

Tamkang University (TKU)

Hsien-Wei Tseng

De Lin Institute of Technology (DLIT)

# Introduction

Figure 1 Example of 802.16n deployment.

In SRD of IEEE 802.16 GRIDMAN [1], it is required that the HR networks shall support mechanisms which enable acceptable operation in the presence of co-channel interference among the HR stations within the same geographical area. The specification is as following:

“6.1.5 Coexistence requirements

6.1.5.3 Interference Mitigation

HR-Network shall support mechanisms which enable acceptable operation in the presence of co-channel interference among HR-stations within the same geographical area.”

As depicted in Fig. 1, co-channel interference at an HR cell is introduced by an overlapping cell operating on same frequency channel simultaneously. Especially, GRIDMAN will operate in VHF bands like 200 MHz [1]. In these bands, the interference is much longer when compare to GHz frequencies. Moreover, as in GRIDMAN mobile BS is supported, meaning that the cells are possible mobile and two or more of them may overlap in most of coverage if they move very close. This is quite a different case when compare to a cell with a fixed BS.

# This document presents mechanism of self-coexistence to reduce co-channel interference in HR networks. In this mechanism, Self-coexistence Beacon (SCB) is used to exchange information for purpose of self-coexistence. SCBs are transmitted in Self-coexistence Zone (SCZ), which is a zone allocated at the end of a GRIDMAN frame. A virtual Superframe consists of 16 frames as described in [2].

# Basic Concepts

# Coexistence

Coexistence is a state by which multiple wireless communications systems in same vicinity share a same radio frequency channel while minimizing harmful interference to each other by appropriate measures.

# Self-coexistence

In HR network, Self-coexistence is coexistence of multiple HR cells.

# Self-coexistence Mechanism

# Self-Coexistence Zone

A Self-coexistence Zone (SCZ) is a space allocated at the end of a GRIDMAN frame as illustrated in Fig. 2. At the beginning of each self-coexistence zone, self-coexistence preamble (SCP) is transmitted. Following SCP is space for transmission of self-coexistence packets. The structure of a SCZ is shown in Fig. 2. A Self-coexistence Zone can be shared by multiple HR cells via contention or reservation.

Figure 2 Self-coexistence Zone for GRIDMAN.

# Self-coexistence Preamble

Self-coexistence Preamble shall be transmitted at the beginning of each Self-coexistence Zone. It is different from other preambles of HR networks.

# Self-coexistence Beacon

Self-coexistence Beacon is a series of message bursts transmitted by HR stations in Self-coexistence Zone to enable Self-coexistence in HR networks. The transmission of Self-coexistence Beacons can be controlled by HR-BS or be scheduled in a distributed manner.

Figure 3 Structure of Self-Coexistence Beacon Packet for IEEE 802.16n-GRIDMAN.

Self-Coexistence Beacon can be used for neighbor discovery among stations of a same cell and for neighboring network discovery among stations if network information is carried. It can be used for HR-MS direct communications.

# Self-coexistence Beacon Packet

The structure of a Self-Coexistence Beacon packet is shown in Fig. 3. It contains a Self-coexistence Preamble which is different from other preambles of HR networks. After the Self-coexistence Preamble, Self-coexistence (SC) MAC PDU shall be transmitted. The SC MAC PDU includes Schedule of the Self-Coexistence, SCB IEs.

# Self-coexistence Mode

In self-coexistence mode, multiple HR cells share the same frequency channel and each coexisting HR cell operates on one or several different frames.

Figure 4 Illustration of self-coexistence mode (a virtual superframe consists of 8 self-coexistence zones as example. The frames assigned to different cells can be interleaved to reduce delay for packets transmission in a cell.).

In IEEE 802.16m, fractional frequency reuse (FFR) is used to mitigate co-channel interference between several cells with overlapped coverage operating on same frequency channel. In this scheme, MSs at the edge of two or more cells use different parts of OFDMA subchannels, while MSs close to BS use full OFDMA subchannels with less power. However, IEEE 802.16m operates in GHz bands, while GRIDMAN may operate in VHF bands. In VHF bands, due to the much longer interference distance, this scheme may become not effective as the interference distance of an MS close to a BS may reach the edge of the cell and introduce co-channel interference to MSs of another cell. Moreover, due to mobility of HR-BS, the HR cells are mobile and two or more HR cells may move very close and overlap in most of their coverage. In this case, FFR will not work.

# Text Proposal for IEEE 802.16n AWD

--------------Start of Proposed Text --------------------------------------------------------------------

17.1 Overview

17.1.x Definitions

Coexistence: Coexistence is a state by which multiple wireless communications systems in same vicinity share a same radio frequency channel while minimizing harmful interference to each other by appropriate measures.

Self-coexistence: In HR network, Self-coexistence is coexistence of multiple HR cells.

17.1.x Operation Modes

HR network can run in two modes: normal mode and coexistence mode. A HR cell runs in normal mode by default and transmits to self-coexistence mode when the HR cell can detect and decode a Self-Coexistence Beacon from an adjacent HR cell on its operating channel. In normal mode, one HR cell occupies one frequency channel and operates on all the frames of a virtual superframe. In self-coexistence mode, multiple HR cells share the same frequency channel and each coexisting HR cell operates on one or several different frames of a virtual superframe.

--------------End of Proposed Text ---------------------------------------------------------------------

# Reference

[1] 802.16n System Requirements Document including SARM annex, IEEE 802.16n-10/0048

[2] A Proposal of Frame Structure for 802.16n-GRIDMAN Networks, IEEE 802.16n-10/0013r1