| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A Proposal of HR-MS Direct Communication in IEEE 802.16n-GRIDMAN Network |  |
| Date Submitted | 2011-03-14 |  |
| Source(s) | Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, HOANG Vinh Dien, Masayuki Oodo, Hiroshi Harada | E-mail: mingtuo@nict.com.sg; liru@nict.com.sg; zhangxin@nict.com.sg; hvdien@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp |
| Re: | Call for contributions for the 802.16n Amendment Working Draft |  |
| Abstract | This provides a proposal of self-coexistence for interference mitigation in 802.16n networks |  |
| Purpose | To be discussed and adapted by TGn for draft 802.16n AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# A Proposal of HR-MS Direct Communication in IEEE 802.16n-GRIDMAN Network

Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Masayuki Oodo, Hiroshi Harada

NICT

# Introduction

HR-MS shall provide direct communication, i.e., the origination and termination of the data are at the HR-MS. This document presents proposal of facilities to support HR-MS direct communication in IEEE GRIDMAN network, including Direct Communication Zone and scheduling method.

# HR-MS to HR-MS Direct Communication

## Hops of HR-MS Direct Communication

(a)                                 (b)

Figure 1 Example of one-hop and two-hop HR-MS to HR-MS direct communication.

In SRD of IEEE 802.16n-GRIDMAN, HR-MS direct communication is specified in Section 6.1.3.1 [1]:

#### “2.1.1.1 MS to MS Direct Communication

HR-MS shall provide direct communication i.e. the origination and termination of the data are at the HR-MS. Association establishment procedure of an HR-MS to another HR-MS shall be supported.

##### 2.1.1.1.1 Single hop

HR-MS shall support communication of user data and control signaling between an HR-MS and one or more HR-MSs that are 1-hop away.

##### 2.1.1.1.1 Two-hop

HR-MS shall support forwarding of user data and control signaling between an HR-MS and one or more HR-MSs.”

Based on the above specifications, HR-MS to HR-MS direct communication (HR-MS DC) is categorized into one-hop HR-MS DC and two-hop HR-MS DC, as shown in Fig. 1.

In one-hop HR-MS DC, an HR-MS transmits or receives data packets to or from another HR-MS (or more than one HR-MS) directly, without relaying on or passing through other stations.

Two-hop HR-MS DC is that an HR-MS communicate with another HR-MS (or more HR-MSs) by user data and control signaling forwarding provided by a middle HR-MS.

## Network topology of HR-MS direct communication

HR-MS direct communication can happen in case of the HR-network has infrastructure stations as well as has no infrastructure stations.

## Scheduling of HR-MS direct communication

### Direct Communication Zone

A DCZ is a space of continuous subchannels along frequency and continuous symbols along time in a frame allocated for HR-MS direct communication [2].

DCZ is for priority transmission of HR-MS direct communication for purpose of QoS provision and mitigating transmit/receive transition time of HR-MSs. Its size is not fixed but can be configured dynamically.

HR-MS direct communication has higher priority being scheduled in a DCZ than downlink and uplink communication for purpose to provide QoS support of HR-MS direct communication.

An HR-MS DC can be scheduled in space other than DCZ (like normal uplink and downlink communication space) if the current DCZ is occupied by other HR-MS DC.

### Scheduling Scheme

An HR-MS DC can be scheduled in two ways: centralized scheduling and distributed scheduling. In centralized scheduling, an HR-BS schedules the HR-MS DC transmission. In distributed scheduling, HR-MSs involved in the direct communication and their neighbors schedule the transmission in a distributed manner.

Centralized scheduling can be used for one-hop and two-hop HR-MS DC transmission. Distributed scheduling can only be used for one-hop scheduling.

For two-hop HR-MS DC, in centralized scheduling, an HR-BS can assign resources for the first hop and second hop HR-MS DC by a single action. Through this way, delay in resource assignment for two-hop HR-MS DC can be saved compared to assign resource step by step.

## Direct Communication MAP Message

The Direct Communication MAP (DC-MAP) message is to signal resource assignment and other control information for HR-MS to HR-MS direct communication. The resource assignment is not limited in Direct Communication Zone since HR-MS to HR-MS direct communication is not limited to be scheduled in Direct Communication Zone.

# Text Proposal for IEEE 802.16n AWD

--------------Start of Proposed Text --------------------------------------------------------------------

17.x.2 Direct communication between HR-MSs

HR-MS direct communication can happen in case of the HR-network has infrastructure stations as well as has no infrastructure stations.

HR-MS direct communication has priority to be scheduled in direct communication zone for QoS provision and mitigating transmit/receive transition time of HR-MSs. It also can be scheduled out of direct communication zone when the present direct communication zone is occupied by other HR-MS direct communications with same or higher priority.

The Direct Communication MAP (DC-MAP) message is to signal resource assignment and other control information for HR-MS direct communication if it is under control of infrastructure stations.

17.x.2.y Hop number of HR-MS direct communication

HR-MS direct communication (HR-MS DC) is categorized into one-hop HR-MS DC and two-hop HR-MS DC.

In one-hop HR-MS DC, an HR-MS transmits or receives data packets to or from another HR-MS (or more than one HR-MS) directly. In two-hop HR-MS DC, an HR-MS communicate with another HR-MS (or more HR-MSs) by user data and control signaling forwarding provided by a middle HR-MS.

Both one-hop and two-hop HR-MS DC can be scheduled by centralized scheduling that is controlled by an HR-BS.

--------------End of Proposed Text ---------------------------------------------------------------------

# Reference

[1] 802.16n System Requirements Document including SARM annex, IEEE 802.16n-10/0048

[2] Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Masayuki Oodo, Hiroshi Harada, A Proposal of Frame Structure for 802.16n-GRIDMAN Networks, IEEE C802.16n-11/0013r1