| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Enhanced Local Forwarding Support for Multicast Service for the IEEE 802.16n Networks |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Choong Hock Mar, Shoukang Zheng, Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Wai Leong Yeow. Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2256 E-mail: hwang@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this document, we propose an enhancement of the local forwarding method in (Proposal_No) so as to more efficiently support 2-way multicast service such as Push-To-Talk Service. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Enhanced Local Forwarding Support for Multicast Service for the IEEE 802.16n Networks

Choong Hock Mar, Shoukang Zheng, Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, and Wai Leong Yeow.

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

Scenarios may arise where the data packet of a HR-MS needs to be multicast to a group of HR-MS under the control of the same HR-RS. In this proposal, we refer the source HR-MS as HR-MS1 and the destination HR-MS as HR-MS2. One possible application is the PTT (Push-To-Talk) service as shown in Fig. 1. The data flow for the normal operation of the PTT service is inefficient because of repeated data retransmissions from the controlling HR-RS to the PTT server and back again to the HR-RS. The matter is made worst if the PTT server sits in the backbone. Based on the SRD requirement [1] for multicasting, we propose an enhancement to the local forwarding method in (Proposal number) for the PTT service by providing local forwarding of multicast packets.

# 2. Local Forwarding Support for Multicast Service for the IEEE 802.16n Network

Work In Progress.

# 3. Summary

We propose further enhancements to the local forwarding mechanism in () so as to support Multicast Service. This is based on the SRD [1] requirement for enhanced MAC support for 2-way multicast service such as Push-To-Talk (PTT). In our proposal, we provide the procedures for setting up the local forwarding of multicast packets and the handover situations. We propose that:

- HR-BS has a table that stores all the source-destination IP addresses of all uplink data connection that is marked for PTT service.

- A one bit field (PTT flag) is included in the DSA-REQ packet to help the HR-BS identified that the connection request is for PTT service.

In our proposal, the followings remain the same and are unaffected:

- The upper layer control signaling used in the PTT service

- Both the BS and RS initiated binding and unbinding procedures for local forwarding in the unicast case ()

- The DSx procedures to set up the multicast and Enhanced Multicast Broadcasting Service (EMBS)

# 4. Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

The following text is to be inserted into 17.3.9 Multicast support

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16m MC-DG Document (C802.16m-D11)]

17.3.9.4  Local Forwarding of Multicast Packets

17.3.9.4.1  Support for setting up the Local Forwarding of Multicast Packets

Multiple local forwarding opportunities may arise when a source HR-MS and the destination multicast sub-groups share common HR-RSs (or common upstream HR-RSs in the multi-hop scenario).  An example is the 2-way Push-To-Talk (PTT) service. In such a scenario HR-BS can coordinate and establish multiple local forwarding among the source HR-MS and various multicast sub-groups under its control. The upper layer signaling between the PTT server and its clients such as the request and grant of Right-To-Transmit and Transmit-Release are beyond the scope of the standard. The following support shall be provided. After the HR-MS receives the grant of Right-To-Transmit from the PTT server, it shall send a DSA-REQ with the PTT flag set to 1 to establish a uplink unicast connection. Upon receiving the DSA-REQ with the PTT flag set to 1, HR-BS saves the IP address of the source and destination of the uplink connection created in the table. Subsequently, when the HR-BS first receives a IP multicast packet such that the source IP address matches the destination IP address in the table (which is the IP address of the PTT server), it shall determine whether local forwarding and perform bindings as explain in section 17.2.6 on local forwarding. Note that the local forwarding binding is done separately for different Multicast sub-groups. Also, the creation of the multicast downlink shall follow the procedure outlined in this section.

17.3.9.4.1  Handover procedure for source HR-MS when it moves

The handover procedure is the same as the unicast case for local forwarding in section 17.2.6

17.3.9.4.2  Handover procedure when one of the multicast member moves

The handover procedure will depend on whether it affects the existing local forwarding bindings and there are four cases.

Work In Progress.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.