| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HR-MS Neighbor Discovery in 802.16n |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Zhongding Lei, Shoukang Zheng Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2256 E-mail: hwang@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this document, we discuss approaches for HR-MS neighbor discovery in 802.16n |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# HR-MS Neighbor Discovery in 802.16n

Haiguang Wang, Hoang Anh Tuan, Jaya Shankar,

Zhongding Lei, Shoukang Zheng

Institute for Infocomm Research (I2R)

# 1. Introduction

In the 802.16n System Requirement Document [1], Section 6.1.3.3, it is stated that “HR-Network shall support neighbor and path discovery between HR-MSs as well as between HR-MS and HR-Network infrastructure stations”. In fact, neighbor discovery by HR-MS will be one of the enabling mechanisms to support other advanced features of 802.16n such that HR-MS direct communications, HR-MS forwarding to network, path discovery and management, and fast network recovery.  This contribution discusses HR-MS neighbor discovery under two scenarios, i.e., when registered HR-MSs discover their neighbors, and when out-of-coverage HR-MS discover network infrastructure via the help of a connected HR-MS.

# 2. Design Considerations

When designing HR-MS neighbor discovery, the following criteria should be taken into account:

- Minimizing changes to the existing 802.16 specifications;

- Reliable but incurring low overhead and complexity;

- Allowing flexible scheduling and configuration.

2.1 Neighbor Discovery for Registered HR-MSs

To discover each other, registered HR-MSs can broadcast some predefined signals so that other HR-MSs can try to detect and confirm whether or not they are within the communication range. Here, the predefined signals can be preamble sequences, which are assigned to HR-MSs by HR-BS/HR-RS.

Upon detecting one or a number of neighbors, an HR-MS shall report the information to its serving HR-BS (HR-RS).

2.2 HR-MS Discover Network Infrastructure (Coverage Extension)

As specified in the 802.16n SRD [1], HR-MS network is required to support local forwarding (illustrated in Figure 1). To enable an HR-MS that is outside the coverage of an HR-BS to discover network infrastructure and get connected, a nearby HR-MS at edge of a cell can transmit preambles, together with network configuration information for the far-away HR-MS to start network entry. At the same time, when the cell-edge HR-MS transmit such a preamble signal, it also allows other registered HR-MS to scan and discover their relationship. Part of the radio band may be use by other HR-MS to receive ranging sequences from other HR-MSs who want to access the network.

Figure 1: Neighbor discovery method supports HR-MS forwarding

# 3. Text Proposal for IEEE 802.16n AWD

Xxx

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n Document (XXX places)]

17.2.7 Path Discovery and Management

17.2.7.1 Neighbor Discovery for Registered HR-MS

To discover each other, registered HR-MSs can broadcast some predefined signals so that other HR-MSs can try to detect and confirm whether or not they are within the communication range. Here, the predefined signals can be preamble sequences, which are assigned to HR-MSs by HR-BS/HR-RS.

Upon detecting one or a number of neighbors HR-MSs, an HR-MS shall report the information to its serving HR-BS (HR-RS). Based on reported information, HR-BS/HR-RS may request the HR-MSs to carry out extra scanning and measurements so that other activities such as HR-MS direct communications and HR-MS forwarding to network can be initiated.

17.2.7.2 HR-MS Discover Network Infrastructure

To enable an HR-MS that is outside the coverage of an HR-BS to discover network infrastructure and get connected, a nearby registered HR-MS at cell edge can transmit preambles, together with network configuration information, for the far-away HR-MS to start network entry. OFDMA resource shall also be allocated for the out-of-coverage HR-MS to carry out ranging and other necessary steps with the cell-edge HR-MS so that it can eventually join the network. At the same time, when the cell-edge HR-MS transmits preamble signal (for coverage extension), other registered HR-MSs can also scan to discover their neighbor relationship.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.