| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HR-MS to HR-MS Synchronization in 802.16n |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Hoang Anh Tuan, Haiguang Wang, Jaya Shankar, Joseph Chee Ming Teo, Wai Leong Yeow, and Shoukang Zheng Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | We discuss approaches to achieve synchronization between HR-MSs so that HR-MS direct communication and HR-MS forwarding to network can be supported in 802.16n. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# HR-MS to HR-MS Synchronization in 802.16n

Hoang Anh Tuan, Haiguang Wang, Jaya Shankar,

Joseph Chee Ming Teo, Yeow Wai Leong, and Shoukang Zheng

Institute for Infocomm Research (I2R)

# 1. Introduction

The IEEE 802.16n System Requirement Document (SRD) [1] mandates the requirements to support HR-MS direct communications as well as HR-MS forwarding to network. In both operations, one of the key issues is to enable HR-MSs to achieve frame-level synchronization and OFDMA-symbol-level synchronization so that they can reliably transmit to and receive from each other. This contribution discusses approaches to achieve such levels of synchronization and proposes text to be adopted into 802.16n AWD.

# 2. Use Cases

- 2.1. HR-MS Direct Communications

- As specified in Section 6.1.3.1 of 802.16n SRD [1]:

- “HR-MS shall provide direct communication i.e. the origination and termination of the data are at the HR-MS. Association establishment procedure of an HR-MS to another HR-MS shall be supported.

- HR-MS shall support communication of user data and control signaling between an HR-MS and one or more HR-MSs that are 1-hop away.

- HR-MS shall support forwarding of user data and control signaling between an HR-MS and one or more HR-MSs.”

The above specifications cater for two main use cases, when both HR-MSs are under control of an HR-BS (or HR-RS) and when there is no infrastructural station in the network and HR-MS needs to relay data for each other. HR-MS/HR-MS synchronization must be established in both of these scenarios.

2.2. HR-MS Forwarding to Network

As specified in 802.16n SRD [1]:

“HR-MS forwarding is defined as the case where the origination and termination of data are at the HR-MS and network respectively and vice versa.

HR-Network shall support HR-MS forwarding of user data and control signaling between HR-MS and HR-BS and between HR-MS and HR-RS.  The control signaling and data transmission for the HR-MS to HR-MS direct link shall at least be capable of operating within the frequency band that the HR-BS operates. An association establishment shall be supported.”

Based on the above specifications, there can be two main use cases for HR-MS forwarding to network (FTN), i.e., when the forwarded HR-MS is within or outside the coverage of the corresponding HR-BS (or HR-RS). HR-MS/HR-MS synchronization must be established in both of these scenarios.

# 3. Design Criteria

In designing mechanisms to support HR-MS to HR-MS synchronization, the important design criteria are as follows:

- Minimizing changes to the existing 802.16 specifications;

- Avoiding interference to other .16 operations;

- Incurring low overhead and complexity;

- Allowing flexible scheduling and configuration.

# 4. Supporting HR-MS/HR-MS Synchronization

  - Both HR-MSs are within the coverage of HR-BS

This scenario is applicable for both HR-MS direct communications and HR-MS FTN. Here, both HR-MSs can receive DL preambles broadcasted by HR-BS and use these to achieve common frame-level synchronization.

  - Only one HR-MS is within the coverage of HR-BS

Suppose HR-MS1 and HR-MS2 want to transmit to and receive from each other. Furthermore, suppose that only HR-MS1 is within the coverage of HR-BS. In this scenario, HR-MS1 should follow normal DL synchronization with its serving HR-BS. For HR-MS/HR-MS frame-level synchronization, HR-MS1 shall periodically broadcast preamble for far-away HR-MS2 to receive and synchronize.

  - Both HR-MSs are outside of the coverage of HR-BS

An example of this scenario is when HR-MS1 and HR-MS2 are having direct communications in an infrastructure-less deployment (or due to single point of failure). For this, an HR-MS (which can be HR-MS1, HR-MS2, or another node) should first be elected as the network coordinator. It is assumed that either one or both HR-MS1 and HR-MS2 then are within the coverage of the elected coordinator. The mechanism to elect such a coordinator is [TBD].

After being elected, the coordinator shall periodically broadcast preambles for frame-level synchronization. With this, the control is back to one of the two earlier scenarios.

# 5. Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n AWD Document (C802.16x-xx/xxxx)]

# 17.2.x HR-MS to HR-MS Synchronization

Synchronization between pairs of HR-MSs shall be supported to enable HR-MS direct communications and HR-MS forwarding to network. The actual mechanisms to achieve such synchronization depend on the specific operational scenarios, as described below.

      - Both HR-MSs are within the coverage of HR-BS (or HR-RS)

When both HR-MSs are able to receive preambles from HR-BS/HR-RS, they use these to achieve frame-level synchronization (with respect to HR-BS/HR-RS and between themselves).

      - Only one HR-MS is within the coverage of HR-BS (or HR-RS)

When two HR-MSs need to achieve pair-wise synchronization and only one of them is within the coverage of HR-BS/HR-RS, the inside-of-coverage HR-MS should first synchronize with HR-BS. After that, this inside-of-coverage HR-MS shall broadcast preambles at predefined intervals to allow the out-of-coverage HR-MS to receive and co-synchronize.

      - Both HR-MSs are outside infrastructure mode

An example of this scenario is when HR-MS1 and HR-MS2 are having direct communications in a infrastructure-less deployment (or due to single point of failure). For this, an HR-MS (which can be HR-MS1, HR-MS2, or another node) should first be elected as the network coordinator. It is assumed that either one or both HR-MS1 and HR-MS2 then are within the coverage of the elected coordinator. The mechanism to elect such a coordinator is [TBD]. After being elected, the coordinator shall periodically broadcast preambles for frame-level synchronization. With this, the control is back to one of the two earlier scenarios.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.