| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support of Local Forwarding for the IEEE 802.16n Networks |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Shoukang Zheng, Haiguang Wang, Choong Hock Mar, Jaya Shankar, Hoang Anh Tuan, Yeow Wai Leong Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2252 E-mail: skzheng@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this document, we propose a local forwarding method for IEEE 802.16n network for more efficient communication |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Support of Local Forwarding for the IEEE 802.16n Networks

Shoukang Zheng, Haiguang Wang, Choong Hock Mar, Jaya Shankar, Hoang Anh Tuan and Yeow Wai Leong

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

The SRD in 802.16n requires that HR-Network should allow local forwarding, which allows one HR-MS to communicate to one or more HR-MSs via infrastructure station without going through the backhaul. Comparing to data transfer between a BS and MS, local forwarding is more efficient to allow the data forwarding done by the relay station without going through BS or the backhaul. In this document, we discuss the method to support local forwarding for unicast flow and consider two handover cases. We assume that the IEEE 802.16n amendment will be an extension of the IEEE 802.16m amendment.

# 2. Proposed protocol

An example of local forwarding for unicast flows is shown as in Fig. 1, where two HR-MSs start communication via infrastructure stations HR-RS and HR-BS in the left figure and via HR-RS performing local forwarding in the right figure. In Fig. 1, the solid line indicates the links between the stations and the dash line represents the data path. Obviously the link bandwidth between HR-RS and HR-BS could be saved to improve the efficiency when local forwarding is performed by HR-RS.

Fig. 1 Communication path for 802.16m networks without local forwarding (left figure)

and 802.16n networks with local forwarding (right figure)

The local forwarding opportunity can be detected by HR-RS. During network entry and initialization, HR-MS is assigned with IP address and station ID (STID), which is known to HR-RS. When the source HR-MS starts to communicate with the destination HR-MS, control messages (e.g. DSA-REQ) including source and destination IP addresses will be received by HR-RS if both HR-MSs are associated with the same HR-RS. Hence once the FIDs for uplink flow from the source to HR-RS and downlink flow from HR-RS to the destination are assigned by HR-RS, these two FIDs (or equivalently station flow IDs i.e. SFIDs) can be associated with each other with the aid of HR-RS. Optionally if required by HR-BS, the HR-RS is able to establish the uplink connection from itself to HR-BS for the backup of the data that is locally forwarded by HR-RS.

In handover we should consider two cases (i) when HR-MS (either the source or the destination) is going out of the control of HR-RS that performs local forwarding for it, and (ii) when HR-MSs (both the source and the destination) are going to be under the control of the same HR-RS that will perform local forwarding for them. In first case, serving HR-RS has to relinquish the local forwarding as both are not under the same HR-RS. This results in the connection re-establishment after handover same as the normal scenario without local forwarding. In second case, the target HR-RS has to perform the connection re-establishment.

# 3. Text Proposal for IEEE 802.16n AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Insert the following text into Section 17.2.6 Local Forwarding of the 802.16n Document (C802.16n-10/0049)]

HR-RS should detect the local forwarding opportunity and be able to bind together the uplink flow ID from the source and the downlink flow ID to the destination for two communicating HR-MSs within its control during connection establishment or connection re-establishment for handover, if it is allowed by HR-BS. After the binding HR-RS is able to forward the data from the source to the destination without going through HR-BS and may optionally forward to HR-BS one copy of the data that is being locally forwarded, if required.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.