| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Alternative Paths for a BS failure after a BS failure for 802.16n networks |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Wai Leong Yeow, Hoang Anh Tuan, Haiguang Wang, Jaya Shankar, Joseph Teo Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2254 E-mail: wlyeow@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this document, we propose mechanisms to prepare alternative paths for a BS failure |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Alternative Paths for a BS failure for 802.16n networks

Wai Leong Yeow, Hoang Anh Tuan, Haiguang Wang,

Jaya Shankar, Joseph Teo

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

A base station failure is very disruptive. All stations served by the BS will have to look for neighboring BS to perform network entry. This will cause a usage spike in physical resources, and may cause neighboring BSs to be swarmed. There may be further delay incurred since initial ranging is contention-based. Furthermore, all ongoing service flows will have to be reinitiated, resulting in longer delays for service re-connections at the upper layers.

The 802.16n SRD requires the HR-network to support establishment and maintenance of alternative paths to support fast recovery in the event of disruption. We propose to factor the possibility of multi-mode into alternative path establishment in event of a BS failure, in order to minimize disruption caused in terms of recovery time and physical resource utilization. Specifically, the HR-network shall prepare and maintain a failover topology of alternate paths with role-changed stations, and support fast network recovery with failover configuration.

# 2. Failover topology and configuration after a BS failure for HR-Network

Upon a HR-BS failure, all stations in network would be in one of the following networks after attempts to recover:

- Join another BS/RS. Another BS/RS is in the vicinity.

- Standalone network without BS/RS. No infrastructure stations are reachable.

With HR-MSs capable of multi-mode, two additional cases are possible, and the size of the standalone network may be further reduced.

- Backup BS (B-BS). A HR-MS assumes the role of a HR-BS. Neighboring stations join the new HR-BS.

- Join another BS via a backup RS (B-RS). A HR-MS assumes the role of a HR-RS. The new HR-RS joins another BS in the vicinity, and neighboring stations join it via the new HR-RS.

# 3. Fast Network Entry

We propose that stations along the alternative path maintain information that a fast network entry in the event of the HR-BS failure. It consists of pre-negotiated information between the subordinate station and along the alternative path.

# 4. Summary

We propose to factor the possibility of multi-mode into alternative path establishment in event of a BS failure, in order to minimize disruption caused. Specifically, we proposed mechanisms to prepare and maintain alternate paths with role-changed stations, and support fast network recovery with pre-negotiated information.

# 5. Text Proposal for IEEE 802.16n AWD

Xxx

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

17.2.1.1 HR-MS Multi-mode capability registration

A HR-MS that is capable of role change to HR-RS and/or HR-BS shall register this capability to the serving HR-BS during network entry, at the basic capability negotiation phase. HR-MS should indicate to the serving HR-BS if it is unavailable to perform a role-change.

17.2.7.3 Alternative Path Establishment and Maintenance

17.2.7.3.x Fast Network Entry after SPOF

HR-Network shall maintain pre-negotiated information at stations along alternative path and support fast network entry.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.