E

| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Two- hop (17.2.2.2) |  |
| Date Submitted | 2011-03-16 |  |
| Source(s) | Yih-Guang Jan, Yang-Han Lee, Jheng-Yao Lin, Chih-Yuan Lo, Liang-Yu, Yen Tamkang University (TKU) Hsien-Wei Tseng De Lin Institute of Technology (DLIT) Youn-Tai Lee, Chun-Yen Hsu Institute for Information Industry (III) Shiann-Tsong Sheu National Central University (NCU) Whai-En Chen National Ilan University (NIU) Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada National Institute of Information and Communications Technology (NICT) | yihjan@yahoo.com hwtseng@mail.dlit.edu.tw lyt@nmi.iii.org.tw stsheu@ce.ncu.edu.tw wechen@niu.edu.tw mingtuo@nict.com.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this contribution we make it clear of Two- hop operation (17.2.2.2) for IEEE 802.16n draft text |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Two- hop

Yih-Guang Jan, Yang-Han Lee, Jheng-Yao Lin, Chih-Yuan Lo, Liang-Yu Yen

Tamkang University (TKU)

Hsien-Wei Tseng

De Lin Institute of Technology (DLIT)

Youn-Tai Lee, Chun-Yen Hsu

Institute for Information Industry (III)

Shiann-Tsong Sheu

- National Central University (NCU)

- Whai-En Chen

- National Ilan University (NIU)

Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

- National Institute of Information and Communications Technology (NICT)

- Introduction

This contribution provides clear description of Two-hop operation in IEEE 802.16n AWD.

- Proposed IEEE 802.16n AWD Text Modification

The text in BLACK color: the existing text in AWD

The text in BLUE color: the new text added to the Definitions

===================Start of Proposed Text====================

17.2.2.2 Two-hop

HR-MS shall support forwarding of user data and control signaling between an HR-MS and or more-HR-MSs that are 2-hop away.

====================End of Proposed Text====================

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.