| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on the Overview for IEEE 802.16n |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon ETRI | E-mail: ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-10/0051,” in response to the CFC for IEEE 802.16n AWD |  |
| Abstract | Overview for the IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on the Overview for IEEE 802.16n

Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon

ETRI

# Introductions

This document provides the overview for IEEE 802.16n based on the SRD (i.e., IEEE 802.16n-10/0048[1]) focusing on the following issues:

- Operation frequencies

- Operating bandwidths

- Duplex

- Backward compatibility

In addition, some definition and abbreviations are provided for IEEE 802.16n.

## Backward Compatibility

HR-Network shall be backward compatible with the WirelessMAN-OFDMA or WirelessMAN-Advanced Air Interface.

## Complexity

IEEE 802.16n amendment should minimize complexity of the architecture and protocols and avoid excessive system complexity.  It should enable interoperability of access networks, support low cost devices and minimize total cost of ownership.

## Services

IEEE 802.16n should support services that require a higher degree of assurance of maintaining sufficient connectivity than can be provided by IEEE 802.16 legacy systems.  Examples of such services can be found in PPDR (Public Protection and Disaster Recovery) and M2M (Machine to Machine) communication networks for utility monitoring and control.

## Operating Frequencies

The HR-Network shall be specified to allow operation in all radio frequencies where 802.16 operates.

The HR-Network shall also be specified to allow operation in unlicensed and lightly licensed spectrum bands below 6 GHz with means and mechanisms to coexist with other radio access technologies (RATs).

As an example for frequencies used for PPDR, the following frequency bands have been identified by WRC 2003 (RESOLUTION 646 (WRC-03): Public Protection and Disaster Relief, The World Radiocommunication Conference (Geneva, 2003).)

For region 1

- 380-385 MHz

- 390-395 MHz

For region 2

- 746-806 MHz

- 806-869 MHz

- 4,940-4,990 MHz

For region 3

- 406.1-430 MHz

- 440-470 MHz

- 806-824 MHz

- 851-869 MHz

- 4,940-4990 MHz

- 5,850-5,925 MHz

One more example is 170-205 MHz that is specified by Japan Ministry of Internal Affairs and Communications (MIC) in 2010 for public broadband applications in Japan”

## Operating bandwidths

HR-Network shall support the operating bandwidths of the WirelessMAN-OFDMA or WirelessMAN-Advanced Air Interface. This bandwidth may be supported by single or multiple RF carriers.

Other bandwidth may be considered as necessary to meet the government or operator requirements.

## Duplex schemes

The HR-Network shall be specified to support TDD and FDD.

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex,” January 2011.

- IEEE Std. 802.16-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16m-09/0034r3, “IEEE 802.16m System Description Document (SDD),” June 2010.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Add the following text in Section 3. Definitions into the 802.16n AWD]

3. Definitions

[Insert the following definitions:]

3.x Degraded Network: The failure of one or more 802.16 network infrastructure nodes or network connectivity.

3.x+1 Robustness: The capability of the network to withstand and automatically recover from degradation to provide the required availability to support mission critical applications (essential to the core function of society and the economy) including recovery from a single point of failure.

3.x+2 Mobile Base Station: A base station which is capable of maintaining service while moving.

3.x+3 Radio Path Redundancy: The ability to provide alternative paths between base stations, relay stations, and subscriber stations.

3.x+4 HR-MS: A subscriber station that complies with the requirements for subscriber stations in high reliable network.

3.x+5 HR-BS: A base station that complies with the requirements for base stations in high reliable network.

3.x+6 HR-RS: A relay that complies with the  requirements for relays in high reliable network.

3.x+7 HR-Network: A network whose stations comply with their respective HR requirements.

3.x+8 HR-station: An HR-MS, HR-BS, or HR-RS.

3.x+9 Infrastructure station: An HR-BS or HR-RS.

[Remedy2: Add the following text in Section 4. Abbreviations and acronyms into the 802.16n AWD]

4. Abbreviations and acronyms

[Insert the following abbreviations:]

HR			High Reliability

PPDR			Public Protection and Disaster Relief

SPOF			Single Point of Failure

[Remedy3: Add the following text in Section 17.1 Overview into the 802.16n AWD]

17. WirelessMAN-High Reliability Network

17.1 Overview

17.1.1 Operating frequencies and channel bandwidths

17.1.2 Operating bandwidths

17.1.3 Duplex

The High Reliable (HR) Network supports deployment in all frequencies as described in 1.3 and Annex T. The HR-Network also supports all channel bandwidth along with TDD and FDD duplex mode as described in 6.37 and 16.3.

The HR-Network shall also be specified to allow operation in unlicensed and lightly licensed spectrum bands below 6 GHz with means and mechanisms to coexist with other radio access technologies (RATs).

The HR-Network supports the following frequency bands have been identified by WRC 2003 (RESOLUTION 646 (WRC-03): Public Protection and Disaster Relief, The World Radiocommunication Conference (Geneva, 2003).) for PPDR

For region 1

- 380-385 MHz

- 390-395 MHz

For region 2

- 746-806 MHz

- 806-869 MHz

- 4,940-4,990 MHz

For region 3

- 406.1-430 MHz

- 440-470 MHz

- 806-824 MHz

- 851-869 MHz

- 4,940-4990 MHz

- 5,850-5,925 MHz

17.1.417.1.2 Backward compatibility

The HR-Network shall be backward compatible with the WirelessMAN-OFDMA or WirelessMAN-Advanced Air Interface.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]