| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast Connection Establishment in IEEE 802.16n |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon ETRI | E-mail: ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-10/0051,” in response to the CFC for IEEE 802.16n AWD |  |
| Abstract | Connection establishment for multicast in IEEE 802.16n |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multicast Connection Establishment in IEEE 802.16n

Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon

ETRI

# Introductions

HR-Network shall provide optimized MAC protocols for unicast and multicast transmission. In addition, section 6.2.1 in SRD (i.e., IEEE 802.16n-10/0048[1]) of IEEE 802.16n describes some requirements related to service provided on HR-Network to support unicast and multicast communication.

This document provides the overall description of multicast connection establishment meeting the requirements [1][4] in IEEE 802.16n for the 802.16n Amendment Draft Standard.

# Connection Establishment

Figure 1 shows the procedure of multicast connection establishment including following steps:

- Discovery & Registration

- Connection Establishment

- Transmit/Receive multicast specific control channel

- Receive multicast data

Figure 1 – Procedure of multicast connection establishment

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex,” January 2011.

- IEEE Std. 802.16-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16m-09/0034r3, “IEEE 802.16m System Description Document (SDD),” June 2010.

- IEEE C802.16gman-10/0014, “Group Calls and Multicast Operation for Public Safety and Public Protection,” May 2010.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy: Add the following text into Section 17.3.9 Multicast Support in the 802.16n AWD]

## 17. WirelessMAN-High Reliability Network

### 17.3 WirelessMAN HR Advanced air interface

#### 17.3.9 Multicast Support

##### 17.3.9.x Multicast Connection Operation

###### 17.3.9.x.1 Multicast communication establishment

The procedure of multicast communication establishment is showing in Figure xxx. The procedure includes

- Capacity exchange using AAI-REG-REQ/RSP

- DSx procedure containing relevant multicast parameter to establish multicast connection

- Transmission and receiving the HR multicast control channel

To discover multicast service, HR-MS will inform HR-BS of support of multicast transmission by AAI-REG-REQ message and the HR-BS will indicate if it supports multicast for that HR-MS through AAI-REG-RSA message. The basic multicast capability exchange in AAI_REG-REQ/RSP message is described in 16.2.3.8 and 16.2.3.9.

When an HR-MS registers to receive multicast services, the serving HR-BS or the HR-MS may initiate the DSA procedure for multicast connections. The HR-MS's discovery and registration of multicast services with the HR-BS through upper layer signaling are outside the scope of this standard.

The AAI-DSA, AAI-DSC and AAI-DSD messages are used to establish, change, and delete multicast service flows respectively. The HR-BS shall send the AAI-DSA-REQ/RSP to the HR-MS with the relevant multicast parameters including Multicast Group ID.

To receive multicast data, an HR-MS receives the multicast allocation information in the multicast control channel (i.e., multicast assignment MAP).

Figure xxx – Procedure of multicast communication establishment

[-------------------------------------------------End of Text Proposal----------------------------------------------------]