| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on when an HR-BS switches to an HR-RS mode |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Yeongmoon Son, Youngbin Chang, Jaehyuk Jang , Kyoungkyu Kim, Rakesh Taori Samsung Electronics | E-mail: ym1004.son@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for contributions for AWD |  |
| Abstract | This contribution proposes the condition on when HR-BS switches to HR-RS mode. |  |
| Purpose | To be discussed and adopted in IEEE802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification on when an HR-BS switches to an HR-RS mode

Yeongmoon Son, Youngbin Chang, Jaehyuk Jang, Kyoungkyu Kim, Rakesh Taori
Samsung Electronics

# Introduction

This contribution provides clarification on trigger condition for HR-BS to switch to HR-RS mode.

In IEEE802.16n system, an HR-BS can switch to an HR-RS mode as described in 6.1.1.2 of SRD documents (i.e. IEEE802.16n-10/0048):

“HR-Network shall support HR-BS communication with another HR-BS in order to support the relaying function to provide continuous network connectivity.”

However, it is very ambiguous when the HR-BS has to switch to HR-RS mode. We need to make a rule on when the HR-BS should switch to an HR-RS mode.

When the HR-BS loses link with backhaul, it needs to be attached to another backhaul to which a neighbor HR-BS is connected, in order to maintain the connection in HR-MSs belonging to the HR-BS. Therefore, we can consider the following scenario:

- When a backhaul of the serving HR-BS is disrupted, the serving HR-BS broadcasts ‘Link Loss with Backhaul’ to HR-MSs. In addition, the serving HR-BS shall perform network entry with a neighbor HR-BS. The serving HR-BS shall transmit AAI-RNG-REQ message including ‘Link Loss with Backhaul’ and Serving HR-BSID to the neighbor HR-BS.

- Upon receiving the broadcast (message) from the serving HR-BS, the HR-MS may perform HR-BS re-selection, or stay at the serving HR-BS without UL data transmission.

  - In case of HR-BS re-selection, the HR-MS will perform the network re-entry procedure with the discovered HR-BS (i.e. the HR-MS transmits a RNG-REQ message including ‘indication for the serving HR-BS Link Loss with Backhaul’).

- When an HR-MS performs network recovery procedure with the discovered HR-BS, the discovered HR-BS shall check via network entity (e.g. ASN G/W) whether the serving HR-BS is alive.

  - In case of failure to communicate with the serving HR-BS, if the serving HR-BS has been already reported as being still alive but losing link with backhaul by another HR-MSs, the discovered HR-BS shall instruct the HR-MS to perform full network reentry. Otherwise, the discovered HR-BS shall instruct the HR-MS to switch to HR-RS mode through the RNG-RSP message.

  - In case that serving HR-BS is still alive (i.e. successful communication), the discovered HR-BS shall instruct the HR-MS to continue to perform (normal) network reentry.

- When HRㅓㅏㅣㄴㅓㄴㄹㅇㄹDDFSFthe serving HR-BS is recovered from disruption, the new HR-BS will instruct the HR-MS to stop HR-RS mode and go back to HR-MS mode.

# Proposed Text

Adopt the following Remedies.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy #1: Insert new section 17.2.1.1 after 17.2.1 as follows:]

17.2.1 Multi-mode operation

17.2.1.1 HR-BS acting as an HR-RS

An HR-BS shall be capable of communication with another HR-BS in order to support the relaying function to provide continuous network connectivity.

When the HR-BS detects link loss with backhaul, the HR-BS shall broadcast ‘Link Loss with Backhaul’ through a TBD message. In addition, the HR-BS shall perform network entry with a neighbor HR-BS to provide continuous network connectivity to HR-MSs belonging to the HR-BS. The HR-BS shall transmit AAI-RNG-REQ message including ‘Link Loss with Backhaul’ and Serving HR-BSID to the neighbor HR-BS. When the neighbor HR-BS receives the AAI-RNG-REQ message and accepts the HR-BS’s switching to HR-RS mode, the neighbor HR-BS shall instruct the HR-BS to change its role to a relay station mode by including HR-RS mode = 0b1 and Preamble for HR-RS mode in AAI-RNG-RSP message. When the HR-BS receives the AAI-RNG-RSP message with HR-RS mode = 0b1 and Preamble for HR-RS mode, the HR-BS shall switch to HR-RS mode and perform network entry with the neighbor HR-BS. After complete network entry, the HR-BS shall use the Preamble for HR-RS mode indicated in the AAI-RNG-RSP message to act as an HR-RS and shall broadcast ‘Recovery of Link Loss with Backhaul’ through a TBD message so that HR-MSs in the HR-BS can resume UL data transmission.

When the HR-BS acting as HR-RS detects the backhaul is recovered, it shall broadcast ‘Recovery of Link Loss with Backhaul’ through a TBD message and may stop HR-RS operation by transmitting AAI-DREG-REQ message with De-registration_Request_Code = 0x06(i.e. Stop of HR-RS operation) to the neighbor HR-BS. In this case, the neighbor HR-BS shall transmit AAI-DREG-RSP message with Action code = 0x10 or 0x11 (see table 699).

On the other hand, upon receiving the TBD message indicating link loss with backhaul, the HR-MS may perform HR-BS re-selection, or stay at the serving HR-BS without UL data transmission. In case of HR-BS re-selection, the HR-MS shall inform a neighbor HR-BS that the HR-BS has a backhaul disrupted (i.e. the HR-MS transmits a RNG-REQ message including HR-MSID, Serving HR-BSID, and Serving HR-BS Link Loss with Backhaul). In this case, the neighbor HR-BS shall instruct the HR-MS to perform full network reentry.

Afterwards, when the neighbor HR-BS receives the RNG-REQ message including Ranging Purpose Indication = 0b1000 (i.e. indicating a network reentry after a coverage loss), HR-MSID (i.e. MAC address) and Serving HR-BSID from an HR-MS, the neighbor HR-BS shall check via network entity (e.g. ASN G/W) whether the serving HR-BS is alive.

- In case of failure to communicate with the serving HR-BS, if the serving HR-BS has been already reported as being still alive but losing link with backhaul by another HR-MS’s report, the neighbor HR-BS shall instruct the HR-MS to perform full network reentry. Otherwise, the neighbor HR-BS shall instruct the HR-MS to switch to HR-RS mode through the RNG-RSP message including HR-RS mode = 0b1, HR-RSID and Preamble for HR-RS mode.

- In case that the serving HR-BS is still alive (i.e. successful communication), the neighbor HR-BS shall instruct the HR-MS to continue to perform network reentry.

When the HR-MS, which has been staying at the serving HR-MS without UL data transmission, receives ‘Recovery of Link Loss with Backhaul’ through a TBD message, the HR-MS may resume UL data transmission.

[Remedy #2: Modify the Table 678 as follows:]

16.2.3.1 AAI-RNG-REQ

…

Table 678.—AAI-RNG-REQ message Field Description

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Purpose Indication |  |  |  |
| … |  |  |  |
| For(j=0; j<N_CSG_IDs; j++) { |  |  |  |
| CSGID |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Link Loss with Backhaul | 1 | Indicates Disruption of backhaul 0: Reserved 1: Backhaul is disrupted | Shall be included when HR-BS detects backhaul disruption and performs network entry with a neighbor HR-BS. |
| Serving HR-BS Link Loss with Backhaul |  | Indicates Serving HR-BS is suffering from link loss with a backhaul 0: Reserved 1: Serving HR-BS is suffering from link loss with a backhaul | Shall be included when an HR-MS receives a TBD message including ‘Link Loss with Backhaul’ from Serving HR-BS and performs network entry with a neighbor HR-BS. |

[Remedy #3: Modify the Table 679 as follows:]

16.2.3.2 AAI-RNG-RSP

…

Table 679.—AAI-RNG-RSP message Field Description

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Abort |  |  |  |
| If(Ranging Abort == 1) { |  |  |  |
| ... |  |  |  |
| } else { |  |  |  |
| ... |  |  |  |
| For(i=0; i<N_SFIDs; i++) { |  |  |  |
| SFID |  |  |  |
| } |  |  |  |
| HR-RS mode | 1 | Indicates switching to HR-RS mode. 0: Reserved 1: Requests the HR-BS to switch to HR-RS mode | Shall be included when HR-BS or HR-MS performing network entry needs to be acting as HR-RS mode. |
| If(HR-RS mode == 0b1) { |  |  |  |
| HR-RSID | TBD | The identifier required when HR-MS acts as HR-RS | Shall be included when HR-MS needs to be acting as HR-RS mode |
| Preamble index for HR-RS mode | TBD | The HR-BS acting as an HR-RS shall use preamble specified by this parameter, after network entry |  |
| } |  |  |  |
| } |  |  |  |

[Remedy #4: Modify the Table 698 as follows:]

16.2.3.21 AAI-DREG-REQ message

…

Table 698.—AAI-DREG-REQ message format

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| De-Regstration_Request_Code | 3 | Used to indicate the purpose of this message 0x00: AMS deregistration request from ABS and network 0x01: request for AMS deregistration from S-ABS and initiation of AMS idle mode. 0x02: response for the unsolicited AAIDREG-RSP message with action code 0x05 by the ABS. 0x03: reject for the unsolicited AAIDREG-RSP message with action code 0x05 by the ABS. This code is applicable only when an AMS has a pending UL data to transmit. 0x04: request for AMS deregistration from S-ABS to enter DCR mode 0x05: response for the unsolicited AAIDREG-RSP message with action code0x00, 0x01, 0x02 or 0x03 0x06: request Stopping HR-RS operation 0x06-0x07: reserved |  |
| … | … | … | … |

[Remedy #5: Modify the Table 699 as follows:]

16.2.3.22 AAI-DREG-RSP message

…

Table 699.—AAI-DREG-RSP message format

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action code | 4 | Used to indicate the purpose of this message 0x00: AMS shall immediately terminate service with the ABS and should attempt network entry at another ABS 0x01: AMS shall listen to the current ABS but shall not transmit until a RESCMD message or AAI-DREG-RSP message with action code 0x02 or 0x03 is received. 0x02: AMS shall listen to the current ABS but only transmit on the control connection. 0x03: AMS shall return to normal operation and may transmit on any of its active connections. 0x04: This option is valid in response to a AAI-DREG-REQ message with De-Registration Request Code=0x00. The AMS shall terminate current Connected State with the ABS. 0x05: AMS shall begin idle mode initiation: a) to signal AMS to begin idle mode in unsolicited manner or b) to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration 0x06: This option is valid only in response to a AAI-DREG-REQ message with De-Registration Code 0x01: a) to reject AMS-initiated idle mode request or b) to allow AMS to transmit AMS-initiated idle mode request at the REQDuration expiration 0x07: This option is valid in response to a AAI-DREG-REQ message with Deregistration-request-code= 0x01 to allow AMS-initiated idle mode request. 0x08: This option is valid only in response to an AAI-DREG-REQ message with De-Registration Request Code 0x04 to allow retention of the AMS's connection information 0x09: This option is valid only in response to an AAI-DREG-REQ message with De-Registration Request Code 0x04 to reject retention of the AMS's connection information. 0x10: This option is valid only in response to an AAI-DREG-REQ message with Deregistration-request-code= 0x06 to allow Stopping HR-RS operation. 0x11: This option is valid only in response to an AAI-DREG-REQ message with Deregistration-request-code= 0x06 to reject Stopping HR-RS operation. 0x1012-0x15: reserved |  |
| … | … | … | … |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------