| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on when an HR-BS switches to an HR-RS mode |  |
| Date Submitted | 2011-03-17 |  |
| Source(s) | Yeongmoon Son, Youngbin Chang, Jaehyuk Jang , Kyoungkyu Kim, Rakesh Taori Samsung Electronics | E-mail: ym1004.son@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for contributions for AWD |  |
| Abstract | This contribution proposes the condition on when HR-BS switches to HR-RS mode. |  |
| Purpose | To be discussed and adopted in IEEE802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on when an HR-BS switches to an HR-RS mode

Yeongmoon Son, Youngbin Chang, Jaehyuk Jang, Kyoungkyu Kim, Rakesh Taori
Samsung Electronics

# Introduction

This contribution provides clarification on trigger condition for HR-BS to switch to HR-RS mode.

In IEEE802.16n system, an HR-BS can switch to an HR-RS mode as described in 6.1.1.2 of SRD documents (i.e. IEEE802.16n-10/0048):

“HR-Network shall support HR-BS communication with another HR-BS in order to support the relaying function to provide continuous network connectivity.”

However, it is very ambiguous when the HR-BS has to switch to HR-RS mode. We need to make a rule on when the HR-BS should switch to an HR-RS mode.

When the HR-BS loses link with backhaul, it needs to be attached to another backhaul to which a neighbor HR-BS is connected, in order to maintain the connection in HR-MSs belonging to the HR-BS. Therefore, we can consider the following scenario:

- When a backhaul of the serving HR-BS is disrupted, the serving HR-BS broadcasts ‘Link Loss with Backhaul’ to HR-MSs. In addition, the serving HR-BS shall perform network entry with a neighbor HR-BS. The serving HR-BS shall transmit AAI-RNG-REQ message including ‘Link Loss with Backhaul’ and Serving HR-BSID to the neighbor HR-BS.

- Upon receiving the broadcast (message) from the serving HR-BS, the HR-MS may perform HR-BS re-selection, or stay at the serving HR-BS without UL data transmission.

  - In case of HR-BS re-selection, the HR-MS will perform the network re-entry procedure with the discovered HR-BS (i.e. the HR-MS transmits a RNG-REQ message including ‘indication for the serving HR-BS Link Loss with Backhaul’).

- When an HR-MS performs network recovery procedure with the discovered HR-BS, the discovered HR-BS shall check via network entity (e.g. ASN G/W) whether the serving HR-BS is alive.

  - In case of failure to communicate with the serving HR-BS, if the serving HR-BS has been already reported as being still alive but losing link with backhaul by another HR-MSs, the discovered HR-BS shall instruct the HR-MS to perform full network reentry. Otherwise, the discovered HR-BS shall instruct the HR-MS to switch to HR-RS mode through the RNG-RSP message.

  - In case that serving HR-BS is still alive (i.e. successful communication), the discovered HR-BS shall instruct the HR-MS to continue to perform (normal) network reentry.

- When HRㅓㅏㅣㄴㅓㄴㄹㅇㄹDDFSFthe serving HR-BS is recovered from disruption, the new HR-BS will instruct the HR-MS to stop HR-RS mode and go back to HR-MS mode.

# Proposed Text

Adopt the following Remedies.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy #1: Insert new section 17.2.1.1 after 17.2.1 as follows:]

17.2.1 Multi-mode operation

17.2.1.1 HR-BS acting as an HR-RS

An HR-BS shall be capable of communication with another HR-BS in order to support the relaying function to provide continuous network connectivity.

When the HR-BS detects link loss with backhaul, the HR-BS shall broadcast the indication of backhaul disruption. In addition, the HR-BS shall perform network entry with a neighbor HR-BS to provide continuous network connectivity to HR-MSs belonging to the HR-BS. The HR-BS shall transmit AAI-RNG-REQ message including ‘Link Loss with Backhaul’ and Serving HR-BSID to the neighbor HR-BS. When the neighbor HR-BS receives the AAI-RNG-REQ message, the neighbor HR-BS may instruct the HR-BS to change its role to a relay station mode through AAI-RNG-RSP message. After complete network entry, the HR-BS shall broadcast the indication of backhaul recovery so that HR-MSs in the HR-BS can resume UL data transmission.

When the HR-BS acting as HR-RS detects the backhaul is recovered, it shall broadcast the indication of backhaul recovery and may stop HR-RS operation by deregistration transaction with the neighbor HR-BS.

On the other hand, upon receiving the broadcast indicating the backhaul disruption, the HR-MS may perform HR-BS re-selection, or stay at the serving HR-BS without UL data transmission. In case of HR-BS re-selection, the HR-MS shall inform a neighbor HR-BS that the HR-BS has a backhaul disrupted. In this case, the neighbor HR-BS shall instruct the HR-MS to perform full network reentry and shall check via network entity (e.g. ASN G/W) whether the serving HR-BS is alive.

- In case of failure to communicate with the serving HR-BS, if the serving HR-BS has been already reported as being still alive but losing link with backhaul by another HR-MS’s report, the neighbor HR-BS shall instruct the HR-MS to perform full network reentry. Otherwise, the neighbor HR-BS shall instruct the HR-MS to switch to HR-RS mode.

- In case that the serving HR-BS is still alive (i.e. successful communication), the neighbor HR-BS shall instruct the HR-MS to continue to perform network reentry.

When the HR-MS, which has been staying at the serving HR-MS without UL data transmission, receives broadcast indicating the recovery of  backhaul disruption, the HR-MS may resume UL data transmission.

---------------------------------------------------- End of Proposed Text ------------------------------------------------------