| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on when an HR-MS switches to an HR-RS mode |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Yeongmoon Son, Youngbin Chang, Jaehyuk Jang , Kyoungkyu Kim, Rakesh Taori Samsung Electronics | E-mail: ym1004.son@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for contributions for AWD |  |
| Abstract | This contribution proposes the condition on when HR-MS switches to HR-RS mode. |  |
| Purpose | To be discussed and adopted in IEEE802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification on when an HR-MS switches to an HR-RS mode

Yeongmoon Son, Youngbin Chang, Jaehyuk Jang, Kyoungkyu Kim, Rakesh Taori
Samsung Electronics

# Introduction

This contribution provides clarification on trigger condition for HR-MS to switch to HR-RS mode.

In IEEE802.16n system, an HR-MS can switch to an HR-RS mode as described in 6.1.1.2 of SRD documents (i.e. IEEE802.16n-10/0048):

“HR-Network shall support an HR-MS changing its role to relay data between other mobile or relay stations and a functioning HR-BS or HR-RS”

However, it is very ambiguous when the HR-MS needs to switch to HR-RS mode. Maybe, the HR-MS can switch to the HR-RS mode anytime. But it may cause interference against the HR-BSs and make unnecessary multiple hops between an HR-MS and an HR-BS. We need to make a rule on when the HR-MS should switch to an HR-RS mode.

Basically, a relay station is used to extend cell coverage. Similarly, when the HR-MS changes its role, it should be done for this purpose. But the HR-MS will be used to only temporally extend the coverage. Therefore, we can consider the following scenario:

- A serving HR-BS is disrupted. As a result, an HR-MS will try to find a new HR-BS due to coverage loss.

- The HR-MS will perform the network re-entry procedure with the new HR-BS (i.e. the HR-MS transmits a RNG-REQ message with Serving BSID and Ranging Purpose Indication set to Coverage Loss).

- The new HR-BS will check whether the serving HR-BS is alive via network entity (e.g. ASN G/W)

- If the new HR-BS will instruct the HR-MS to switch to the HR-RS mode through the RNG-RSP message

- When HRㅓㅏㅣㄴㅓㄴㄹㅇㄹDDFSFthe serving HR-BS is recovered from disruption, the new HR-BS may instruct the HR-MS to stop HR-RS operation.

# Proposed Text

Adopt the following Remedies.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy #1: Insert new section after 17.2.1 as follows:]

17.2.1 Multi-mode operation

17.2.1.1 HR-BS acting as an HR-RS

..

17.2.1.2HR-MS acting as an HR-RS

An HR-MS shall be capable of changing its role to relay data between other mobile or relay stations and a functioning HR-BS or HR-RS.

When the HR-MS detects coverage loss with an HR-BS as described in Section 16.2.26.2, the HR-MS shall perform coverage loss recovery procedure as indicated in Section 16.2.26.3. Once the HR-MS achieving PHY synchronization and DL synchronization with the discovered HR-BS, the HR-MS shall begin with network reentry by transmitting AAI-RNG-REQ message with the Ranging Purpose Indication = 0b1000 (i.e. indicating a network reentry after a coverage loss), HR-MSID(i.e. MAC address) and Serving HR-BSID to the discovered HR-BS,.

After receiving the AAI-RNG-REQ message, the discovered HR-BS shall check whether the HR-BS with the serving HR-BSID is alive. How to check this is out of scope. If the discovered HR-BS detects the serving HR-BS is still alive, the HR-BS shall instruct the HR-MS to continue to perform normal network re-entry with it as described in 16.2.26.3. Otherwise, the discovered HR-BS shall instruct the HR-MS to change its role to a relay station mode by including HR-RS mode = 0b1, HR-RSID and Preamble for HR-RS mode in AAI-RNG-RSP message.

When the HR-MS receives the AAI-RNG-RSP message with HR-RS mode = 0b1 and Preamble for HR-RS mode, the HR-MS shall switch to HR-RS mode and perform network entry with the discovered HR-BS. After complete network entry, the HR-MS acting as HR-RS mode shall use the Preamble for HR-RS mode indicated in the AAI-RNG-RSP message as a preamble broadcasted for HR-RS mode.

[Remedy #2: Modify the Table 679 as follows:]

16.2.3.2 AAI-RNG-RSP

…

Table 679.—AAI-RNG-RSP message Field Description

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Abort |  |  |  |
| If(Ranging Abort == 1) { |  |  |  |
| ... |  |  |  |
| } else { |  |  |  |
| ... |  |  |  |
| For(i=0; i<N_SFIDs;i++) { |  |  |  |
| SFID |  |  |  |
| } |  |  |  |
| HR-RS mode | 1 | Indicates switching to HR-RS mode. 0: Reserved 1: Requests the HR-MS to switch to HR-RS mode | Shall be included when HR-MS need to be acting as HR-RS mode. |
| If(HR-RS mode == 0b1) { |  |  |  |
| HR-RSID | TBD | The identifier required when HR-MS acts as HR-RS |  |
| Preamble index for HR-RS mode | TBD | The HR-MS acting as an HR-RS shall use preamble specified by this parameter, after network entry/re-entry |  |
| } |  |  |  |
| } |  |  |  |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------