| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multimode Operation for IEEE 802.16n |  |
| Date Submitted | 2011-05-09 |  |
| Source(s) | Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon ETRI | E-mail: ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-11/0002,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Multimode Operation for the IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multimode Operation for IEEE 802.16n

Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon

ETRI

# Introductions

This document provides the multimode operation for IEEE 802.16n based on the SRD (i.e., IEEE 802.16n-10/0048r1[1]).

# References

- IEEE 802.16n-10/0048r1, “802.16n System Requirements Document including SARM annex,” March 2011.

- IEEE Std. 802.16-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16m-09/0034r3, “IEEE 802.16m System Description Document (SDD),” June 2010.

- IEEE C802.16n-11/0049, “General Description of Multi-mode Operation for 802.16n,” March 2011.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy: Add the following text in Section 17.2.1 & 17.3.1 into the 802.16n AWD. Following only shows in section 17.3.1. For section 17.2.1, same proposed text and changed section number 17.2.x from 17.3.x shall be adopted.]

17.3.1 Multi-mode operation

17.3.1.1 Relay function for HR-BS

An HR-BS (affected HR-BS) having no connection to backhaul may operate as a relay station to communicate with another HR-BS (serving HR-BS) that has connection to backhaul. To the serving HR-BS, the affected HR-BS can be regarded as a normal HR-RS. To the subordinate stations of the affected HR-BS, the affected HR-BS can be regarded as either an HR-BS or an HR-RS.

An HR-BS acting as RS mode operates in either TTR mode or STR mode.

The procedures for RS mode change consist of three steps:

a) inform subordinate stations that backhaul services are unavailable

b) establish a relay link with a neighbor HR-BS

c) reconfigure the physical frame and request subordinate stations to perform handover to neighbor cell (if necessary)

Before establishing relay link with neighbor HR-BS, the HR-BS may request some subordinate HR-stations to perform handover to neighbor cell.

The HR-BS may store MAC context information (e.g., Basic capabilities, Security capabilities, and etc.) of the subordinate stations to allow those stations to perform optimized network reentry when returning back to the HR-BS upon its recovery.

If the HR-BS recovers from the failure of backhaul, it informs network or notifies neighbor HR-BS through the backhaul network interface. The neighbor HR-BS may initiate subordinate stations handover back to the HR-BS.

17.3.1.2 Relay function for HR-MS

An HR-MS may operate as an HR-RS to provide connectivity for multiple out-of-coverage HR-MSs. While operating as HR-RS, the station may maintain certain HR-MS functionalities. A mode switch to HR-RS shall be initiated by its superordinate HR-BS.

HR-MS acting as an HR-RS is operated in either TTR mode or STR mode and its relay mode is determined by the negotiation between HR-MS and HR-BS.

After establishing relay link, the superordinate HR-BS may request some subordinate HR-MSs to perform handover to the HR-MS acting as RS. The HR-BS may store MAC context information of the HR-MSs to allow HR-MS to perform optimized network reentry when returning back to the HR-BS upon the HR-MS acting as an HR-RS releases the role of RS.

If the HR-MS release its role from the relay mode, it informs subordinate HR-MS to perform handover to the neighbor infrastructure station.

The HR-MS acting as RS may be replaced by other HR-MS capable of relay functionality.

17.3.1.3 Base station function for HR-MS

An HR-MS may operate as an HR-BS to provide connectivity for itself and other HR-MSs. While operating as an HR-BS, the station may maintain certain HR-MS functionalities. A mode switch to HR-BS may be initiated by the HR-MS itself or may be directed by the superordinate HR-BS of the HR-MS.

The HR-MS acting as BS may be replaced by other HR-MS capable of base station functionality.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]