| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Local Forwarding for IEEE 802.16n |  |
| Date Submitted | 2011-05-18 |  |
| Source(s) | Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon ETRI | E-mail: ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-11/0002,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Local forwarding operation for the IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Local Forwarding for IEEE 802.16n

Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon

ETRI

# Introductions

In IEEE 802.16n based on the SRD (i.e., IEEE 802.16n-10/0048r1[1]), local forwarding should be supported to allow one HR-MS to communicate to one or more HR-MSs via infrastructure station without going through the backhaul. Different operation between for local forwarding and for standalone network is whether the backhaul connection is available or not.

# References

- IEEE 802.16n-10/0048r1, “802.16n System Requirements Document including SARM annex,” March 2011.

- IEEE Std. 802.16-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16m-09/0034r3, “IEEE 802.16m System Description Document (SDD),” June 2010.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Add the following text in Section 17.3.6 into the 802.16n AWD.

NOTE: following text (with minor modification) shall also be adopted in Section 17.2.6.]

17.3.6 Local Forwarding

To establish the local forwarding, some control signaling is exchanged through the backhaul connection between infrastructure station and network but how to exchange is out of scope of this specification.

An infrastructure station capable of providing local forwarding shall (re)assign and manage the FID for the HR-MS during DSA procedure. Any available FID may be used for the local forwarding (i.e., there are no dedicated FIDs for local forwarding connections).

An HR-MS may not be aware of local forwarding but shall follow the same procedure defined in 16.2.

Data traffic for HR-MSs sent on the connection with the FID shall be forwarded to other one or more HR-MSs without going through the backhaul. Each MAC PDU shall begin with a Generic MAC Header. The format of AGMH is the same as defined in 16.2.2.1.1. The AGMH may be followed by one or more extended headers. The MAC PDU shall also contain a payload. The payload consists of one or more HR-MSs’ data traffic. MAC PDUs sent on the connection used by local forwarding shall follow the same format as shown in Figure 386.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]