| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Path Management for IEEE 802.16n |  |
| Date Submitted | 2011-05-19 |  |
| Source(s) | Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon ETRI Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada National Institute of Information and Communications Technology (NICT) Wai Leong Yeow, Hoang Anh Tuan, Haiguang Wang, Shoukang Zheng, Jaya Shankar, Joseph Teo Institute for Infocomm Research | E-mail: ekkim@etri.re.kr scchang@etri.re.kr hvdien@nict.com.sg wlyeow@i2r.a-star.edu.sg |
| Re: | “IEEE 802.16n-11/0002,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Path management operation for the IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Path Management for IEEE 802.16n

Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon

ETRI

Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada

NICT

Wai Leong Yeow, Hoang Anh Tuan, Haiguang Wang, Shoukang Zheng, Jaya Shankar, Joseph Teo

I2R

# Introductions

In IEEE 802.16n-11/0049[4], general description of multimode operation for IEEE 802.16n based on the SRD (i.e., IEEE 802.16n-10/0048r1[1]) due to the some event such as backhaul link failure is proposed. In addition, path management during mode change as well as failure of frequency or carrier may be considered as alternative path. In this contribution, the path management operation is provided for IEEE 802.16n.

# Path management over multimode operation

Figure 1 shows the general network architecture of MS1 served by BS1. As shown the Figure 1, path between MS1 and other network entity (i.e., server) is established via BS1. In the event of disruption such as backhaul link failure, the BS1 may change its role as RS as shown in Figure 2. As shown the Figure 2, when the HR-BS (i.e., BS1) establishes relay link to any superordinate HR-BS (i.e., BS2), BS2 will be included into the path between MS1 and server (a). If the channel quality between MS1 and BS2 is more reliable than that between MS1 and BS2, (1) the HR-BS acting as RS can make MS1 to perform HO and reselect the path without HR-BS1 or (2) the MS initiates handover to BS2 and reselect the path without HR-BS1.

Figure 1 – General Network Architecture between MS1 to server via BS1

Figure 2 – Path Reselection (from via BS1 to via BS2) after establishment of relay link of HR-BS acting as RS

# Path management due to SPOF

Figure 3 shows an example path reselection after SPOF (i.e., failure of one frequency in an HR-BS). As shown the Figure 3, HR-MS may perform inter-frequency handover from FA1 to FA2 under the following condition.

- FA1 is unavailable to continue communicating between HR-BS and HR-MS

- Channel quality of FA1 is worse than that of other frequency

Figure 3 – path reselection from FA1 to FA2 due to failure of FA1 of HR-BS1

Figure 4 shows another example path reselection after SPOF (i.e., failure of one physical carrier of HR-BS). As shown the Figure 4, HR-MS may perform multicarrier handover from physical carrier index 1(CI1) to physical carrier index(CI2) under the following condition.

- CI1 is unavailable to continue communicating between HR-BS and HR-MS

- Channel quality of CI1 is worse than that of other carrier

Figure 4 - path reselection (from CI1 to CI2) on the multicarrier deployment due to failure of CI1 of HR-BS1

# References

- IEEE 802.16n-10/0048r1, “802.16n System Requirements Document including SARM annex,” March 2011.

- IEEE Std. 802.16-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16m-09/0034r3, “IEEE 802.16m System Description Document (SDD),” June 2010.

- IEEE C802.16n-11/0049, “General Description of Multi-mode Operation for 802.16n,” March 2011.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Adapt the following change in Section 17.2.7 Path Discovery and Management in the 802.16n AWD]

17.2.7 Path Discovery and Management

HR-Network shall support establishment and maintenance of alternative paths that are used for fast network recovery in the event of disruption, e.g. when the serving HR-RS or HR-BS fails. An alternative path is determined by the serving HR-BS. In normal operation, there are no active connections on any alternative path. The alternative path can be activated when the disruption event is detected.

17.2.7.1 Preparation for SPOF

# In order to support Preparation for SPOF, alternative path described in this subsection shall be supported.

# An alternative path may include HR-MS that switches mode to RS or BS.

Network entry including handover as described in 6.3.21 shall be supported in the event of SPOF. An indication of whether MAC context information of the subordinate HR-MS is being shared by infrastructure stations shall be transmitted to HR-MS.

To support fast network reentry to the neighbor HR-MSs, HR-MS shall transmit its neighbor information to HR-BS. HR-MSs capable of forwarding to the network and/or multimode operation shall share the MAC context information with the HR-MS performing local forwarding to the network.

If the most reliable alternative path of the HR-MS is not valid, the second most reliable alternative path shall be selected as the alternative path.

17.2.7.2 recovery from SPOF

Network reentry including handover as described in 6.3.21 shall be supported in the event of SPOF. Whether MAC context information of the subordinate HR-MS is shared by the infrastructure stations shall be transmitted to HR-MS.

Alternative path may be selected during the role change or release the mode as described in 17.2.1.

[Remedy2: Adapt the following change in Section 17.2.7 Path Discovery and Management in the 802.16n AWD]

17.3.7 Path Discovery and Management

HR-Network shall support establishment and maintenance of alternative paths that are used for fast network recovery in the event of disruption, e.g. when the serving HR-RS or HR-BS fails. An alternative path is determined by the serving HR-BS. In normal operation, there are no active connections on any alternative path. The alternative path can be activated when the disruption event is detected.

17.3.7.1 Preparation for SPOF

# In order to support preparation for SPOF, alternative path described in this subsection shall be supported.

# An alternative path may include HR-MS that switches mode to RS or BS.

Network entry including handover as described in 16.2.6 and 16.2.8.2.9 shall be supported in the event of SPOF. An indication of whether MAC context information of the subordinate HR-MS is being shared by infrastructure stations shall be transmitted to HR-MS.

To support fast network reentry to the neighbor HR-MSs, HR-MS shall transmit its neighbor information to HR-BS. HR-MSs capable of forwarding to the network and/or multimode operation shall share the MAC context information with the HR-MS performing local forwarding to the network.

If the most reliable alternative path of the HR-MS is not valid, the second most reliable alternative path shall be selected as the alternative path.

17.3.7.2 recovery from SPOF

Network reentry including handover as described in 16.2.6 and 16.2.8.2.9 shall be supported in the event of SPOF. Whether MAC context information of the subordinate HR-MS is shared by the infrastructure stations shall be transmitted to HR-MS.

Alternative path may be selected during the role change or release the mode as described in 17.3.1.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]