| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast Operation in IEEE 802.16n |  |
| Date Submitted | 2011-05-18 |  |
| Source(s) | Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon ETRI | E-mail: ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-11/0002,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Functional description of multicast in IEEE 802.16n |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multicast Operation in IEEE 802.16n

Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon

ETRI

# Introductions

HR-Network shall provide optimized MAC protocols for unicast and multicast transmission. In addition, section 6.2.1 in SRD (i.e., IEEE 802.16n-10/0048r1[1]) of IEEE 802.16n describes some requirements related to service provided on HR-Network to support unicast and multicast communication.

This document provides the detail description of multicast addressing meeting the requirements [1][4] in IEEE 802.16n for the 802.16n Amendment Draft Standard focusing on the following issues based on [5][6][7][8]:

- General description including addressing

- Multicast establishment

- Operation in state (connected state, idle state)

- Multicast downlink control structure

- Multicast security (submitted separate contribution[9])

# References

- IEEE 802.16n-10/0048r1, “802.16n System Requirements Document including SARM annex,” March 2011.

- IEEE Std. 802.16-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16m-09/0034r3, “IEEE 802.16m System Description Document (SDD),” June 2010.

- IEEE C802.16gman-10/0014, “Group Calls and Multicast Operation for Public Safety and Public Protection,” May 2010.

- IEEE C802.16n-11/0045, “Overall Multicast Support in IEEE 802.16n,” March 2011.

- IEEE C802.16n-11/0046, “Multicast connection establishment in IEEE 802.16n,” March 2011.

- IEEE C802.16n-11/0047, “Downlink Control for Multicast in IEEE 802.16n,” March 2011.

- IEEE C802.16n-11/0048, “Security Supporting Multicast in IEEE 802.16n,” March 2011.

- IEEE C802.16n-11/0070, “Security for Multicast in IEEE 802.16n,” May 2011.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Adapt the following change in Section 16.2.3.1 AAI-RNG-REQ in the 802.16n AWD]

[Change Table 678 in section 16.2.3.1 as indicated:]

Table 678.—AAI-RNG-REQ message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Purpose Indication | 4 | 0b0000 = Initial network entry 0b0001 = HO reentry 0b0010 = Network reentry from idle mode 0b0011 = Idle mode location update 0b0100 = DCR mode extension 0b0101 = Emergency call setup (e.g., E911) 0b0110 = Location update for updating service flow management encodings of E-MBS flows 0b0111 = Location update for transition to DCR mode from idle mode 0b1000 = Reentry from DCR mode, coverage loss or detection of different ABS restart count. 0b1001 = Network reentry from a Legacy BS 0b1010 = Zone switch to MZONE from LZONE 0b1011 = Location update due to power down. 0b1100 = Interference mitigation request to a CSG Femto ABS when experiencing interference from the CSG Femto ABS 0b1101 = NS/EP call setup 0b1110 -0b1111 = reserved 0b1110 = HR multicast service flow update 0b1111 = reserved | - |
| … | … | … | … |

[Remdy2: Adapt the following change in section 16.3.5.5.2.4 Assignment A-MAP in the 802.16n AWD]

[Change section 16.3.5.5.2.4 as indicated:]

16.3.5.5.2.4 Assignment A-MAP IE

Table 842 describes Assignment A-MAP IE Types.

Table 842 – Assignment A-MAP IE Types

| A-MAP IE Type | Usage | Property |
| --- | --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE | Unicast |
| 0b0001 | UL Basic Assignment A-MAP IE | Unicast |
| 0b0010 | DL Subband Assignment A-MAP IE | Unicast |
| 0b0011 | UL Subband Assignment A-MAP IE | Unicast |
| 0b0100 | Feedback Allocation A-MAP IE | Unicast |
| 0b0101 | UL Sounding Command A-MAP IE | Unicast |
| 0b0110 | CDMA Allocation A-MAP IE | Unicast |
| 0b0111 | DL Persistent Allocation A-MAP IE | Unicast |
| 0b1000 | UL Persistent Allocation A-MAP IE | Unicast |
| 0b1001 | Group Resource Allocation A-MAP IE | Multicast |
| 0b1010 | Feedback Polling A-MAP IE | Unicast |
| 0b1011 | BR-ACK A-MAP IE | Multicast |
| 0b1100 | Broadcast Assignment A-MAP IE | Broadcast/Multicast |
| 0b1101 | Reserved HR-Multicast DL Assignment A-MAP IE | NA. Multicast |
| 0b1110 | Reserved | NA. |
| 0b1111 | Extended Assignment A-MAP IE | NA. |

CRC Mask

A 16-bit CRC is generated based on the randomized contents of assignment A-MAP IE and is masked by 16-bit CRC mask using the bitwise XOR operation.

The 16-bit masked CRC is constructed using a 1 bit masking prefix, a 3 bit message type indicator, and 12 bit Masking Code as described in Table 843.

Table 843 – Description of CRC Mask

| Masking Prefix (1 bit MSB) | Remaining 15 bit LSBs |  |
| --- | --- | --- |
| 0b0 | Type Indicator | Masking Code |
|  | 0b000 | 12 bit STID or TSTID |
|  | 0b001 | Refer to Table 844 |
|  | 0b010 | Refer to Table 845 |
| 0b1 | 15 bit RA-ID: The RA-ID is derived from the AMS' random access attributes (i.e., superframe number (LSB 5bits), frame_index (2 bits), preamble code index for ranging or BR (6 bits) and opportunity index for ranging or BR (2 bits)) as defined below: RA-ID = (LSB 5bits of superframe number ¦ frame_index ¦ preamble_code_index ¦ opportunity_index) |  |

……

Table 845 – Description of Masking Code for type indicator 010

| Decimal Value | Description |
| --- | --- |
| 4095 | Used to mask Broadcast A-MAP IE for multicast assignment |
| Others | Reserved 12 bit MGID is used to make HR-Multicast DL Assignment A-MAP IE for high reliable multicast assignment |

……

[Remedy3: Add the following text into Section 17.3.9 Multicast Support in the 802.16n AWD]

## 17. WirelessMAN-High Reliability Network

### 17.3 WirelessMAN HR Advanced air interface

#### 17.3.9 Multicast Support

Each HR-BS capable of providing multicast communication belongs to a certain multicast group zone. A multicast zone defined as a set of HR-BSs where the same Multicast Group ID and FID is used for transmitting the content of certain service flow(s).

An HR-BS may provide the HR-MS with multicast content locally within its coverage and independently of other HR-BSs. The single HR-BS provision of multicast is therefore a configuration where a Multicast Zone is configured to consist of a single HR-BS only. In this case, the HR-BS uses any Multicast Group ID and FID for providing multicast service, independently of other HR-BSs, so the HR-MS received the multicast data from its serving HR-BS, and the HR-MS should not expect the service flow for this multicast connection to continue when the HR-MS leaves the serving HR-BS’ coverage. However, if the HR-MS moves to an HR-BS that is transmitting the same multicast flow in another HR Multicast Group Zone, HR-MS may update its service flow management encodings to continue to receive the same multicast flows.

To ensure proper multicast operation on networks of HR-BS employing multicast, the Multicast Group IDs and FIDs used for common multicast content and service shall be the same for all HR-BSs within the same HR Multicast Group Zone. This allows the HR-MS which has already registered with a service to be seamlessly synchronized with multicast transmissions within an HR Multicast Group Zone without communicating in the UL or re-registering with other HR-BS within that HR Multicast Group Zone.

##### 17.3.9.1 Multicast communication operation

An HR-BS establishes a DL multicast service by creating a multicast connection with each HR-MS to be associated with the service. Multicast service flows are not dedicated to the specific HR-MS and are maintained even though the HR-MS is either connected state or idle state. When an HR-MS is registered at an HR-BS for receiving multicast service, multicast service flows shall be instantiated as multicast connections. An HR-MS regardless of what mode the HR-MS is currently in may receive data of multicast service flows transmitted from HR-BS. Any available FID is used for the multicast service (i.e., there are no dedicated FIDs for multicast transport connections). To ensure proper multicast operation, the Multicast Group ID and FID used for the service shall be the same for all HR-MSs on the same channel that participate in the connection in a multicast zone. Mapping of multicast service flows to corresponding Multicast Group IDs and FIDs shall be known and be the same for all HR-BSs belonging to the same HR Multicast Group Zone.

###### 17.3.9.1.1 Multicast communication establishment

The procedure of multicast communication establishment includes capacity exchange, establishment multicast connection, transmission and receiving the HR-multicast control channel as shown in Figure xxx. The procedure includes

- Capacity exchange using AAI-REG-REQ/RSP

- DSx procedure containing relevant multicast parameter to establish multicast connection

- Transmission and receiving the HR multicast control channel

To discover multicast service, HR-MS will inform HR-BS of support of multicast transmission by AAI-REG-REQ message and the HR-BS will indicate if it supports multicast for that HR-MS through AAI-REG-RSA message. The basic multicast capability exchange in AAI_REG-REQ/RSP message is described in 16.2.3.8 and 16.2.3.9.

When an HR-MS registers to receive multicast services, the serving HR-BS or the HR-MS may initiate the DSA procedure for multicast connections. The HR-MS's discovery and registration of multicast services with the HR-BS through upper layer signaling are outside the scope of this standard.

The AAI-DSA, AAI-DSC and AAI-DSD messages are used to establish, change, and delete multicast service flows respectively. The HR-BS shall send the AAI-DSA-REQ/RSP to the HR-MS with the relevant multicast parameters including Multicast Group ID.

To receive multicast data, an HR-MS receives the multicast allocation information in the multicast control channel (i.e., multicast assignment MAP).

Figure xxx – Procedure of multicast communication establishment

###### 17.3.9.1.2 Multicast communication operation in connected state

When an HR-MS moves across Multicast zone boundaries in Active Mode or Sleep Mode, the HR-MS performs the handover procedure as described in 16.2.6.3.

When the HR-MS transits to a new Multicast Zone while in Active Mode or Sleep Mode, the HR-MS shall send AAI-RNG-REQ message with Ranging Purpose Indication = 0b1110 at the target HR-BS and the ABS shall include Multicast Group ID and FID Update in AAI-RNG-RSP parameters to provide updated service flow management encodings for any affected E-MBS flow as part of the handover procedure.

###### 17.3.9.1.3 Multicast communication operation in idle state

When an HR-MS in Idle state moves to an HR-BS which does not belong to HR-MS' previous Multicast Group Zone, the HR-MS is expected to update the multicast service flow management encodings at that HR-BS to provide continuous reception of multicast content. The HR-MS may obtain the multicast information in the target Multicast zone through broadcast messages in the Multicast Zone of the service HR-BS. If the idle HR-MS has not received such information from the serving Multicast Zone, the HR-MS shall use location update procedure to acquire updated multicast service flow management encodings. In order to perform the multicast location update process, the HR-MS shall transmit AAI-RNG-REQ message with Ranging Purpose Indication = 0b1110. In response to the request for multicast location update, the HR-BS shall transmit AAI-RNG-RSP message which may include the Multicast Group Zone identifier, Multicast Group ID, and FID to provide update service flow management encodings for any affected multicast flow(s).

##### 17.3.9.2 Multicast protocol features and functions

###### 17.3.9.2.1 Downlink control channel for multicast communication

HR-multicast control channel (i.e., HR-Multicast DL Assignment A-MAP IE) carries configuration information (including allocation/change/releasement) for multicast communication for one multicast zone in an HR-BS. In HR-Multicast DL Assignment A-MAP, allocation period indicates a period of persistent allocation of multicast resource and Lifetime is a timer indicating the next instance of HR-Multicast DL-Assignment A-MAP IE. Unless the Lifetime expires, this HR-Multicast DL Assignment A-MAP does not change during the allocation duration. At the time the Lifetime expires, the HR-Multicast DL Assignment A-MAP shall change or release the allocation.

Table xx – HR-Multicast DL Assignment A-MAP IE*

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| HR-Multicast_DL_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | HR-Multicast DL Assignment A-MAP IE |
| Allocation period | 2 | Period of persistent allocation of multicast resource. If (Allocation Period==0b00), it indicates the deallocation of persistent resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 6 frames |
| If (Allocation Period == 0b00) { |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number for AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD (long TTI) |
| Reserved | 22 |  |
| } else if(Allocation Period != 0b00) { |  |  |
| Isizeoffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Multi-layer encoding 0b11: CDR |
| If (MEF ==0b01) { |  | Parameter for vertical encoding |
| Mt | 3 | Number of streams in transmission Mt<=Nt Nt: Number of transmit antennas at the HR-BS 0b000: 1 stream 0b001: 2streams 0b010: 3streams 0b011: 4streams 0b100: 5streams 0b101: 6streams 0b110: 7streams 0b111: 8streams |
| Reserved | 1 |  |
| } else if (MEF == 0b10) { |  | Parameters for multi-layer encoding |
| Si | 4 | Index to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO, and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation = QPSK 0b0001: 2 streams with PSI=stream1 and other modulation = 16QAM 0b0010: 2 streams with PSI=stream1 and other modulation = 64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 streams with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number for AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD (long TTI) |
| Lifetime(L) | 4 | Indicates the time to transmit next HR-Multicast DL Assignment A-MAP and the information of this HR-Multicast DL Assignment A-MAP does not change during the allocation duration. The next HR-Multicast DL Assignment A-MAP is at the superframe whose superframe number, Nsuperframe, satisfies the following condition. Nsuperframe modulo L+1 = 0 |
| Reserved | 7 |  |
| } |  |  |
| } |  |  |

*A 16bit CRC is generated based on the randomized contents of the HR-Multicast DL Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask (with masking prefix = 0b0 and message type indicator = 0b010) generated according to Table 843 as describe in 16.3.5.5.2.4.

##### 17.3.9.3 Multicast key management

Multicast key is managed as described in 17.3.10.x.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]