| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposals of HR-MS Forwarding in 16n Network |  |
| Date Submitted | 2011-05-09 |  |
| Source(s) | Sungcheol Chang, Eunkyung Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon ETRI | E-mail: scchang@etri.re.kr |
| Re: | Call for Comments on the 802.16n Amendment Working Document < 80216n-10_0049r1.pdf > |  |
| Abstract | This provides AWD text proposals for HR-MS forwarding in 16n network |  |
| Purpose | To be discussed and adopted by 802.16 TGn |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Text Proposals of HR-MS Forwarding in 16n Network

Sungcheol Chang, Eunkyung Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon

ETRI

# Introductions

This contribution provides AWD text proposals of HR-MS forwarding operation. A entities configuration of HR-MS forwarding functionality consists of an HR-BS, a forwarding HR-MS, and one or more HR-MSs. The forwarding HR-MS is under HR-BS coverage, but near boundary of HR-BS coverage. One or more HR-MSs are out of BS coverage. The forwarding HR-MS has two interfaces: 1) infrastructure air interface between HR-BS and a forwarding HR-MS; 2) Direct communication air interface between a forwarding HR MS and HR-MSs out of the BS coverage. The one follows infrastructure communication protocol of WirelessMAN-Advanced Air interface and the other follows direct communication protocol of WirelessMAN HR Advanced Air interface.

The direct communication protocol is out of scope of this contribution, but a set of the following contributions are proposed for 16n direct communication[6][7][8]:

- PHY Frame and Physical Structure

- PHY Control Structure

- MAC Control procedure

A forwarding HR-MS has an interworking function between the two interfaces. To do this, HR-BS has information of HR-MSs out of BS coverage and provides additional forwarding functionalities in infrastructure air interface. When HR-BS supports an extension of infrastructure communication using direct communication to HR-MS out of BS coverage, additional transport connection is established between HR-BS and the forwarding HR-MS. The forwarding HR-MS forwards all the data packets on the additional transport connection from HR-BS to HR-MS out of BS coverage. The forwarding HR-MS forwards all the data packets from HR-MS out of BS coverage to HR-BS using the additional transport connection. Figure 1 illustrates that a forwarding HR-MS has two types of radio interfaces: infrastructure communication interface and direct communication interface.

Figure 1: A configuration for HR-MS forwarding functionality.

The additional transport connection between HR-BS and the forwarding HR-MS is called as forwarding transport connection. It supports forwarding functions at an infrastructure air interface and the forwarding transport connection carries MAC PDUs at infrastructure communication interface, which minimizes the modification of MAC PDU format and its transmission operation at infrastructure communication interface. The forwarding transport connections are managed using the dynamic service flow management procedures and modifications of service flow description is required to identify service flow used for forwarding traffic data at forwarding MS. Figure 2 illustrates that the forwarding HR-MS, MS A, maps forwarding transport connections, FID k and FID n, to direct communication links, a direct communication link between MS A and MS B and a direct communication link between MS A and MS C, respectively.

Figure 2: MS A maps FID k and FID n to direct communication links, respectively.

The HR-BS maintains the list of HR-MSs out of coverage so that the HR-MSs may be paged or be connected to network via forwarding HR-MS. The procedure to update the list of HR-MSs follows:

- The MS list is advertised from the BS to all the MSs within the coverage.

- The forwarding MS detects an MS not listed in the MS list. The MS is out of service coverage and is allowed only direct communication between the forwarding MS and the MS.

- The forwarding MS updates the MS list using DM-List Update Request/Response (DM-LU-REQ/RSP).

Figure 3 illustrates that a forwarding HR-MS updates the list of HR-MSs out of BS coverage when it detects some HR-MSs out of BS coverage are missed in the list of HR-MSs.

Figure 3: A procedure to update the list of HR-MSs out of coverage for HR-MS forwarding functionality.

MAC management messages used for support of HR-MS forwarding functionality at infrastructure communication interface has FWD (forwarding) prefix in their names. The proposed MAC management procedures consist of the followings:

- Paging

- Link management with establishment and release procedures

- Flow management with addition, change, and deletion procedures of flow

Figure 4 illustrates procedures and MAC management messages for support of HR-MS forwarding functionality. A procedure and its MAC management messages are defined at both infrastructure communication and direct communication interfaces so that MAC management messages at infrastructure communication interface are related to ones at direct communication interface.

Figure 4: Procedures and MAC management messages at infrastructure communication and direct communication interfaces.

Each control procedure consists of two types of control signal procedures: the one is for peer-to-peer transmission (unicast), and the other is for point-to-multipoint transmission (multicast). A MAC control procedure for peer-to-peer transmission is a two-way handshake with request and response MAC management messages generally. A MAC control procedure for point-to-multipoint transmission is a one-way transmission of command MAC management messages. For an example, two types of link establishment or dynamic service flow addition procedures are shown for peer-to-peer communication and point-to-multipoint communication in Figure 5, respectively.

Figure 5: peer-to-peer procedures (left) and point-to-multipoint procedures (right) of link establishment or dynamic service flow addition procedures.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy1: Adapt the following change in Section 17.3.3 in the 802.16n AWD]

17.3.3 HR-MS Forwarding to network

[note: DC RG output provides text proposals for the following sections:

17.3.3.1 General description]

17.3.3.2 HR-MS discoveries

A forwarding HR-MS shall maintain a list of HR-MSs that are in communication range.

[note: How to get MS information on a direct communication link is in scope of direct communication]

An HR-BS shall maintain a list of HR-MSs that are collected from forwarding HR-MSs for HR-MS forwarding. An HR-BS broadcast the HR-MS list for HR-MS forwarding to forwarding HR-MSs. When new HR-MS is added or HR-MSs are deleted, the forwarding HR-MS shall update the HR-MS list by an exchange of MAC Management message with HR-BS.

17.3.3.3 Connection management

A forwarding unicast connection is a unicast connection between HR-BS and forwarding HR-MS to support HR-MS forwarding function.

A forwarding multicast connection is a multicast connection between HR-BS and forwarding HR-MS to support HR-MS forwarding function.

17.3.3.3.1 Control connection

All the MAC control messages to support HR-MS forwarding function are carried on control connections of infrastructure communication between HR-BS and forwarding HR-MS. The peer-to-peer MAC control messages are carried on a unicast control connection. The point-to-multipoint MAC control messages are carried on a multicast control connection.

17.3.3.3.2 Transport connections

A unicast transport connection between HR-BS and forwarding HR-MS is established to forward data traffic in one-way from HR-BS to forwarding HR-MS or vice versa.

A multicast transport connection between HR-BS and forwarding HR-MS is established to forward data traffic in one-way from HR-BS to forwarding HR-MSs.

Each unicast or multicast transport connection, which is established for supporting HR-MS forwarding, carries forwarding data packets. When HR-BS sends data packets on a transport connection, forwarding HR-MS discriminates the data packets with type of transport connection and its FID and forwards the data packets on a direct communication link. When a forwarding HR-MS receives data packets on a direct communication link, the forwarding HR-MS forwards the data packets on a transport connection toward HR-BS.

17.3.3.4 Forwarding Link management

A forwarding link is a mapping between two MAC peers of HR-BS and forwarding HR-BS. The forwarding links consists of several forwarding transport connections and a control connection.

17.3.3.4.1 Forwarding Link establishment

When a forwarding HR-MS are requested to establish a forwarding transport connection from HR-MSs out of BS’s coverage at first, the forwarding HR-BS establishes a forwarding link. The control connection is the existing control connection established for infrastructure communication and is not established during forwarding link establishment. During forwarding link establishment, a forwarding transport connection shall be established and multiple forwarding transport connections may be established.

An HR-BS and a forwarding HR-MS establish a unicast forwarding link with two-way handshake of FWD-RNG-REQ/RSP messages.

An HR-BS and forwarding MSs establish a multicast forwarding link with one-way FWD-RNG-CMD message.

17.3.3.4.2 Forwarding Link release

The forwarding link is terminated with forwarding link release. On forwarding link release, the context of forwarding link are removed at both HR-BS and forwarding HR-MSs. All the forwarding transport connections are terminated automatically, but the control connection is not terminated.

An HR-BS and a forwarding HR-MS release a unicast link with FWD-DREG-REQ/RSP messages.

An HR-BS and forwarding HR-MSs release a multicast link with FWD-DREG-CMD messages.

17.3.3.5 QoS management

QoS concept of HR-MS forwarding shall be the same as described as in section 16.2.12 with the exception of QoS described in this section.

17.3.3.5.1 Service Flow Management

Service flows may be created, changed, or deleted. This is accomplished through a series of MAC management messages referred to as FWD-DSA, FWD-DSC, and FWD-DSD. The FWD-DSA messages create a new service flow. The FWD-DSC messages change an existing service flow. The FWD-DSD messages delete an existing service flow.

17.3.3.6 Paging

An HR-BS pages an HR-MS out of BS’s coverage via forwarding HR-MSs.

The HR-BS selects a forwarding HR-MS that is in coverage of direct communication with a paged HR-MS and sends a forwarding paging message, FWD-PAG-ADV on unicast control channel. The forwarding HR-MS forwards this paging message on direct communication link. When the forwarding HR-MS receives a request of direct communication link establishment from the target HR-MS, the forwarding HR-MS acquires the paging success. This paging step on unicast control channel fails when the HR-BS does not receive an establishment request of direct communication link from the forwarding HR-MS within a timer expiration

After that, the HR-BS broadcasts a forwarding paging messages, FWD-PAG-ADV. All the forwarding HR-MSs receive this paging message and forward it on direct communication links.

17.3.3.7 Support for two hop communication

[note: text proposals for two hop communication will be after one hop operation is defined]

## [Remedy2: Adapt the following change in Section 16.2.3 in the 802.16n AWD]

16.2.3 MAC Control messages

Table 677 – MAC control messages

| No. | Functional Areas | Message names | Message description | Secuirty | Connection |
| --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … |
| n+1 | Forwarding Link Establishment | FWD-RNG-REQ | Ranging Request |  | Unicast |
| n+2 | Forwarding Link Establishment | FWD-RNG-RSP | Ranging Response |  | Unicast |
| n+3 | Forwarding Link Release | FWD-DREG-REQ | Deregistration Request |  | Unicast |
| n+4 | Forwarding Link Release | FWD-DREG-RSP | Deregistration Response |  | Unicast |
| n+5 | Forwarding Link Release | FWD-DREG-ACK | Deregistration Acknowledgement |  | Unicast |
| n+6 | Forwarding Flow Management | FWD-DSA-REQ | Dynamic Service Addition Request |  | Unicast |
| n+7 | Forwarding Flow Management | FWD-DSA-RSP | Dynamic Service Addition Response |  | Unicast |
| n+8 | Forwarding Flow Management | FWD-DSA-ACK | Dynamic Service Addition Acknowledgement |  | Unicast |
| n+9 | Forwarding Flow Management | FWD-DSC-REQ | Dynamic Service Change Request |  | Unicast |
| n+10 | Forwarding Flow Management | FWD-DSC-RSP | Dynamic Service Change Response |  | Unicast |
| n+11 | Forwarding Flow Management | FWD-DSC-ACK | Dynamic Service Change Acknowledgement |  | Unicast |
| n+12 | Forwarding Flow Management | FWD-DSD-REQ | Dynamic Service Deletion Request |  | Unicast |
| n+13 | Forwarding Flow Management | FWD-DSD-RSP | Dynamic Service Deletion Response |  | Unicast |
| n+14 | Forwarding Flow Management | FWD-DSD-ACK | Dynamic Service Deletion Acknowledgement |  | Unicast |
| n+15 | Forwarding Paging | FWD-PAG-ADV | Paging |  | Broadcast |
| n+16 | Forwarding MS List | FWD-MS-ADV | MS list Advertisement |  | Broadcast or multicast or unicast |
| n+17 | Forwarding MS list Update | FWD-LU-REQ | MS List Update Request |  | Unicast |
| n+18 | Forwarding MS list Update | FWD-LU-RSP | MS List Update Response |  | Unicast |
| n+19 | Forwarding Link Establishment | FWD-RNG-CMD | Ranging Command |  | Multicast |
| n+20 | Forwarding Link Release | FWD-DREG-CMD | Deregistration Command |  | Multicast |
| n+21 | Forwarding Flow Management | FWD-DSA-CMD | Dynamic Service Addition Command |  | Multicast |
| n+22 | Forwarding Flow Management | FWD-DSC-CMD | Dynamic Service Addition Command |  | Multicast |
| n+23 | Forwarding Flow Management | FWD-DSD-CMD | Dynamic Service Addition Command |  | Multicast |

6.2.3.64 FWD-RNG-REQ

A forwarding HR-MS transmits a FWD-RNG-REQ message to trigger establishing a peer-to-peer link of direct communication.

6.2.3.65 FWD-RNG-RSP

An HR-BS transmits a FWD-RNG-RSP message in response to a received FWD-RNG-REQ.

6.2.3.66 FWD-DREG-REQ

A forwarding HR-MS transmits a FWD-DREG-REQ message to trigger releasing either a peer-to-peer link or a point-to-multipoint link of direct communication.

6.2.3.67 FWD-DREG-RSP

An HR-BS transmits a FWD-DREG-RSP message in response to a received FWD-DREG-REQ.

6.2.3.69 FWD-DREG-ACK

A forwarding HR-MS transmits a FWD-DREG-ACK message in response to a received FWD-DREG-RSP.

6.2.3.70 FWD-DSA-REQ

A forwarding HR-MS or HR-BS transmits a FWD-DSA-REQ message to trigger creating a new service flow at a direct communication link.

6.2.3.71 FWD-DSA-RSP

A corresponding HR-BS or forwarding HR-MS transmits a FWD-DSA-RSP message in response to a received FWD-DSA-REQ.

6.2.3.72 FWD-DSA-ACK

A corresponding forwarding HR-MS or HR-BS transmits a FWD-DSA-ACK message in response to a received FWD-DSA-RSP.

6.2.3.73 FWD-DSC-REQ

A forwarding HR-MS or HR-BS transmits a FWD-DSC-REQ message to trigger changing the parameters of an existing service flow at a direct communication link

6.2.3.74 FWD-DSC-RSP

A corresponding HR-BS or forwarding HR-MS transmits a FWD-DSC-RSP message in response to a received FWD-DSC-REQ.

6.2.3.75 FWD-DSC-ACK

A corresponding forwarding HR-MS or HR-BS transmits a FWD-DSC-ACK message in response to a received FWD-DSC-RSP.

6.2.3.76 FWD-DSD-REQ

A forwarding HR-MS transmits a FWD-DSD-REQ message to trigger deleting an existing service flow at a direct communication link.

6.2.3.77 FWD-DSD-RSP

An HR-BS transmits a FWD-DSD-RSP message in response to a received FWD-DSD-REQ.

6.2.3.78 FWD-DSD-ACK

A forwarding HR-MS transmits a FWD-DSD-ACK message in response to a received FWD-DSD-RSP.

6.2.3.79 FWD-PAG-ADV

An HR-BS transmits a FWD-PAG-ADV message to trigger paging an HR-MS via a forwarding HR-MS.

6.2.3.80 FWD-MS-ADV

An HR-BS transmits a FWD-MS-ADV message to advertise an MS list for HR-MS forwarding.

6.2.3.81 FWD-LU-REQ

A forwarding HR-MS trantsmit a FWD-LU-REQ message to update a MS list for HR-MS forwarding.

6.2.3.82 FWD-LU-RSP

An HR-BS transmits a FWD-LU-RSP message in response to a received FWD-LU-RSP.

6.2.3.83 FWD-RNG-CMD

An HR-BS transmits a FWD-RNG-CMD message to trigger establishing a point-to-multipoint link of direct communication.

6.2.3.84 FWD-DREG-CMD

An HR-BS transmits a FWD-DREG-CMD message to trigger releasing a point-to-multipoint link of direct communication.

6.2.3.85 FWD-DSA-CMD

An HR-BS transmits a FWD-DSA-CMD message to trigger creating a new service flow on a point-to-multipoint link of direct communication.

6.2.3.86 FWD-DSC-CMD

An HR-BS transmits a FWD-DSC-CMD message to trigger changing the parameters of an existing service flow on a point-to-multipoint link of direct communication

6.2.3.87 FWD-DSD-CMD

An HR-BS transmits a FWD-DSD-CMD message to trigger deleting an existing service flow on a point-to-multipoint link of direct communication.

[-------------------------------------------------Begin of Text Proposal----------------------------------------------------]

# References

[1] IEEE C802.16n-11/0051, “Dedicated resources allocation for direct communications in IEEE 802.16n.”

[2] IEEE C802.16n-11/0052, “Allocation of common dedicated resources for direct communications in IEEE 802.16n.”

[3] IEEE C802.16n-11/0053, “Channel Structures for Direct Communication.”

[4] IEEE C802.16n-11/0054, “Control Procedures for Direct Communication.”

[5] IEEE C802.16n-rg-11/0002, “ [DC] AWD text proposals for direct communication.”

[6] IEEE C802.16n-11/0071, “ Text Proposals of PHY Frame and Physical Structure for 16n Direct Communication.”

[7] IEEE C802.16n-11/0072, “ Text Proposals of PHY Control Structure for 16n Direct Communication.”

[8] IEEE C802.16n-11/0073, “ Text Proposals of MAC Control procedure for 16n Direct Communication.”

[9] IEEE C802.16n-11/0058, “ Forwarding Operation for Direct Communication.”

[10] IEEE C802.16n-rg-11/0018, “ [DC] AWD text proposals of forwarding operation for direct communication.”