| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Consolidated Contributions for Security Ad Hoc Group in IEEE 802.16n |  |
| Date Submitted | 2011-05-18 |  |
| Source(s) | Joseph Teo Chee Ming Institute For Infocomm Research Eunkyung Kim ETRI | E-mail: cmteo@i2r.a-star.edu.sg |
| Re: | in response to the Security Ad Hoc Group activity |  |
| Abstract | Consolidated Contributions for Security discussion in IEEE 802.16n |  |
| Purpose | Initial GRIDMAN System Requirements working document to enable capturing the requirements for the prospective IEEE 802.16n AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Consolidated Contributions for Security in IEEE 802.16n

Joseph Teo Chee Ming

Institute For Infocomm Research

Eunkyung Kim

ETRI

# Introductions

This consolidated contribution contains proposed text from all contributions submitted before/during session #72, Singapore. The following is the list of contributions that were marked for the Security. The contributions are tagged as follows:

- yellow : Security

Table 1 – List of contributions related to Security

|  | Contribution | Title | Author | Affiliation | Category | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | C80216n-11_0009r1.doc | Network-aided and Autonomous Secure Direct Communications in wireless access network | Joseph Chee Ming Teo, Jaya Shankar, Yeow Wai Leong, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang, Mar Choon Hock | Institute for Infocomm Research | Security |  |
| 2 | C80216n-11_0010r1.doc | Secure Direct Communications in wireless access network without network infrastructure | Joseph Chee Ming Teo, Jaya Shankar, Yeow Wai Leong, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang, Mar Choon Hock | Institute for Infocomm Research | Security |  |
| 3 | C80216n-11_0011r1.doc | Autonomous Secure Direct Communications in wireless access network | Joseph Chee Ming Teo, Jaya Shankar, Yeow Wai Leong, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang, Mar Choon Hock | Institute for Infocomm Research | Security |  |

The following procedure was used in consolidating the proposed texts from all contributions.

- Proposed texts are placed in the Section they address.  If the text in the contribution addresses more than one section, then the text is split and placed under the appropriate Section numbers (or categories).

- All proposed texts from different contributions that address a particular section are together.

- Existing Section numbers are in black color with a bold face.

- Existing text are colored black

- Proposed text and sections from a contribution are bound by the delimiters [Begin of Text Proposal: From C802.16n- 11/xxxx] and [End of Text Proposal: From C802.16n- 11/xxxx] where xxxx is the contribution number from where the proposed text was taken. They will be removed from the RG contribution that is going to be submitted to the TG.

- All proposed sections are colored blue with an underline and bold face.

- All proposed text are initially colored blue with an underline.

[-------------------------------------------------Begin of Text Proposal----------------------------------------------------]

# 17.2.10 Security

# 17.2.10.x Security procedures in HR-Networks

17.2.10.x.y security procedure for direct communication

The following security procedures shall be used for secure direct communication between HR-MSs.

## 17.2.10.x.y.z Autonomous Mutual Authentication of HR-MS and data security for Direct Communications

### 17.2.10.x.y.z.a Secure direct communication using pre-established shared key

In order to support secure direct communication between two or among more HR-MSs, pre-established shared key (i.e., DMK) is used.

The DMK is established prior to the start of this direct communications.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]