| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for A Procedure to Setup HR-MS to HR-MS Direct Communication with Supporting from Infrastructure Stations |  |
| Date Submitted | 2011-05-09 |  |
| Source(s) | Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming, Mar Choong Hock Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2256 E-mail: hwang@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this proposal, we proposed a procedure for HR-networks in setting up HR-MS direct communication in HR-BS or HR-RS appears in the network. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Text Proposal A Procedure to Setup HR-MS Direct Communication with Supporting from Infrastructure Station

Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming, Mar Choong Hock

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

HR-MS to HR-MS direct communication is new feature for IEEE 802.16 networks. It will be defined in the IEEE 802.16n amendment.  This feature will significantly improve the capability of HR-MS in various harsh environment and increase network throughput when local traffic dominates the network.

In 802.16n System Requirement Document [1] section 6.1.3.1, it says that “HR-MS shall provide direct communication i.e. the origination and termination of the data are at the HR-MS. Association establishment procedure of an HR-MS to another HR-MS shall be supported.”.

Since the requirement does not set constraint on whether HR-MSs have associated to an infrastructure stations, in this contribution, we assume an HR-BS is presented in the network and the HR-MSs involved in the direct communication are associated to the same HR-BS or HR-RS.

# 2. HR-MS to HR-MS Direct Communication

Supporting the HR-MS to HR-MS direct communication is required by IEEE 802.16n network. The HR-MS direct communication can happen with our without the supporting form infrastructure nodes. In this contribution, we focus on the use cases of two HR-MS either direct under the same HR-BS or the same HR-RS as figure 1 shows. We provide solutions for HR-MS direct communication link setup between HR-MSs.

Figure 1: Use cases of HR-MS to HR-MS direct communication with infrastructure stations

- Existing Signaling Procedure

With existing IEEE 802.16 protocols, direct communication between MS is not possible. If an MS want to communicate to another MS, following procedures and signaling exchanges are required to deliver data packets from one MS to another MS under the same infrastructures, for example, a BS.

Figure 2: Procedures and signaling exchange for MS to MS communication with existing 802.16 protocols

To enable the HR-MS to HR-MS direct communication, a few issues need to be considered, for example:

- Detect the feasibility of direct communication.

- Allocate resource for direct communication.

- HR-MS link synchronization

- Data transmission

- Proposed Signaling Procedure

Therefore, we may need change above procedure to setup HR-MS direct communication with help from infrastructure stations. The changes are illustrated in the Figure 2 and our explanations to the procedure are as follows:

- Since the DSA_REQ contains the source and destination IP address of the flow and the HR-BS/HR-RS can get the information of destination node based on their IP address and therefore knows whether the source and destination node can set up a direct communication link or not based on the neighbor table. The HR-BS/HR-RS should maintain a neighbor table since neighbor discovery is required by the 802.16n system.

- If a direct communication link can be setup between the source and destination node, then the HR-BS/HR-RS holds on the transmission of DSA_RSP to the source node. It allocates flow_id and radio resource for the communication based on their link quality.

- The HR-BS/HR-RS notifies the destination HR-MS of the flow information, including the flow_id and radio resource allocated, by sending a DSA_REQ.

- On receiving the DSA_REQ, the destination HR-MS should response with a DSA_RSP to the HR-BS. On receiving the DSA_RSP, the HR-BS/HR-RS should reply a DSA_ACK to the destination HR-MS. And at the same time, the HR-BS/HR-RS should send the DSA_RSP, which also including the same flow_id and radio resource allocated, to the source HR-MS.

- On receiving the DSA_RSP, the source HR-MS reply with a DSA_REQ.  At this stage, a direct communication link is considered being setup by the source and destination HR-MS with the help from HR-BS/HR-RS.

- After sending out the DSA_ACK to HR-BS/HR-RS, the source and destination HR-MS should start a link synchronization process. Ringing signal can be sent for synchronization. We propose that the source HR-MS send a ranging signal to the destination HR-MS first on the assigned direct communication link and the destination HR-MS send back a ranging signal for both ranging and acknowledgement purpose.

- After the link synchronization process is successfully performed, the source HR-MS can now send data packets to the destination HR-MS once it gets the transmission opportunity.

- The destination HR-MS may transmit ARQ information on the direct communication link also.

- A periodic synchronization between the source and destination HR-MS may be performed since the topology is dynamic. A pair of ranging signal should be exchanged between the source and destination HR-MS.

- The direct communication link may be terminated once the data transmission is finished.

Figure 3: The procedure of direct communication establishment between HR-MS

# 3. Summary

In this contribution, we provide a procedure for 802.16n HR-MS in setting up direct communication link with the help from infrastructure stations.

# 4. Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n Document (XXX --- document number)]

17.2.2 Direct communication between HR-MSs

17.2.2.x HR-MS Direct Communication with Infrastructure Stations

HR-BS shall check DSA_REQ messages received from HR-MS and determine whether HR-MS direct communication can be adopted for a flow. If a direct communication is feasible between the source and destination HR-MSs, then the HR-BS shall help the source and destination HR-MSs setting up a direct communication link through DSA signaling.

17.3.2 Direct communication between HR-MSs

17.3.2.x HR-MS Direct Communication with Infrastructure Stations

HR-BS shall check DSA_REQ messages received from HR-MS and determine whether HR-MS direct communication can be adopted for a flow. If a direct communication is feasible between the source and destination HR-MSs, then the HR-BS shall help the source and destination HR-MSs setting up a direct communication link through DSA signaling.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.