| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text related to path management from MM RG in IEEE 802.16n |  |
| Date Submitted | 2011-05-09 |  |
| Source(s) | Eunkyung Kim, Anh Tuan Hoang, MM RG co-chairs | E-mail: ekkim@etri.re.kr athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Output of RG MM related to path management in IEEE 802.16n |  |
| Purpose | Overall description of GRIDMAN path management to enable capturing the general description for the prospective IEEE 802.16n AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text related to path management from MM RG in IEEE 802.16n

Eunkyung Kim, Anh Tuan Hoang

MM RG Chairs

# Introductions

This contribution provides general description of path management as an output of MM RG based on the MM RG activity between IEEE 802.16 Session #72 and #73.

# References

- IEEE 802.16n-11/0036r6, “802.16n GRIDMAN Contributions spreadsheet for Session 72,” March 2011.

- IEEE 802.16n-11/0062, “Work Plan of Multi-Mode and Multicast (MM) Rapporteur Group (RG) between Session #72 and Session #73,” March 2011.

- IEEE 802.16n-11-rg/0001, “[MM] Consolidated Contributions for MM RG in IEEE 802.16n,” April 2011.

- IEEE 802.16n-rg-11/0059r2, “[MM] Proposed harmonized text related to Path management for MM RG in IEEE 802.16n,” May 2011.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Adapt the following change in Section 17.2.7 Path Discovery and Management in the 802.16n AWD]

17.2.7 Path Discovery and Management

HR-Network shall support establishment and maintenance of alternative paths that are used for fast network recovery in the event of disruption, e.g. when the serving HR-RS or HR-BS fails. An alternative path is determined by the serving HR-BS. In normal operation, there are no active connections on any alternative path. The alternative path can be activated when the disruption event is detected.

[Remedy2: Adapt the following change in Section 17.2.7 Path Discovery and Management in the 802.16n AWD]

17.3.7 Path Discovery and Management

HR-Network shall support establishment and maintenance of alternative paths that are used for fast network recovery in the event of disruption, e.g. when the serving HR-RS or HR-BS fails. An alternative path is determined by the serving HR-BS. In normal operation, there are no active connections on any alternative path. The alternative path can be activated when the disruption event is detected.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]