| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal of Relay Mechanism in IEEE 802.16n-GRIDMAN Network |  |
| Date Submitted | 2011-05-09 |  |
| Source(s) | Xin Zhang, Liru Lu (Alina), Mingtuo Zhou, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada NICT | E-mail: zhangxin@nict.com.sg; liru@nict.com.sg; mingtuo@nict.com.sg; hvdien@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp |
| Re: | Call for Comments for 802.16n AWD |  |
| Abstract | In this contribution, we propose text of relay mechanism for 802.16n. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal of Relay Mechanism in IEEE 802.16n-GRIDMAN Network

Xin Zhang, Liru Lu (Alina), Mingtuo Zhou

Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

NICT

# 1. Introduction

As described in the System Requirement Document (SRD), the relay function shall support all the additional functional requirements that are indicated in Section 6, which mainly include multi-mode operation and multi-hop relaying. In order to maintain backward compatibility, we keep the major part of the relay operation in 802.16m and 802.16j intact. Since 802.16j specified the multi-hop relay operation, in this document, we focus on the relay function that supports multi-mode operation, which are the relay function for HR-BS and the relay function for HR-MS in relay mode as illustrated in figure 1 and 2.

In our 802.16gman network, the support for relay is a compulsory feature. Mobile station can change its role to relay data between other mobile station or relay station and a functioning HR-BS or HR-RS. Base Station (BS) shall be able to communicate to other BS through relay function in order to provide continuous network connectivity.

In this document, relaying in 802.16gman is performed using a decode and forward paradigm. Relay mode is decided by HR-RS capability negotiation phase of the HR-RS network entry. Distributed scheduling mode is adopted here where each infrastructure station (HR-BS or HR-RS) schedules the radio resources on its subordinate link. The HR-BS notifies the HR-RSs and HR-MSs of the frame structure configuration.

# 2. Text Proposal for IEEE 802.16n AWD

Xxx

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

# 17.2.5 Relaying Operation

In WirelessMan High Reliability OFDMA air interface, support for relay is a compulsory feature. In order to provide high reliability in a degraded network, the relay function for HR-BS and HR-MS are supported so as to provide a continuous network.

# 17.3.5 Relaying Operation

In WirelessMan High Reliability Advanced air interface, support for relay is a compulsory feature. In order to provide high reliability in a degraded network, the relay function for HR-BS and HR-MS and at least 2-hop relaying function are supported so as to provide a continuous network.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]