| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposed of Standalone Network Operation in IEEE 802.16n Networks |  |
| Date Submitted | 2011-05-09 |  |
| Source(s) | Liru Lu (Alina), Ming-Tuo Zhou, Xin Zhang, Vinh Dien Hoang,Masayuki Oodo, Hiroshi Harada NICT | E-mail: liru@nict.com.sg; mingtuo@nict.com.sg; zhangxin@nict.com.sg; hvdien@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp |
| Re: | Call for Comments for 802.16n AWD |  |
| Abstract | In this contribution, we propose text of standalone network operation for 802.16n. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposed of Standalone Network Operation in IEEE 802.16n Networks

Liru Lu (Alina), Ming-Tuo Zhou, Xin Zhang

Vinh Dien Hoang,Masayuki Oodo, Hiroshi Harada

NICT

# 1. Introduction

Document IEEE 802.16n-10/0048 specifies the function requirements of HR-Network. Section 6.1.2 defines the requirements related to infrastructure Single Point of Failure immunity. Subsection 6.1.2.1 specifies the requirements for standalone network.

"HR-Network shall provide local connectivity to the HR-MSs within the coverage of the HR-BS without HR-BS’s connectivity to the backbone network.

When the HR-BS loses the backbone connection, the established service flow between HR-MSs within the coverage of the HR-BS should be maintained."

In this file, we propose the mechanisms for the operation of standalone network including

 Maintenance of local connectivity

 Entry process

 Handover process

2. Maintenance of Local Connectivity

When HR-BS loses the connectivity to the backbone network and the network becomes standalone, the local connectivity shall be provided for the mobile stations within the coverage of HR-BS. When the HR-BS loses the backbone connection, the established service flow between HR-MS within the coverage of the HR-BS should be maintained.

3. Entry Process to Standalone Network

To ensure the local connectivity to all the HR-MSs within the coverage of the HR-BS when the network is standalone, the network shall allow the entry of a new HR-MS to the network and establish the connection between HR-BS and the new HR-MS.

4. Handover Process to Standalone Network

When HR-BS lost the connectivity to the backbone network, the HR-BS and the stations within the coverage of HR-BS form a standalone network. When a mobile station is moving into the standalone network coverage area, handover procedure may be required if the mobile station is associated with another Base Station. MS shall make a decision if to perform handover procedure to the standalone network or remain at its serving BS according to the trigger condition of handover.

The detailed handover process trigger condition will be updated in another contribution document at a later stage.

# 5. Text Proposal for IEEE 802.16n AWD

Xxx

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

17.2.4 Standalone Network

For WirelessMAN HR-OFDMA air interface, when HR-BS loses the connectivity to the backbone network, the network nodes under the coverage of this HR-BS will become a standalone network. The local connectivity shall be provided for the HR-MS within the coverage of HR-BS. When the HR-BS loses the backbone connection, the established service flow between HR-MS within the coverage of the HR-BS shall be maintained.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]