| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | [Text Proposal of Standalone Network Operation in IEEE 802.16n Networks] |  |
| Date Submitted | [2011-05-18] |  |
| Source(s) | Liru Lu (Alina), Ming-Tuo Zhou Xin Zhang, Vinh Dien Hoang, Masayuki Oodo, Hiroshi Harada(NICT) | Voice: [65-67711006] E-mail: [liru@nict.com.sg; mingtuo@nict.com.sg; zhangxin@nict.com.sg; hvdien@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp] |
| Re: | [Call for contributions for the 802.16n Amendment Working Draft Base Contribution: IEEE 802.16n-10/0048] |  |
| Abstract | [This document proposes some mechanisms to be used for standalone network operations.] |  |
| Purpose | [To be discussed in 802.16n Task Group.] |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Mechanisms for Standalone Network Operation

Liru Lu (Alina), Ming-Tuo Zhou, Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

NICT

# Introduction

Document IEEE 802.16n-10/0048 specifies the function requirements of HR-Network. Section 6.1.2 defines the requirements related to infrastructure Single Point of Failure immunity. Subsection 6.1.2.1 specifies the requirements for standalone network.

"HR-Network shall provide local connectivity to the HR-MSs within the coverage of the HR-BS without HR-BS’s connectivity to the backbone network.

When the HR-BS loses the backbone connection, the established service flow between HR-MSs within the coverage of the HR-BS should be maintained."

In this file, we propose the mechanisms for the operation of standalone network including

- Maintenance of local connectivity

- Entry process

- Handover process

# Maintenance of Local Connectivity

When HR-BS loses the connectivity to the backbone network and the network becomes standalone, the local connectivity shall be provided for the mobile stations within the coverage of HR-BS. When the HR-BS loses the backbone connection, the established service flow between HR-MS within the coverage of the HR-BS should be maintained.

# Entry Process to Standalone Network

To ensure the local connectivity to all the HR-MSs within the coverage of the HR-BS when the network is standalone, the network shall allow the entry of a new HR-MS to the network and establish the connection between HR-BS and the new HR-MS.

# Handover Process to Standalone Network

When HR-BS lost the connectivity to the backbone network, the HR-BS and the stations within the coverage of HR-BS form a standalone network. When a mobile station is moving into the standalone network coverage area, handover procedure may be required if the mobile station is associated with another Base Station. MS shall make a decision if to perform handover procedure to the standalone network or remain at its serving BS according to the trigger condition of handover.

# Proposed Text

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

17.2.4 Standalone Network

For WirelessMAN HR-OFDMA air interface, when HR-BS lost the connectivity to the backbone network and the neighboring HR-BSs, the network stations under the coverage of this HR-BS shall form a standalone network. The local connectivity shall be provided for the HR-MS within the coverage of affected HR-BS. The established service flow between HR-MS within the coverage of the affected HR-BS shall be maintained.

17.2.4.1 Maintenance of Local Connectivity

For maintenance of local connectivity, all the HR-BSs shall maintain a network topology table of HR-MS/HR-RS within its coverage area. The network topology table shall be updated periodically by broadcasting STN-REQ message from HR-BS and receiving acknowledgement message STN-ACK from HR-MS or HR-RS within its coverage area.

17.2.4.2 Entry Process for Standalone Network

The HR standalone network with WirelessMAN HR-OFDMA air interface shall allow the entry of an unassociated HR-MS into the standalone network and establish the connection with standalone network HR-BS. The unassociated HR-MS is referred to the HR-MS which is not associated with any Base Station.

17.3.4 Standalone Network

For WirelessMAN HR Advanced air interface, when HR-BS lost the connectivity to the backbone network and the neighboring HR-BSs, the network nodes under the coverage of this HR-BS shall form a standalone network. The local connectivity shall be provided for the mobile stations within the coverage of Base station. When the Base Station loses the backbone connection, the established service flow between mobile stations within the coverage of the base station shall be maintained.

17.3.4.1 Maintenance of Local Connectivity

For maintenance of local connectivity, all the HR-BSs shall maintain a network topology table of HR-MS/HR-RS within its coverage area. The network topology table shall be updated periodically by broadcasting a STN-REQ message from HR-BS and receiving acknowledgement message STN-ACK from HR-MS or HR-RS within its coverage area.

The maintenance of local connectivity for standalone network with WirelessMAN HR Advanced air interface shall according to the process defined in section 17.2.4.1

17.3.4.2 Entry Process for Standalone Network

The HR standalone network with WirelessMAN HR Advanced air interface shall allow the entry of an unassociated HR-MS into the standalone network and establish the connection with standalone network HR-BS. The unassociated HR-MS is referred to the HR-MS which is not associated with any Base Station.

The entry process is as defined in Section 17.2.4.2.

----------------------------------------------------- End of Proposed Text ------------------------------------------------------