| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fast Network Entry for Alternative Path |  |
| Date Submitted | 2011-05-09 |  |
| Source(s) | Wai Leong Yeow, Hoang Anh Tuan, Haiguang Wang, Shoukang Zheng, Jaya Shankar, Joseph Teo Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2254 E-mail: wlyeow@i2r.a-star.edu.sg |
| Re: | Call for Comments on the 802.16n Amendment Working Document <80216n-10_0049r1.pdf> |  |
| Abstract | We propose mechanisms to facilitate fast network entry for stations using alternative paths. |  |
| Purpose | To be discussed and adopted in TGn AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Fast Network Entry for Alternative Path

Wai Leong Yeow, Hoang Anh Tuan, Haiguang Wang, Shoukang Zheng, Jaya Shankar, Joseph Teo

Institute for Infocomm Research

# Introduction

In the SRD Section 6.1.3.4 (page 6, line 157), alternative paths are used for fast recovery in events of disruption. Recovery cannot be fast if the affected HR-RS or HR-MS uses regular network entry. Scanning and determining physical channel parameters of the neighbor station along the alternative path consumes time. Furthermore, it is likely to have several other stations affected by the same disruption event, resulting in high contention and wasted bandwidth if regular network entries are performed.

# Summary

We propose mechanisms to facilitate fast network entry for alternative paths. In the event of disruption when the alternative path of an HR-MS or HR-RS is activated, network entry can be accelerated for fast recovery.

# Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

17.2.7 Path Discovery and Management

17.2.7.x Fast Network Entry along Alternative Path

TBD.

17.3.7 Path Discovery and Management

17.3.7.x Fast Network Entry along Alternative Path

TBD.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.