| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HR-MS Neighbor Discovery – Use case 1: Registered HR-MSs discover each other |  |
| Date Submitted | 2011-05-09 |  |
| Source(s) | Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Zhongding Lei, Shoukang Zheng, Yeow Wai Leong, Teo Joseph Chee Ming, Mar Choon Hock Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2256 E-mail: hwang@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this document, we discuss approaches for HR-MS neighbor discovery in 802.16n |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

HR-MS Neighbor Discovery – Use case 1: Registered HR-MSs discover each other

Haiguang Wang, Hoang Anh Tuan, Jaya Shankar,

Zhongding Lei, Shoukang Zheng, Yeow Wai Leong,

Teo Joseph Chee Ming, Mar Choon Hock

Institute for Infocomm Research (I2R)

# 1. Introduction

In Section 6.1.3.3 of 802.16n System Requirement Document [1], it is stated that “HR-Network shall support neighbor and path discovery between HR-MSs as well as between HR-MS and HR-Network infrastructure stations”. In fact, neighbor discovery by HR-MS will be one of the enabling mechanisms to support other advanced features of 802.16n such that HR-MS direct communications (DCm), HR-MS forwarding to network (FTN), path discovery and management, and fast network recovery.  This contribution discusses HR-MS neighbor discovery under the scenario when registered HR-MSs discover their neighbors.

# 2. Design Considerations

When designing HR-MS neighbor discovery, the following criteria should be taken into account:

- Minimizing changes to the existing 802.16 specifications;

- Reliable but incurring low overhead and complexity;

- Allowing flexible scheduling and configuration.

3. Proposed Approach

By registered HR-MSs, we refer to those stations that have completed network entry and are fully under control of a serving HR-BS/HR-RS.

For these registered HR-MSs to discover each other, the serving HR-BS/HR-RS can schedule some HR-MSs to broadcast some predefined self-advertizing (PSA) signals so that other HR-MSs can try to receive and verify their neighbor relationship. Upon detecting one or a number of neighbors, an HR-MS shall report the information to its serving HR-BS/HR-RS.

There are different choices for the PSA signals used for neighbor discovery. Current 802.16-2009 and 802.16m specified various predefined signal sequences, including: DL preambles, PA/SA preambles, Midamble, Non-synchronized/synchronized ranging preambles, Bandwidth request preambles, and Channel sounding sequences. To minimize changes to the existing specifications, it is desirable that the PSA signal be selected among these predefined signals of 802.16-2009 and/or 802.16m.

For the purpose of low-cost, fast, and flexible neighbor discovery, we propose to use ranging sequences as PSA signals. This can be carried out in the following steps:

- The serving HR-BS/HR-RS schedules one or multiple registered HR-MSs to broadcast dedicated ranging sequences in dedicated ranging channels. HR-MS_i is going to broadcast ranging sequence s_i in ranging channel c_k. Two HR-MSs may share the same ranging sequence or the same ranging channel, but not both. The ranging preambles and/or channels shall not overlap with those allocated for normal initial and periodic ranging. At the same time, the serving HR-BS/HR-RS also schedules some other HR-MSs to listen on those ranging channels scheduled for PSA signals.

- Each HR-MS that is scheduled to receive PSA signals shall determine what ranging sequences it can properly decode, together with related information such as estimations of time/frequency offsets and signal strength.

- The receiving HR-MSs then report their measurements to the serving HR-BS/HR-RS.

The reported measurements of transmitted ranging-based PSA signals shall allow the serving HR-BS/HR-RS to determine the neighbor relationship among registered HR-MSs. Construction of one-hop neighborhood information can also be done at HR-MSs in a distributed manner.

Based on reported information, HR-BS/HR-RS may request the HR-MSs to carry out extra scanning and measurements so that other activities such as HR-MS direct communications (DCm) and HR-MS forwarding to network (FTN) can be initiated.

Some advantages of using ranging preambles as PSA signals in neighbor discovery are:

- Using signal sequences that have been defined in existing 802.16 specifications. The added capability here is for HR-MS to receive and process the ranging-based PSA signals. This capability is, in fact, within the scope of 802.16n, as HR-MS are required to support mode change to HR-BS and HR-RS.

- Consuming low resource, as compared to other choices of, say, using full-bandwidth preambles.

- Catering for the lack of pair-wise synchronization among registered HR-MSs.

In the above process, the serving HR-BS/HR-RS can schedule registered HR-MSs to transmit PSA signals in either DL or UL area. For DL PSA scheduling, the receive-to-transmit switching time for transmitting HR-MSs must be taken into account. For UL PSA scheduling, the transmit-to-receive switching time for receiving HR-MSs must be taken into account.

# 4. Text Proposal for IEEE 802.16n AWD

Xxx

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n Document (XXX places)]

17.2.7 Path Discovery and Management

17.2.7.2 Neighbor Discovery for Registered HR-MSs

For registered HR-MSs to discover each other, the serving HR-BS/HR-RS shall schedule some HR-MSs to broadcast predefined self-advertizing (PSA) signals so that other HR-MSs can try to receive and verify their neighbor relationship.

Ranging sequences shall be used as PSA signals, and the process of neighbor discovery for registered HR-MSs is as follows:

- The serving HR-BS/HR-RS schedules one or multiple registered HR-MSs to broadcast dedicated ranging sequences in dedicated ranging channels. Two HR-MSs may share the same ranging sequence or the same ranging channel, but not both. The ranging preambles and/or channels shall not overlap with those allocated for normal initial and periodic ranging. At the same time, the serving HR-BS/HR-RS also schedules some other HR-MSs to listen on those ranging channels scheduled for PSA signals.

- Each HR-MS that is scheduled to receive PSA signals shall determine what ranging sequences it can properly decode, together with related information such as estimations of time/frequency offsets and signal strength.

- The receiving HR-MSs then report their measurements to the serving HR-BS/HR-RS.

The serving HR-BS/HR-RS shall determine neighbor topology based on reported measurements of transmitted PSA signals. Construction of one-hop neighborhood topology can also be done at HR-MSs in a distributed manner. How HR-BS/HR-RS/HR-MS construct neighbor topology is outside of the scope of this standard.

Based on reported information, HR-BS/HR-RS can request the HR-MSs to carry out extra scanning and measurements so that other activities such as HR-MS direct communications (DCm) and HR-MS forwarding to network (FTN) can be initiated.

17.3.7 Path Discovery and Management

17.3.7.2 Neighbor Discovery for Registered HR-MSs

For registered HR-MSs to discover each other, the serving HR-BS/HR-RS shall schedule some HR-MSs to broadcast predefined self-advertizing (PSA) signals so that other HR-MSs can try to receive and verify their neighbor relationship.

Ranging sequences shall be used as PSA signals, and the process of neighbor discovery for registered HR-MSs is as follows:

- The serving HR-BS/HR-RS schedules one or multiple registered HR-MSs to broadcast dedicated ranging sequences in dedicated ranging channels. Two HR-MSs may share the same ranging sequence or the same ranging channel, but not both. The ranging preambles and/or channels shall not overlap with those allocated for normal initial and periodic ranging. At the same time, the serving HR-BS/HR-RS also schedules some other HR-MSs to listen on those ranging channels scheduled for PSA signals.

- Each HR-MS that is scheduled to receive PSA signals shall determine what ranging sequences it can properly decode, together with related information such as estimations of time/frequency offsets and signal strength.

- The receiving HR-MSs then report their measurements to the serving HR-BS/HR-RS.

The serving HR-BS/HR-RS shall determine neighbor topology based on reported measurements of transmitted PSA signals. Construction of one-hop neighborhood topology can also be done at HR-MSs in a distributed manner. How HR-BS/HR-RS/HR-MS construct neighbor topology is outside of the scope of this standard.

Based on reported information, HR-BS/HR-RS can request the HR-MSs to carry out extra scanning and measurements so that other activities such as HR-MS direct communications (DCm) and HR-MS forwarding to network (FTN) can be initiated.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.