| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Relay Function for HR-BS |  |
| Date Submitted | 2011-05-09 |  |
| Source(s) | Hoang Anh Tuan, Wai Leong Yeow, Haiguang Wang, Shoukang Zheng, Jaya Shankar, Joseph Teo Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this document, we discuss the operation of HR-BS when switching to HR-RS mode |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Relay Function for HR-BS

Hoang Anh Tuan, Wai Leong Yeow, Haiguang Wang,

Jaya Shankar, Joseph Teo

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

We discuss the scenario in which BS has backhaul failure and need to switch to RS mode to connect to a neighboring BS in order to maintain connectivity for its subordinate stations.

# 2. Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

17.2.1 Multi-mode Operations

17.2.1.4 Relay function for HR-BS

When experiencing backhaul failure, HR-BS can switch mode to HR-RS to connect to another nearby HR-BS (that has backbone access). Prior to/during/and after carrying out mode switching, the HR-BS shall perform appropriate PHY-level configurations and MAC-level control/signaling to maintain connectivity for its subordinate stations. To the serving HR-BS, the affected HR-BS can be regarded as a normal HR-RS. To the subordinate stations of the affected HR-BS, the affected HR-BS can be regarded as either an HR-BS or an HR-RS.

17.3.1 Multi-mode Operations

17.3.1.4 Relay function for HR-BS

When experiencing backhaul failure, HR-BS can switch mode to HR-RS to connect to another nearby HR-BS (that has backbone access). Prior to/during/and after carrying out mode switching, the HR-BS shall perform appropriate PHY-level configurations and MAC-level control/signaling to maintain connectivity for its subordinate stations. To the serving HR-BS, the affected HR-BS can be regarded as a normal HR-RS. To the subordinate stations of the affected HR-BS, the affected HR-BS can be regarded as either an HR-BS or an HR-RS.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.