| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal of Self-Coexistence Mechanism for 802.16n Networks |  |
| Date Submitted | 2011-05-09 |  |
| Source(s) | Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada NICT Yih-Guang Jan, Yang-Han Lee Tamkang University (TKU) Hsien-Wei Tseng De Lin Institute of Technology (DLIT) | E-mail: mingtuo@nict.com.sg; zhangxin@nict.com.sg; liru@nict.com.sg;hvdien@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp yihjan@yahoo.com hwtseng@mail.dlit.edu.tw |
| Re: | Call for Comments for 802.16n AWD |  |
| Abstract | AWD text proposal of Self-Coexistence Mechanism for 802.16n networks |  |
| Purpose | To be discussed and adapted by TGn for draft 802.16n AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Text proposal of Self-Coexistence Mechanism for 802.16n Networks

Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada

NICT

Yih-Guang Jan, Yang-Han Lee

Tamkang University (TKU)

Hsien-Wei Tseng

De Lin Institute of Technology

# Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to AWD

--------------Start of Text Proposal --------------------------------------------------------------------

3. Definitions

Coexistence: Coexistence is a state by which multiple wireless communications systems in same vicinity share a same radio frequency channel while minimizing harmful interference to each other by appropriate measures.

Self-coexistence: In HR network, self-coexistence is coexistence of multiple HR cells.

Self-coexistence mode: Self-coexistence mode is an operation mode of HR network, in which multiple HR cells share the same frequency channel and each coexisting HR cell operates on one or several different frames of a virtual superframe.

--------------End of Text Proposal ---------------------------------------------------------------------