| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | DC RG Harmonized Text on HR-MS Direct Communication and Forwarding to Network |  |
| Date Submitted | 2011-05-09 |  |
| Source(s) | Sungcheol Chang, Ming-Tuo Zhou DC RG co-chairs | E-mail: scchang@etri.re.kr, mingtuo@nict.com.sg |
| Re: | In response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Output of DC RG related to HR-MS DC and FTN in IEEE 802.16n |  |
| Purpose | To be discussed and adopted by 16n working group for 16n AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# DC RG Harmonized Text on HR-MS Direct Communication and Forwarding to Network

Sungcheol Chang, Ming-Tuo Zhou

DC RG Co-chairs

# Introductions

This contribution provides AWD text proposal of general description of HR-MS direct communication and forwarding to network. It is harmonized text of DC RG based on activity like email discussion and straw poll between IEEE 802.16 Session #72 and #73.

# References

- IEEE C802.16n-11/0061r1, “Work Plan of Direct Communication (DC) Rapporteur Group between Session #72 and Session #73,” March 28 2011.

- IEEE C802.16n-11/0054r3, “[DC] Proposal of Harmonized Text of HR-MS Direct Communication and HR-MS Forwarding to Network for AWD of IEEE 802.16n,” April 25 2011.

- IEEE C80216n-rg-11/0045r2, “[DC] Consolidated Email Discussion on Usages and Frame Structure for DC of GRIDMAN” April 25 2011.

- IEEE C80216n-rg-11/0053, “[DC] Consolidated Email Discussion on Usages and Frame Structure for DC of GRIDMAN” April 24 2011.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

17.2.2 Direct communication between HR-MSs

17.2.2.1 General Description

In HR-MS direct communication, the two communicating HR-MSs are the source and the sink of data. The data packets are passed from upper layers to MAC at the source HR-MS and back to upper layers at the sink HR-MS. Data packets are exchanged between the two HR-MSs directly or by passing through another HR-MS.

HR-MS direct communication is applicable when 1) the two HR-MSs are in coverage of and are associated to an HR infrastructure station; 2) one HR-MS is in coverage of and associated to an HR infrastructure station, while the other HR-MS is out of coverage of any HR infrastructure stations; 3) the two HR-MSs are out of coverage of any HR infrastructure stations.

Resource for HR-MS direct communication can be allocated by the HR infrastructure station for cases (1) and (2). When HR infrastructure station does not exist (case 3), resource allocation of HR-MS direct communication is [TBD].

17.2.3 HR-MS Forwarding to network

17.2.3.1 General Description

In HR-MS Forwarding to Network, an HR-MS forwards user data and control signaling between an HR-MS and an HR infrastructure station. The user data and control signaling do not go through higher layer at the forwarding HR-MS. The origination and termination of the user data and control signaling are at the forwarded HR-MS and the HR infrastructure station respectively and vice versa.

HR-MS Forwarding to Network is applicable when 1) the forwarded HR-MS and the forwarding HR-MS are in coverage of and associated to an infrastructure station; 2) the forwarding HR-MS is in coverage of and associated to an HR infrastructure station, while the forwarded HR-MS is out of coverage of any HR infrastructure stations.

Resource for HR-MS Forwarding to Network can be allocated by the HR infrastructure station with which the forwarding HR-MS is associated.

17.3.2 Direct communication between HR-MSs

17.3.2.1 General Description

Same as 17.2.2.1

17.3.3 HR-MS Forwarding to network

17.3.3.1 General Description

Same as 17.2.3.1.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]