| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast robustness improvement for HR-Network |  |
| Date Submitted | 2011-07-05 |  |
| Source(s) | Eldad Zeira Anh Tuan Hoang, Haiguang Wang, Yeow Wai Leong Ming-Tuo Zhou The authors are affiliated with InterDigital, I2R, and NICT | E-mail: eldad.zeira@interdigital.com athoang@i2r.a-star.edu.sg mingtuo@nict.com.sg |
| Re: | Call for comments to AWD in 802.16n-11/0009r1 |  |
| Abstract | The text proposal in this contribution introduces feedback as a means to enhance multicast robustness |  |
| Purpose | To be discussed and accepted by 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multicast robustness improvement for HR-Network

# Eldad Zeira

# Anh Tuan Hoang, Haiguang Wang, Wai Leong Yeow

# Ming-Tuo Zhou

# Introduction

HR-Network devices will be deployed to support a large variety of mission critical applications in areas such as Smart Grid and PPDR. In most cases, applications running on the devices will receive instructions sent by the network user. Failure to receive such instructions on a timely basis could, in some cases, result in loss of functionality of the application at the time it is needed the most. In some cases the consequences could be catastrophic. For these cases it is important that the network user has a high degree of certainty that the message has been received.

In many cases these messages could be sent to a large number of devices at the same time and involve operations critical to safety of equipment or personnel. For example, in Smart Grid application every cell is expected to support thousands of devices. An electrical utility company could signal many residential devices to reduce consumption at the same time (e.g. by shutting off non-essential AC units) as a quick way to reduce power consumption when needed. Other applications could include SW updates etc. In some cases, it is important that the sender of the commands knows which of the intended recipients of the command has actually received it. In others, it is sufficient that the sender knows, with a high degree of certainty, how many have.

An efficient way of transmitting the same message to a large number of devices is through broadcast (transmitting to all devices) or multicast (transmitting to a group). The 802.16n SRD [1] recognizes this need in the requirement: (the underline is not part of the SRD)

### 6.2.2	High reliability control and reporting

The HR-network shall provide MAC enhancements to support high reliability latency intolerant control and reporting.

Multicast operation can support timely transmission to a large number of devices. It can also provide any desired robustness through repetitions. It does not, in itself, provide the network user with the information of who has received the message successfully or even the multicast success rate.

Therefore we propose that HR-Network supports feedback for multicast. Please see text proposal for [2] below.

# References

- IEEE 802.16n-10/0048r1, “802.16n System Requirements Document including SARM annex,” March 2011.

- IEEE 802.16n-11/0009r1 802.16n AWD.

# Proposed Text for the 802.16n Amendment Working Document (AWD) [2]

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

# Insert at the bottom of 17.3.9 (P.37, L. 1261)

To ensure robust multicast and provide the network operator with specific or statistical information of its reception a feedback operation is defined between an HR-MS that is an addressee of a multicast transmission and its serving HR-BS or HR-RS.

The conditions for providing feedback are defined by the network per each multicast channel and include positive feedback only (logical ACK), negative feedback only (logical NAK) or both (logical ACK/NAK). It is expected that all intended recipients of a multicast channel obey the same rules but those can be changed by the network. UL resources for the feedback are also provided by the HR-BS. Feedback parameters may be unicast or multicast.

Feedback operation is supported by multicast addressees in connected as well as in idle states.

The procedure for providing the feedback is TBD.

# Modify fig. XXX (P. 38, L. 1304)

Modify figure as follows:

Discovery

&

Registration

Connection Establishment

/

Configuration

AAI

_

REG

-

REQ

/

RSP

HR

-

MS

Receiving multicast control channel

Receiving multicast data

AAI

_

DSA

-

REQ

/

RSP

HR

-

BS

Multicast assignment MAP

Multicast burst

Providing feedback

Figure xxx – Procedure of multicast communication establishment

# Modify 17.3.9.1.2 (P. 39 L. 1311-)

When the HR-MS transits to a new Multicast Zone while in Active Mode or Sleep Mode, the HR-MS shall send AAI-RNG-REQ message with Ranging Purpose Indication = 0b1110 at the target HR-BS and the ABSHR-BS shall include Multicast Group ID and FID Update and feedback parameters in AAI-RNG-RSP parameters to provide updated service flow management encodings for any affected E-MBS flow as part of the handover procedure.

# Modify 17.3.9.1.3 (P. 39 L. 1327-)

In response to the request for multicast location update, the HR-BS shall transmit AAI-RNG-RSP message which may include the Multicast Group Zone identifier, Multicast Group ID, and FID and feedback parameters to provide update service flow management encodings for any affected multicast flow(s).