| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Initialization of HR-MS Relay Mode |  |
| Date Submitted | 2011-07-20 |  |
| Source(s) | Liru Lu (Alina), Ming-Tuo Zhou, Xin Zhang, Vinh Dien Hoang, Masayuki Oodo, Hiroshi Harada NICT Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: liru@nict.com.sg; mingtuo@nict.com.sg; zhangxin@nict.com.sg; hvdien@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp ekkim@etri.re.kr |
| Re: | Call for Comments for 802.16n AWD |  |
| Abstract | In this contribution, we propose text of multi-mode operation for 802.16n. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Initialization of HR-MS Relay Mode

Liru Lu (Alina), Ming-Tuo Zhou, Xin Zhang

Vinh Dien Hoang, Masayuki Oodo, Hiroshi Harada

NICT

Eunkyung Kim, Sungcheol Chang, Sungkyung Kim,

Won-Ik Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

# 1. Introduction

This document is to provide the contribution to AWD Document IEEE 802.16n-11/0009r1 for the specification of relay function initialization for HR-MS in OFDMA interface.

# 2. Text Proposal for IEEE 802.16n AWD

Xxx

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to AWD

[---------------------------------Insert the following texts to section 17.2.1.2---------------------------------------]

# 17.2.1.2 Initialization of Relay Function for HR-MS

17.2.1.2.1 Relay link establishment

To support relay function for HR-MS, HR-MS capable of relay function may establish relay link with HR-BS.

An HR-MS acting as HR-RS is operated in either TTR mode or STR mode and its relay mode is determined by HR-BS.

To request subordinate HR-MS to change its role as HR-RS, HR-BS transmits MMRS-REQ message described in 6.3.2.3.x1 including relay mode (i.e., either TTR or STR mode).

In response to MMRS-REQ, the HR-MS transmits MMRS-RSP message described in 6.3.2.3.x2.

During establishing relay link, HR-BS transmits RS_Config-CMD message described in 6.3.2.3.63 to configure the operation parameters of HR-RS.

17.2.1.2.2 Relay link configuration

While HR-MS is acting as relay mode, the superordinate HR-BS may send an RCD message to configure the Relay operation parameters as specified in 6.3.9.18.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]