| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Recovery Process for Standalone Network |  |
| Date Submitted | 2011-07-20 |  |
| Source(s) | Liru Lu (Alina), Ming-Tuo Zhou, Xin Zhang, Vinh Dien Hoang,Masayuki Oodo, Hiroshi Harada NICT | E-mail: liru@nict.com.sg; mingtuo@nict.com.sg; zhangxin@nict.com.sg; hvdien@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp |
| Re: | Call for Comments for 802.16n AWD |  |
| Abstract | In this contribution, we propose text of standalone network operation for 802.16n. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Recovery Process for Standalone Network

Liru Lu (Alina), Ming-Tuo Zhou, Xin Zhang

Vinh Dien Hoang, Masayuki Oodo, Hiroshi Harada

NICT

# 1. Introduction

This document is to present a proposed recovery process for standalone network. It is specified in the System Requirement Document that when the HR-BS loses connectivity to the backbone network and the neighboring HR-BSs, the network stations under the coverage of the HR-BS shall form a standalone network. When the connection of HR-BS to backbone network and/or the neighboring HR-BSs recovers, a recovery process is required to integrate the standalone network into backbone connected network.

2. Recovery Process for Standalone Network

When a HR network is standalone, the network only supports local connectivity for its subordinate stations which are covered under the standalone network HR-BS.

When the HR-BS of standalone network recovers its connectivity to the backbone network, the standalone network shall be incorporated to the backbone connected network. The standalone network HR-BS shall obtain the neighbor HR-BSs information through upper layer message.

The subordinate stations shall be informed on the availability of backbone connection and remain the association with the serving HR-BS of the original standalone network. The handover service for the subordinate stations of standalone network will be reinstated.

# 2. Text Proposal for IEEE 802.16n AWD

Xxx

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to AWD

[----------------------------Insert the following text to Page 20 Section 17.2.4.3-------------------------------------]

17.2.4.3 Recovery Process of Standalone Network

When a standalone network HR-BS recovers the backbone connection, the standalone network shall be incorporated to the backbone connected network. The standalone network HR-BS shall obtain the information of neighbor HR-BS through upper layer message for inter-BS communications. The upper layer message is not defined here as it is out of the scope of this standard. The subordinate stations shall remain the association with the serving HR-BS of the original standalone network or re-associate to the serving HR-BS.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]