| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Determining alternative path in Path Discovery and Management |  |
| Date Submitted | 2011-07-20 |  |
| Source(s) | Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada National Institute of Information and Communications Technology (NICT) | E-mail: hvdien@nict.com.sg; mingtuo@nict.com.sg; zhangxin@nict.com.sg; liru@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp |
| Re: | Call for contributions for the 802.16n Amendment Working Draft |  |
| Abstract | This provides a proposal of establishment and maintenance of alternative path in 802.16n networks (Original document in Session #72: C80216n-11_0026.doc) |  |
| Purpose | To be discussed and adapted by TGn for draft 802.16n AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# A Proposal for determining alternative path in path management of 802.16n-GRIDMAN networks

Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada

NICT

# Introduction

# Session 17.2.7.2 of IEEE 802.16n AWD 802.16n-11/0009r1 specifies the guidelines for alternative path to help HR_MS recovering from SPOF. However, it does not provide a detail specification on when to prepare the alternative path and how to do so.

# In this proposal, we propose a detail framework for establishment and maintenance of alternative paths to support fast recovery in the event of SPOF.

# Target scenario

Our target scenario in presented in Figure 1. In this scenario, HR-MS4 currently uses HR-RS/MS1 as the relay station to HR-BS. To prepare for the SPOF, HR-MS4 needs to construct an alternative path to HR-BS. There’re two candidates: HR-RS/MS2, HR-RS/MS3.

Figure 1: 802.16n Network

## 2.1 Alternative path determination by HR-BS

To calculate alternative path for HR-MS4, HR-BS need to know which HR-RS/MSs are the neighbor of HR-MS4. HR-BS can request HR-MS4 to do the neighbor discovery and report the result to HR-BS.

Once HR-BS gets the neighbor discovery report and know which neighbor HR-RS/MS is more suitable (higher signal strength, etc). HR-BS will decide (algorithm is out of scope of this contribution) which HR-RS/MS is the target alternative path.

Figure 2: Alternative path done by HR-BS

## 2.2 Alternative path determination by HR-MS

Alternative path could also be determined by the HR-MS4 itself:

- Open the possibility of more flexible, vender specific algorithm to calculate the alternative path

- Less depended on HR-BS ----> Increase the reliability

To calculate the alternative path by itself, HR-MS need to collect the following information:

- Which HR-RS/MS agree to serve HR-MR in the target alternative path

- Comparable information among the candidate HR-RS/MS, e.g: Distance to HR-BS, the nearer to the HR-BS, the higher priority to be candidate for the alternative path.

To do so, HR-MS first have to do the neighbor discovery with the option to ask neighbor nodes to become candidate for alternative path. HR-BS will then inform HR-MS about which HR-MSs are candidate in the alternative path. From these information HR-MS will decide on its alternative path.

# Text Proposal in IEEE 802.16n AWD

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Insert the following text into Section 17.2.7 Path Discovery and Management of 802.16n AWD]

17.2.7.2.1 Preparation for SPOF

…

To support fast network reentry to the neighbor HR-MSs, HR-MS shall transmit its neighbour information to HR-BS.

….

17.2.7.3 Preparation for Alternative path to support fast network reentry to the neighbor HR-MS

HR-MSs capable of forwarding to the network and/or multimode operation shall share the MAC context information with the HR-MS performing local forwarding to the network.

To support fast network reentry to the neighbor HR-MSs, either HR-BS or HR-MS may prepare the alternative path.

To prepare the alternative path by an HR-BS, the HR-BS shall perform operation as follows:

- neighbor discovery as described in 17.2.7.1.1

- collecting HR-MS’ neighbor information as described in 17.2.7.1.1

- determines the alternative path for HR-MS

- informs HR-MS about its alternative path information

To prepare the alternative path by an HR-MS, following operation shall be performed by HR-MS and HR-BS:

- An HR-MS transmit AP-NBR-REQ to the HR-BS to initiate the neighbor discovery process

- HR-BS received AP-NBR-REQ, performs the neighbor discovery as described in 17.2.7.1.1

- HR-BS collects the neighbor information of requesting HR-MS as described in 17.2.7.1.1

- HR-BS transmits AP-NBR-REP message to HR-MS which includes the neighbor information of requesting HR-MS

- HR-MS determines alternative path by itself based on its received neighbor information, but how to determine is out of scope of this specification

[Insert the following text into Section 17.3.7 Path Discovery and Management of 802.16n AWD]

17.3.7.2.1 Preparation for SPOF

…

To support fast network reentry to the neighbor HR-MSs, HR-MS shall transmit its neighbour information to HR-BS.

….

17.3.7.3 Preparation for Alternative path to support fast network reentry to the neighbor HR-MS

HR-MSs capable of forwarding to the network and/or multimode operation shall share the MAC context information with the HR-MS performing local forwarding to the network.

To support fast network reentry to the neighbor HR-MSs, either HR-BS or HR-MS may prepare the alternative path.

To prepare the alternative path by an HR-BS, the HR-BS shall perform operation as follows:

- neighbor discovery as described in 17.3.7.1.1

- collecting HR-MS’ neighbor information as described in 17.3.7.1.1

- determines the alternative path for HR-MS

- informs HR-MS about its alternative path information

To prepare the alternative path by an HR-MS, following operation shall be performed by HR-MS and HR-BS:

- An HR-MS transmit AP-NBR-REQ to the HR-BS to initiate the neighbor discovery process

- HR-BS received AP-NBR-REQ, performs the neighbor discovery as described in 17.3.7.1.1

- HR-BS collects the neighbor information of requesting HR-MS as described in 17.3.7.1.1

- HR-BS transmits AP-NBR-REP message to HR-MS which includes the neighbor information of requesting HR-MS

- HR-MS determines alternative path by itself based on its received neighbor information, but how to determine is out of scope of this specification

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.

- IEEE 802.16n-11/0009r1, “802.16n Amendment Working Draft,” May 2011.