| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal of self-coexistence for IEEE 802.16n AWD |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada NICT | E-mail: mingtuo@nict.com.sg |
| Re: | Call for comments on 802.16n (GRIDMAN) AWD |  |
| Abstract | Text proposal of self-coexistence operation for the IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Text Proposal of Self-coexistence for IEEE 802.16n AWD

Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang,

Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

NICT

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0009r1.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

17.2.11 Self-Coexistence

HR network shall support self-coexistence mechanism to mitigate co-channel interference among HR-stations within the same geographical area.

17.2.11.1 Self-coexistence cycle

A self-coexistence cycle (SCC) consists of TBD frames as shown in Fig. xx1. A frame is identified by frame identification number (FIN), which is in range between 0 to TBD-1. Location of a frame identification number is TBD.

Fig. xx1 Self-coexistence cycle for WirelessMAN OFDMA air interface.

17.2.11.2 Self-coexistence zone Frame structure

A self-coexistence zone is a space can be allocated in a frame for transmission preamble and self-coexistence beacons for self-coexistence of multiple HR cells overlapped in coverage and have to operate on the same frequency channel.

In WirelessMAN HR OFDMA networks, As shown in Fig. xx2, a self-coexistence zone occupies the last 3 symbols of a frame. The first symbol is used as guard time. In the second symbol, preamble shall be transmitted, and in the last symbol self-coexistence beacons are beacon protocol (SCBP) MAC PDU shall be transmitted.

Figure xx2: TDD frame structure with self-coexistence zone.

Figure xx Illustration of self-coexistence zone of WirelessMAN HR OFDMA air interface.

17.2.11.13 Operation modes

HR network can operate in two modes: normal mode and self-coexistence mode. In normal mode, a HR network occupies one frequency channel and operates on all frames of a self-coexistence cycle. In self-coexistence mode, multiple HR cells share the same frequency channel and operate on different frames. A HR cell operates in normal mode by default and transits to self-coexistence mode when the HR cell receives self-coexistence beacon from an adjacent HR cell on its operating channel.

17.2.11.4 Self-coexistence Beacon Protocol (SCBP)

TBD

17.2.11.5 Mechanism for self-coexistence of multiple HR cells

TBD

17.3.11 Self-Coexistence

HR network shall support self-coexistence mechanism to mitigate co-channel interference among HR-stations within the same geographical area.

17.3.11.1 Operation Modes Self-coexistence cycle

TBD

17.3.11.2 Self-coexistence Zone Frame structure

A self-coexistence zone is a space can be allocated in a frame for transmission preamble and self-coexistence beacons for self-coexistence of multiple HR networks overlapped in coverage and have to operate one the same frequency channel.

The structure of self-coexistence zone in WirelessMAN HR Advanced networks is TBD.

17.3.11.13 Operation modes

See 17.2.11.13

17.3.11.4 Self-coexistence Beacon Protocol (SCBP)

TBD

17.3.11.5 Mechanism for self-coexistence of multiple HR cells

TBD

[-------------------------------------------------End of Text Proposal----------------------------------------------------]