| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC control messages for multicast operation |  |
| Date Submitted | 2011-07-19 |  |
| Source(s) | Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-11/0008,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | MAC control messages for multicast operation in IEEE 802.16n |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MAC control messages for multicast operation

Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

# Introductions

IEEE 802.16n AWD [5] describes multicast operation based on the 802.16n SRD [1].

To support multicast operation, some MAC control messages need to modify as follows:

- AAI-RNG-RSP

  - To update multicast service flow during ranging procedure as part of HR or location update based on the HR-MS’ status

- AAI-SCD

  - To broadcast multicast group zone ID (it also indicates whether current BS support multicast or not)

- DSA-REQ/RSP

  - To add multicast service flow including Multicast Group Zone ID, Multicast Group ID, and FID

- DSC-REQ/RSP

  - To update multicast service flow including Multicast Group Zone ID, Multicast Group ID, and FID

- AAI-NBR-ADV

  - To broadcast mapping list of multicast service flow between serving BS and neighbor BS

# References

- IEEE 802.16n-10/0048r1, “802.16n System Requirements Document including SARM annex,” March 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16m-09/0034r3, “IEEE 802.16m System Description Document (SDD),” June 2010.

- IEEE 802.16n-11/0009r1, “802.16n Amendment Working Draft,” May 2011.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy: Add the following text into 16.2.3 in 802.16n AWD.]

16.2.3.2 AAI-RNG-RSP

[Change Table 685 in section 16.2.3.2 as indicated:]

Table 685—AAI-RNG-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Abort | 1 | Set to 1 when an ABS rejects the AMS. | Present when an ABS rejects an AMS. |
| If (Ranging Abort == 1) { |  |  |  |
| Ranging Abort Timer | 16 | Timer defined by an ABS to prohibit the AMS from attempting network entry at this ABS, for a specific time duration. Value: 0 (Do not try ranging again at the ABS.) Value: 1–65534, in units of seconds Value: 65535 (When the received CSGID(s) from the AMS does not match any of the CSGID(s) of the Femto ABS. This value indicates the Ranging Abort Timer is not to be used, and the AMS can range any time.) |  |
| }else{ |  |  |  |
| … | … | … | … |
| Location Update Response | 4 | 0x0= Success of Location Update 0x1= Failure of Location Update 0x2 = Reserved 0x3 = Success of location update and DL traffic pending 0x4 = Allow AMS’s DCR mode initiation request or DCR mode extension request 0x5 = Reject AMS’s DCR mode initiation request or DCR mode extension request 0x6~0xF: Reserved | Shall be included when this message is sent in response to an AAI-RNG-REQ message used to perform location update or DCR mode initiation from Idle Mode or DCR mode extension. |
| If (Location Update Response == 0x0) { |  |  |  |
| … | … | … | … |
| SMS | Variable | Short message contents up to the size of 140bytes | May be included when SMS contents is sent in idle mode. |
| New Multicast Group Zone ID | 12 | Indicates a Multicast Group Zone ID to update in target HR-BS. | Shall be included in HR-Network in response to the AAI-RNG-REQ message where ranging purpose indication is set to 0b1110. |
| For(j=1;j<=M;j++){ |  | Number of Multicast Group ID and FID (M) to update in the target HR-BS[1..16] | Present if it needs to update in HR-Network. |
| Current Multicast Group ID | 12 |  |  |
| Current FID | 4 |  |  |
| New Multicast Group ID | 12 |  |  |
| New FID | 4 |  |  |
| } |  |  |  |
| }//end of If (Location Update Response == 0x0 ) |  |  |  |
| Reentry Process Optimization | 5 | Reentry process optimization bitmap indicates which MAC control message transactions may be omitted during an attempted reentry (i.e., reentry during HO (including zone switching), and reentry from idle mode) A value of 1 in the bitmap indicates that the corresponding MAC control message transaction may be omitted, while a 0 indicates that the corresponding MAC control message transaction shall be completed. The AMS shall only commence Connected State with the T-ABS after completing all the required MAC control message transactions. Bit 0: Omit AAI-SBC-REQ and AAI-SBC-RSP MAC control messages during reentry processing Bit 1: Omit PKM Authentication phase Bit 2: Omit AAI-REG-REQ and AAI-REG-RSP message during reentry processing. Bit 3: Omit higher layer protocol triggering for IP address refresh during reentry processing Bit 4: For the case of reentry during HO including zone switching, a 1 indicates to the AMS that the T-ABS has received the full service and operational states for static and dynamic context (including ARQ window parameters and state machines). For the case of reentry from Idle mode, a 1 indicates to the AMS that the T-ABS has received the static context of the AMS. The static context includes SFIDs and related description (QoS descriptors and CS classifier information) for all service flows that the AMS has currently established as well as any SAs with their related keying information. |  |
| … | … | … | … |
| If (it is under network reentry for HO){ |  |  |  |
| New Multicast Group Zone ID | 12 | Indicates a Multicast Group Zone ID to update in target HR-BS. | Shall be included in HR-Network in response to the AAI-RNG-REQ message where ranging purpose indication is set to 0b1110. |
| For (i = 0; i< M; i++) { |  | Number of Multicast Group ID and FID (M) to update in the T-ABS[1..16]. Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to be updated. Based on the value of Num of Multicast Group ID and FID to be updated. | Present if it needs to be updated. |
| Current Multicast Group ID | 12 |  |  |
| Current FID | 4 |  |  |
| New Multicast Group ID | 12 |  |  |
| New FID | 4 |  |  |
| } |  |  |  |
| }//end of If (it is under network reentry for HO) |  |  |  |
| For (i = 0; i<N_SFIDs; i++) { |  | N_SFIDs is Number of SFIDs supported in MZone when an AMS performs Zone Switching from LZone to MZone. Its maximal number is 24. | Present if CID to FID mapping is done through the AAI-RNG-RSP message during Zone Switching operation. If this field is not present, all FIDs for the transport connection should be reestablished through the AAI-DSA exchanges after completion of network reentry in MZone. |
| SFID | 32 | FID in MZone should be assigned as defined in 16.2.6.4.1.3.1 per each DL/UL connections |  |
| } |  |  |  |
| } //End of else (Ranging Abort==1) |  |  |  |

[Change section 16.2.3.13 AAI-NBR-ADV as indicated:]

16.2.3.13 AAI-NBR-ADV

AAI-NBR-ADV message may sort neighbor ABSs (RSs) according to their deployment types, which is categorized by the following parameters:

- Cell type (macro, micro, macro hot-zone, Femto, relay)

- Physical carrier index referring AAI-Global-CFG message that provides carrier frequency, BW, TDD/FDD and related definitions (expected to be the same given carrier frequency)

- MAC version

ABS determines and indicates the system configuration information included for each deployment type and their corresponding broadcast frequency.

To allow AAI-NBR-ADV fragmentation while providing flexibility for AMS HO operation without requiring acquisition of the whole AAI-NBR-ADV message, ABS always provides the total number of cell types, total number of segments for each type, and the total number of neighbor ABS per segment. Each AAI-NBR-ADV fragment is indicated by AAI-NBR-ADV Segment Index. Once an AAI-NBR-ADV segment is sent for a given cell type, the ABS should not start sending ABS for different cell type until all AAI-NBR-ADV segments for such given cell type are sent. ABSs with identical type are listed in the AAI-NBR-ADV message in descending order of their cell coverage.

Each AAI-NBR-ADV message carries the following:

- AAI-NBR-ADV change count

- Number of total cell types

- Segment information for this AAI-NBR-ADV message

- System information of ABSs of a specific cell type

- Starting ABS Index: Starting ABS Index of the AAI-NBR-ADV segment is the value of one incremented from the last ABS index of the previous AAI-NBR-ADV segment. Starting ABS Index is the index of the first NBR ABS that is sent in a AAI-NBR-ADV segment. The index of the second NBR ABS equals to Starting ABS Index +1, and so on so that all ABSs in this AAI-NBR-ADV segment has consecutive index in ascending order. If this is the first AAI-NBR-ADV segment or the AAI-NBR-ADV message is not segmented, the Starting ABS Index will be 0.

For each cell type, shared configuration parameters are carried as follows:

- Prefix of BSID

- SA-Preamble Index range

- Physical carrier range

The Prefix of BSID, SA-Preamble Index range, and Physical carrier range are optional fields, which can be used when the S-ABS chooses not to broadcast configuration information for each individual ABS within the cell type.

Within each cell type, if S-ABS chooses to broadcast configuration information for each individual ABS instead of specifying SA-Preamble Index range and Physical carrier range, a list of ABSs are provided and the following parameters are carried for each ABS:

- 48-bit BS-ID

- ABS SA-Preamble Index

- Indication whether full system information or partial information is carried for this ABS, which includes the following:

  - SFH information

  - Physical carrier index (6 bits, refer to the “physical carrier index” defined in AAI-Global-CFG)

  - MAC protocol versions (8 bits)

  - Paging carrier indication (1 bit, refer to specify if a carrier is a paging carrier or not)

- Multicast service flow mapping list (for HR-Network)

  - Neighbor Multicast Group Zone ID

  - Mapping of Multicast Group ID + FID and neighbor Multicast Group ID + FID

where for ABS of macrocell type, all the necessary system information shall be included, and the format may only carry delta information fields with respect to the reference ABS (e.g., the S-ABS or the preceding neighbor BS/ABS of this cell type); and for Wireless-MAN-OFDMA reference system, only 48-bit BS-ID and Preamble index are included in AAI-NBR-ADV.

…….

[Change Table 696 in section 16.2.3.13 as indicated:]

Table 696—AAI-NBR-ADV message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ………… | …… | ……… | …… |
| For (i=0; i<N-NBR-ABSs; i++) { |  | N-NBR-ABSs is the number of neighbor ABSs included in this message, and has the range of [1..64]. |  |
| BSID | 48 | Neighbor ABS ID |  |
| MAC protocol version | 8 | MAC protocol version of the BS Consistent with IEEE Std 802.16-2009 definition, with new MAC protocol version 10 defined for AAI. |  |
| CP time | 2 | CP time of the BS 0b00: 1/8 0b01: 1/16 0b10: 1/4 |  |
| Neighbor Multicast Group Zone ID | 12 | Indicates a Multicast Group Zone ID provided by neighbor BS. | Present in HR-Network |
| For(j=1;j<=M;j++){ |  | Number of Multicast Group ID and FID (M) mapping between serving BS and neighbor BS[1..16] | Present if needed |
| Multicast Group ID | 12 |  |  |
| FID | 4 |  |  |
| Neighbor Multicast Group ID | 12 |  |  |
| Neighbor FID | 4 |  |  |
| } |  |  |  |
| For(j=0; j<N-Carrier-Info; j++) { |  | N-Carrier-Info is the number of carrier information listed here for the ABSi |  |
| ………… | …… | ………… | …… |

[Add following rows in the end of Table 714 in section 16.2.3.31 as indicated:]

| Multicast Group Zone ID | 12 | Indicates a Multicast Group Zone ID provided by this BS. Shall not be set to “0.” | In HR-Network |
| --- | --- | --- | --- |

[Change last paragraph in section 16.2.3.47 AAI-DSA-REQ as indicated:]

When an ABS commences multicast service, the following parameters shall be included in the AAI-DSA-REQ message.

- Multicast Group Zone ID: Indicates multicast group zone IDs for the connection that is associated with the service flow in AAI-DSA-REQ in HR-Network.

- Multicast Group ID: Indicates multicast group for the connection that is associated with the service flow in AAI-DSA-REQ.

[Change Table 740 in section 16.2.3.47 as indicated:]

Table 740—AAI-DSA-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ………… | …… | ……… | …… |
| For(i=0; i<N-FIDs-Coupled- Noncommon; i++) { |  | N-FIDs-Coupled-Noncommon is the number of non-common coupled service flow IDs The maximum value of N-FIDs-Coupled-Noncommon is 32. |  |
| FID | 4 |  | Shall be present if NFIDs-Coupled-Noncommon is not zero |
| Non-common for Coupled Group | variable | Non-common service flow encodings that are specific to individual service flows specified in Coupled FID Parameter List Service flow/convergence sublayer parameters in Table 788, except FID, SFID, E-MBS service related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, may be encapsulated in this field. | Shall be present if NFIDs-Coupled-Noncommon is not zero |
| } |  |  |  |
| } |  |  |  |
| Multicast Group Zone ID | 12 | Indicates a multicast group zone to add where the connection for associated service flow is valid. | Present if needed in HR-Network |
| For (i=0; i<Num of Multicast Group ID and FID (M); i++) { |  | Num of Multicast Group ID and FID (M) is the number of Multicast Group IDs to add [1..16] | Present when ABS initiates AAI-DSA-REQ |
| Multicast Group ID | 12 | ID of a group to which the flow is added | Present only if Num of Multicast Group ID and FID (M)> 0 |
| FID | 4 | Multicast specific FID that is associated with Multicast Group ID | Present only if Num of Multicast Group ID and FID (M)> 0 |
| } |  |  |  |
| If (sleep cycle setting is included) { |  |  |  |
| Operation | 2 | This indicates operation request type 0b00~0b01: Reserved 0b10: Change sleep cycle setting 0b11: Switch sleep cycle setting |  |
| ………… | …… | ……… | …… |

[Change last paragraph in section 16.2.3.48 AAI-DSA-RSP as indicated:]

When an AMS commences multicast service, the ABS shall include the following parameters in the AAI-DSA-RSP message:

- Multicast Group Zone ID: Indicates multicast group zone IDs for the connection that is associated with the service flow in AAI-DSA-RSP in HR-Network.

- Multicast Group ID: Indicates multicast group for the connection that is associated with the service flow in AAI-DSA-RSP.

[Change Table 741 in section 16.2.3.47 as indicated:]

Table 741—AAI-DSA-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ………… | …… | ……… | …… |
| Carrier Switching Mode | 1 | 0b0: carrier switching method based on Unicast Available Interval in the AAI-DSA message 0b1: carrier switching method using AAI-E-MBS-REP/RSP message | Present if ABS indicates carrier switching when receiving AMS-initiated DSA |
| If(Carrier Switching Mode == 0b0) { |  |  |  |
| Unicast Available Interval Bitmap | variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2...bN-1 If bi==0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 4 superframes: N = 4 bits NMSI = 8 superframes: N = 8 bits NMSI = 16 superframes: N = 16 bits NMSI = 32 superframes: N = 32 bits Depending on the NMSI, the number of bits per subframe changes, 4 frames per bit |  |
| } |  |  |  |
| Multicast Group Zone ID | 12 | Indicates a multicast group zone to add where the connection for associated service flow is valid. | Present if needed in HR-Network |
| For (i=0; i<Num of Multicast Group ID and FID (M); i++) { |  | Num of Multicast Group ID and FID (M) is the number of Multicast Group IDs to add [1..16] |  |
| Multicast Group ID | 12 | ID of a group to which the flow is added | Present only if Num of Multicast Group ID and FID (M)> 0 |
| FID | 4 | Multicast specific FID that is associated with Multicast Group ID | Present only if Num of Multicast Group ID and FID (M)> 0 |
| } |  |  |  |
| If (sleep cycle setting is included) { |  |  | May be present when sleep cycle setting needs to be changed or switched |
| Response_Code | 2 | This indicates response type of AAI-SLP-RSP message. 0b00: Request by ABS in Unsolicited manner 0b01: Approval of AAI-SLP-REQ 0b10: Rejection of AAI-SLP-REQ 0b11: Reserved | This parameter shall be included only when ABS transmit this control message. |
| Operation | 2 | This indicates operation request type 0b00~0b01: Reserved 0b10: Change sleep cycle setting 0b11: Switch sleep cycle setting |  |
| ………… | …… | ……… | …… |

[Change Table 743 in section 16.2.3.47 as indicated:]

Table 743—AAI-DSC-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ………… | …… | ……… | …… |
| For(i=0; i<N-FIDs-Coupled-Noncommon; i++) { |  | N-FIDs-Coupled-Noncommon is the number of non-common coupled service flow IDs. The maximum value of N-FIDs-Coupled-Noncommon is 32. |  |
| FID | 4 | Flow identifier | Present when N-FIDs-Coupled-Noncommon >0 |
| Non-common for Coupled Group | variable | Non-common service flow encodings that are specific to individual service flows specified in Coupled FID Parameter List. Service flow/convergence sublayer parameters in Table 788, except FID, SFID, E-MBS service-related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, may be encapsulated in this field. | Present when N-FIDs-Coupled-Noncommon >0 |
| } |  |  |  |
| } |  |  |  |
| New Multicast Group Zone ID | 12 | Indicates a multicast group zone to overwrite where the connection for associated service flow is valid. | Present when ABS initiates AAI-DSC-REQ in HR-Network |
| For (i=0; i<Num of Multicast Group ID and FID (MC); i++) { |  | Num of Multicast Group ID and FID (MC) is the number of Multicast Group IDs to add [1..16] | Present when ABS initiates AAI-DSA-REQ |
| Multicast Group ID | 12 | ID of a group to which the flow is added | Present only if Num of Multicast Group ID and FID (M)> 0 |
| FID | 4 | Multicast specific FID that is associated with Multicast Group ID | Present only if Num of Multicast Group ID > 0 |
| } |  |  |  |
| For (i = 0; i< MU; i++) { |  | Number of Multicast Group ID and FID (MU) to update [1..16]. Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to update. Based on the value of Num of Multicast Group ID and FID to update. | Present if it needs to update in HR-network. |
| Current Multicast Group ID | 12 |  |  |
| Current FID | 4 |  |  |
| New Multicast Group ID | 12 |  |  |
| New FID | 4 |  |  |
| } |  |  |  |
| For (i=0; i<Num of Multicast Group ID and FID (MA); i++) { |  | Num of Multicast Group ID and FID (MA) is the number of Multicast Group IDs to add [1..16] | Present when ABS initiates AAI-DSC-REQ Present only if Multicast Group ID to be added exists |
| Multicast Group ID to be added | 12 | Multicast Group ID to be added | Present only if Num of Multicast Group ID and FID (M)> 0 |
| FID | 4 | Multicast specific FID which is associated with newly added Multicast Group ID | Present only if Num of Multicast Group ID and FID (M) > 0 |
| } |  |  |  |
| For (i=0; i<Num of Multicast Group ID and FID (MD); i++) { |  | Num of Multicast Group ID and FID (MD) is the number of Multicast Group IDs to delete [1..16] | Present when ABS initiates AAI-DSC-REQ Present only if Multicast Group ID to be deleted exists |
| Multicast Group ID to be deleted | 12 | Multicast Group ID to be deleted | Present only if Num of Multicast Group ID and FID (M) > 0 |
| FID |  | Multicast specific FID which is associated with newly deleted Multicast Group ID | Present only if Num of Multicast Group ID and FID (M)> 0 |
| } |  |  |  |
| If (sleep cycle setting is included) { |  |  | May be present when sleep cycle setting needs to be changed or switched |
| Operation | 2 | This indicates operation request type 0b00~0b01: Reserved 0b10: Change sleep cycle setting 0b11: Switch sleep cycle setting |  |
| ………… | …… | ……… | …… |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]