| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Relay function for HR-BS |  |
| Date Submitted | 2011-07-11 |  |
| Source(s) | Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-11/0008,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Multimode for HR-BS acting as RS in IEEE 802.16n |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Relay function for HR-BS

Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

# Introductions

IEEE 802.16n AWD [5] describes multimode operation based on the 802.16n SRD [1].

This document provides multimode operation in detail for HR-BS to establish/maintain/release relay link. In addition, MM-ADV message is proposed to support multimode operation as a broadcast message transmitted by infrastructure station including HR-MS acting as HR-BS or HR-RS.

To establish relay link in 802.16m[3], ARS shall perform following steps:

- Scan for DL channel and establish synchronization with the HR-BS having connection to backhaul

- Obtain DL/UL parameters (from SuperFrameHeader)

- Perform ranging

- Basic capability negotiation

- Authorization, authentication, and key exchange, if needed

- Registration with the HR-BS, if needed

- Configuration operational parameters

However, the HR-BS having no backhaul connection may stop service for subordinate HR-MSs to establish relay link. Thus, before initiating establishment relay link with other HR-BS having connection to backhaul, HR-BS may announce subordinate stations the reason why the service is unavailable using MM-ADV message and any HR-MS is not allowed to entry the HR-BS.

Some messages (i.e., AAI-MMRS-REQ/RSP) needs to be transmitted to initiate establishing relay link.

During establishing relay link, some information should be transmitted from HR-BS having connection to backhaul. Some configuration information shall be configured to HR-BS having no connection to backhaul.

Thus, step e) and g) shall be modified to establish relay link between HR-BSs.

[NOTE: detailed message format is submitted by separate contribution because the message handles the other cases (e.g., relay function for HR-MS).]

# In addition, during having relay link or releasing relay link, the procedure should be performed such as re-configuration and release relay link after backhaul link is available.

# Procedure to establish relay link

Figure 1 shows the procedure for HR-BS (having no connection to backhaul) to establish relay link with other HR-BS (having connection to backhaul). As shown the Figure 1, the procedure for HR-BS in detail follows the steps as follows:

- HR-BS having no connection to backhaul transmits RELAY-REQ(ARS-REQ) to request other HR-BS having connection to backhaul to establish relay link

- HR-BS having connection to backhaul transmit RELAY-REQ(ARS-RSP) to respond the request by HR-BS having no connection to backhaul

- Two HR-BSs establish the relay link between themselves

  - Configuration operation parameters are set using AAI-ARS-CONFIG-CMD

Figure 1 – Procedure to establish relay link between HR-BSs

# PHY/MAC parameter configuration during establishing/maintaining relay link

A HR-BS having no connection to backhaul shall receive some parameter to initiate/maintain multimode.

During establishing relay link, HR-BS having connection to backhaul shall transmit message including configuration parameter.

During maintain multimode service to re-configure parameter, HR-BS having connection to backhaul shall transmit message including configuration parameter. If the HR-BS or HR-MS having relay link with superordinate HR-BS needs to configure the parameter and to announce subordinate HR-MSs, those multimode station having relay link shall MM-ADV message with the changed parameter.

During release multimode service (return to original mode), multimode HR-MS needs announce using MM-ADV message subordinate HR-MSs to perform handover to any infrastructure station.

# Procedure to release relay link

If the HR-BS recovers from failure of backhaul, it may inform network or notify the current serving HR-BS of the HR-BS having recovered backhaul link through the backhaul network interface. The superordinate serving HR-BS may then initiate HR-MS handover back to the HR-BS in which the recovered HR-BS should be listed in the first priority. The HR-BS having recovered backhaul may store MAC context information of the serving MSs (basic capabilities, security capabilities, etc.). Such context information allows HR-MS to perform optimized network reentry when returning back to the HR-BS upon its recovery.

# MM-ADV message

Infrastructure stations and HR-MS acting as HR-BS or HR-RS may transmit AAI-MM-ADV message to support multimode operation in the case as follows:

- When the backhaul link is down or up from down

- During maintaining relay link due to unavailable backhaul link, PHY layer parameters need be reconfigured such as power and FA

- During release multimode service (return to original mode), multimode HR-MS needs announce subordinate HR-MSs to perform handover to any infrastructure station.

# References

- IEEE 802.16n-10/0048r1, “802.16n System Requirements Document including SARM annex,” March 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16m-09/0034r3, “IEEE 802.16m System Description Document (SDD),” June 2010.

- IEEE 802.16n-11/0009r1, “802.16n Amendment Working Draft,” May 2011.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy: Adopt the following change in Section 17.3.1 in the 802.16n AWD.]

17.3.1 Multi-mode operation

17.3.1.1 Relay function for HR-BS

An HR-BS (affected HR-BS) may operate as a relay station to communicate with another HR-BS (serving HR-BS) that has connection to backhaul.

An HR-BS acting as RS mode operates in either TTR mode or STR mode.

17.3.1.1.1 STR mode for HR-BS acting as HR-RS

To support STR mode, the affected HR-BS maintains base station functionality.

The procedures for RS mode changer establishment of relay link consist of following activities:

- establish a relay link with a serving HR-BS

- if necessary, inform some subordinate stations to perform handover

- if necessary, reconfigure the physical frame and commence operation in relay mode

17.3.1.1.2 TTR mode for HR-BS acting as HR-RS

To support TTR mode, the affected HR-BS can maintain connectivity with subordinate HR-RS. How to maintain is FFS.

The procedures for RS mode changer establishment of relay link consist of following activities:

- establish a relay link with a serving HR-BS

- if necessary, inform some subordinate stations to perform handover

- if necessary, reconfigure the physical frame and commence operation in relay mode

17.3.1.1.3 Relay link establishment

HR-BS (affected HR-BS) having no connection to backhaul sets Cell Bar bit to 1 in order to prevent HR-MS (re)entry. The HR-BS having no connection to backhaul transmits MM-ADV message with action type = 0b100 described in 16.2.3.xa including expected time of backhaul link available. Based on the expected time, HR-MS handovers to neighbor infrastructure station or staying in the HR-BS until restarting service with available backhaul link.

To establish relay link, the HR-BS having no connection to backhaul follows network entry and initialization for relay link described in 16.2.15 and 16.6.2.10. In addition, after the ranging phase, the HR-BS shall perform the relay link establishment procedure as follows:

- Scan for DL channel and establish synchronization with the HR-BS having connection to backhaul

- Obtain DL/UL parameters (from SuperFrameHeader)

- Perform ranging

- Basic capability negotiation, if needed

- Authorization, authentication, and key exchange, if needed

- Registration with the HR-BS, if needed

- Configuration operational parameters including initiating relay link using AAI-MMRS-REQ/RSP and AAI-ARS-CONFIG-CMD messages

To establish relay link with another HR-BS (serving HR-BS), HR-BS having no connection to backhaul transmits AAI-MMRS-REQ message described in 16.2.3.xb including relay mode (i.e., either TTR or STR mode). In response to AAI-MMRS-REQ, the serving HR-BS transmits AAI-MMRS-RSP message described in 16.2.3.xc to inform the whether the request is accepted or rejected. Upon receiving the AAI-MMRS-RSP message, the affected HR-BS starts establishing the relay link with serving HR-BS immediately or retransmits AAI-MMRS-REQ message at the action time expires. If the serving HR-BS rejects the request, the serving HR-BS informs the HR-BS having no connection to backhaul the rejection of the request. Upon receiving the AAI-MMRS-RSP message with rejection information, the HR-BS either tries to establish relay link with other HR-BS or follows standalone network operation described in 17.3.4.

To support handover as a part of robustness against SPOF as described in 17.3.7.2, an indication of whether MAC context information of the subordinate HR-MS is being shared by infrastructure stations shall be transmitted to HR-MS.

17.3.1.1.4 Relay link configuration

During establishing relay link, serving HR-BS transmits AAI-ARS-CONFIG-CMD message described in 16.2.3.57 to configure PHY layer parameter set including superframe number indicating the time to establish relay link.

While HR-BS is maintaining relay link, the serving HR-BS shall send AAI-ARS-ESI message described in 16.2.3.58 in the DL relay zone when the essential system information in SFH is changed. The HR-BS also shall send AAI-ARS-CONFIG-CMD message in the DL relay zone when PHY layer parameter needs to be reconfigured.

HR-BS acting as relay may transmit AAI-MM-ADV message with action type described in 16.2.3.xa to update PHY/MAC layer parameter after receiving AAI-ARS-ESI or AAI-ARS-CONFIG-CMD message.

17.3.1.1.5 Relay link release

If the HR-BS recovers from failure of backhaul, it may inform network or notify the current serving HR-BS of the HR-BS having recovered backhaul link through the backhaul network interface. The superordinate serving HR-BS may then initiate HR-MS handover back to the HR-BS in which the recovered HR-BS should be listed in the first priority. The HR-BS having recovered backhaul may store MAC context information of the serving MSs (basic capabilities, security capabilities, etc.). Such context information allows HR-MS to perform optimized network reentry when returning back to the HR-BS upon its recovery.

HR-BS transmits AAI-MM-ADV message with action type = 0b101 described in 16.2.3.xa including expected time of backhaul link up. When receiving the AAI-MM-ADV message, HR-MS performs either handover to neighbor infrastructure station and returns to the HR-BS at the expected time or waiting in the HR-BS until restarting service with available backhaul link.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]