| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Alternative Path Preparation |  |
| Date Submitted | 2011-07-11 |  |
| Source(s) | Wai Leong Yeow, Shoukang Zheng, Hoang Anh Tuan, Haiguang Wang, Jaya Shankar, Joseph Teo, Choong Hock Mar Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2254 E-mail: wlyeow@i2r.a-star.edu.sg |
| Re: | Call for Comments on the 802.16n Amendment Working Document <80216n-11_009r1.pdf> |  |
| Abstract | We propose signaling messages for alternative paths with multi-mode to prepare against SPOF. |  |
| Purpose | To be discussed and adopted in TGn AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Alternative Path Preparation

Wai Leong Yeow, Hoang Anh Tuan, Haiguang Wang, Shoukang Zheng, Jaya Shankar, Joseph Teo

Institute for Infocomm Research

# Introduction

A base station failure is very disruptive. All stations served by the BS will have to perform network entry to neighboring BSs, assuming that the stations already skip the neighboring BS scanning phase and use the most recent MOB_NBR_ADV list. Even so, there are three main problems:

- This will cause a usage spike in physical resources, and may cause neighboring BSs to be swarmed.

- Some stations may be left without any network connection even though 802.16n supports range extension with HR-MS forwarding, direct communications, and multi-mode operations.

- There may be further delay incurred since initial ranging is contention-based. Furthermore, all ongoing service flows will have to be reinitiated, resulting in longer delays for service re-connections at the upper layers.

The 802.16n SRD requires the HR-network to support establishment and maintenance of alternative paths to support fast recovery in the event of disruption. In this document, we define alternative path establishment as a preparation phase to deal with possible SPOF. We propose to factor the possibility of multi-mode into alternative path establishment in event of a BS failure, in order to minimize disruption caused in terms of recovery time and physical resource utilization. Specifically, the HR-network shall prepare and maintain a failover topology of alternate paths with role-changed stations, and support fast network recovery with failover configuration. We also discuss the scenario in which BS has backhaul failure and need to switch to RS mode to connect to a neighboring BS in order to maintain connectivity for its subordinate stations.

# Summary

We propose to factor the possibility of multi-mode into alternative path establishment in event of a BS failure, in order to minimize disruption caused. Specifically, we proposed mechanisms to prepare and maintain a failover topology of alternate paths with role-changed stations, and support fast network recovery with failover configuration.

# Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

17.2.7.2 Preparation for SPOF

The serving HR-BS transfers MS context information to the target HR-BS to prepare an alternative path for the HR-MS. The target HR-BS shall inform the serving HR-BS of a recommended ranging code and slot. A recommended ranging code and slot is similar to that of dedicated ranging except that they are not uniquely assigned or reserved.

The serving HR-BS also unicasts a MOB_BSHO-REQ message with mode = 0b111 to HR-MS to communicate alternative path information.

17.3.7.2 Preparation for SPOF

The serving HR-BS transfers MS context information to the target HR-BS to prepare an alternative path for the HR-MS. The target HR-BS shall inform the serving HR-BS of a recommended ranging code and slot. A recommended ranging code and slot is similar to that of dedicated ranging except that they are not uniquely assigned or reserved.

The serving HR-BS also unicasts a AAI-HO-CMD message with mode = 0b11 to HR-MS to communicate alternative path information.

[Modify 6.3.2.3.47 as follows:]

Table 144 – MOB_BSHO-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| … |  |  |
| Mode | 3 | 0b000: HO Request 0b001: MDHO/FBSS request: Anchor BS update with CID update 0b010: MDHO/FBSS request: Anchor BS update without CID update 0b011: MDHO/FBSS request: Diversity set update with CID update 0b100: MDHO/FBSS request: Diversity set update without CID update 0b101: MDHO/FBSS request: Diversity set update with CID update for newly added BS 0b110: MDHO/FBSS request: Diversity set update without CID update for newly added BS 0b111: Reserved. 0b111: Alternative Path. |
| Padding | 5 | Shall be set to zero. |
| If (Mode == 0b000 or 0b111) | - | - |
| … |  |  |
| HO_authorization policy indicator | 1 | Indicates whether Seamless HO mode is supported 0: Not supported 1: Supported |
| If (Mode == 0b000) { | - | - |
| Seamless HO mode flag | 1 | Indicates whether Seamless HO mode is supported 0: Not supported 1: Supported |
| } else If (Mode == 0b111) { | - | - |
| Role | 2 | 0b00: Stay as HR-MS; 0b01: Change to HR-RS; 0b10: Change to HR-BS; 0b11: Reserved. |
| Rendezvous_time | 8 | - |
| CDMA_code | 8 | - |
| Transmission_opportunity_offset | 8 | - |
| Basic CID | 16 | - |
| If (Role == 0b01) { |  |  |
| } else if (Role == 0b10) { |  |  |
| } |  |  |
| If (Role == 0b01 or 0b10) { |  |  |
| } |  |  |
| } |  |  |

[Modify 16.2.3.12 as follows:]

Table 695 – AAI-HO-CMD message format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Mode | 2 | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAAI-HO-CMD message shall not include any T-ABS. 0b11: Reserved. 0b11: Alternative Path preparation. |  |
| If (Mode == 0b00 or 0b11) { |  |  |  |
| … |  |  |  |
| Resource_Retain_Time | 16 | The duration in units of 100 ms to which the T-ABD set the ABS-Resource-Retain-Timer | Present if needed |
| If (HO Reentry Mode == 0b11) { |  |  |  |
| Role | 2 | 0b00: Stay as HR-MS; 0b01: Change to HR-RS; 0b10: Change to HR-BS; 0b11: Reserved. |  |
| If (Role == 0b01) { |  |  |  |
| } else if (Role == 0b10) { |  |  |  |
| } |  |  |  |
| If (Role == 0b01 or 0b10) { |  |  |  |
| } |  |  |  |
| } //end of If (HO Reentry Mode == 0b1) | - | - |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.