| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Network Recovery on Alternative Path |  |
| Date Submitted | 2011-07-11 |  |
| Source(s) | Wai Leong Yeow, Shoukang Zheng, Hoang Anh Tuan, Haiguang Wang, Jaya Shankar, Joseph Teo, Choong Hock Mar Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2254 E-mail: wlyeow@i2r.a-star.edu.sg |
| Re: | Call for Comments on the 802.16n Amendment Working Document <80216n-11_009r1.pdf> |  |
| Abstract | We propose signaling messages for alternative paths with multi-mode to prepare against SPOF. |  |
| Purpose | To be discussed and adopted in TGn AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Network Recovery on Alternative Path

Wai Leong Yeow, Hoang Anh Tuan, Haiguang Wang, Shoukang Zheng, Jaya Shankar, Joseph Teo

Institute for Infocomm Research

# Introduction

The 802.16n SRD requires the HR-network to support establishment and maintenance of alternative paths to support fast recovery in the event of disruption. In this document, we define procedure for network recovery using a previously prepared alternative path. We propose to factor the possibility of a mode change by the HR-MS to RS or BS mode during the recovery.

# Summary

We proposed a network recovery procedure for HR-MS on alternative path. It is based on HO procedure and optimizations are used whenever possible.

# Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

17.2.7.3 Recovery for SPOF

The recovery procedure uses information specified in the most recent MOB_BSHO-REQ message that has not expired. The expiry time of the message is ALT-PATH Interval. The trigger condition specified in the TLV shall be used to activate the recovery procedure. If a mode change to HR-RS or HR-BS is indicated, HR-MS shall change its mode accordingly before performing network reentry to the target station.

Network reentry including handover as described in 6.3.21 shall be supported in the event of SPOF. Whether MAC context information of the subordinate HR-MS is shared by the infrastructure stations shall be transmitted to HR-MS.

HR-MS shall use the recommended ranging slot and CDMA code specified in MOB_BSHO-REQ to perform a possibly contention-free network entry to the target station.

17.3.7.3 Recovery for SPOF

The recovery procedure uses information specified in the most recent AAI-HO-CMD message that has not expired. The expiry time of the message is ALT-PATH Interval. The trigger condition specified in the TLV shall be used to activate the recovery procedure. If a mode change to HR-RS or HR-BS is indicated, HR-MS shall change its mode accordingly before performing network reentry to the target station.

Network reentry including handover as described in 16.2.6 and 16.2.8.2.9 shall be supported in the event of SPOF. Whether MAC context information of the subordinate HR-MS is shared by the infrastructure stations shall be transmitted to HR-MS.

HR-MS shall use the recommended ranging slot and CDMA code specified in AAI-HO-CMD to perform a possibly contention-free network entry to the target station.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.