| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for Format of Network Discovery Signals for HR-MS Direct Communication without Infrastructure Stations |  |
| Date Submitted | 2011-07-11 |  |
| Source(s) | Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2256 E-mail: hwang@i2r.a-star.edu.sg |
| Re: | Call for Comments for 802.16n AWD |  |
| Abstract | In this contribution, we propose a framework for the HR-MSs communicate to each in case there is no infrastructure station such as HR-BS or HR-RS presented in the network. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for Format of Network Discovery Signals for HR-MS Direct Communication without Infrastructure Stations

Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

HR-MS to HR-MS direct communication is a new feature for IEEE 802.16 networks. It will be defined in the IEEE 802.16n amendment.  This feature will improve the capability of HR-MS significantly in various harsh environment and increase network throughput when local traffic dominates the network. In this contribution, we focus on the use cases of HR-MSs communicate to each other without getting support from infrastructure stations as figure 1 shows. A few HR-MSs connect to each other and form a communication network.

Figure 1: Use cases of HR-MS to HR-MS direct communication without infrastructure stations

# 2. Format of Network Discovery Signals

The current 802.16 standard is designed for cellular communication and a BS, which is a central network controller, is necessary for the communication. Transmissions of MSs in the networks are controlled by BS. To avoid significant change over the protocols, we also propose to a central network controller, named as coordinator, in the networks formed by HR-MS.  The coordinator basically is a simplified BS.  Since every HR-BS has the potential to become a coordinator in certain location, and in many cases, only one of them should become a coordinator, else, the network may be separated into a few networks. Therefore, HR-MS should discover the network topology before they make a decision to become a coordinator.

To facilitate HR-MSs discover the network topology, a signaling format should be defined for both 802.16-2009 and 802.16m interface. To avoid redesign of the preambles, we propose to reuse the preambles defined in both 802.16-2009 and 802.16m.

For 802.16-2009, we propose to send out a preamble and followed by some necessary discovery information. Other HR-MSs should know that the signal is from HR-MS by decoding the information followed the preamble.  Figure 1 shows an example.

Figure 2: Network Discovery Message Transmission in 802.16n Network

For 802.16m, we propose that HR-MS transmits PA-Preamble first, and is followed by an SA-Preamble. Discovery information should follow the SA-Preamble. Figure 2 shows an example.

Figure 3: Discovery Signals Transmission for 802.16m-based HR-MS Networks without Infrastructure stations

In this proposal, we further specify the detailed message format for the HR-MS discovery.

We propose that an HR-MS is allowed to indicate its intention of communicating with other HR-MS in the discovery message and the target HR-MS is allowed to indicate whether it accepts/rejects the invitation.

We also propose that HR-MS shall be allowed to transmit a short data packet. The data packet may from upper layer.

# 3. Summary

In this proposal, we have proposed format of signals used by HR-MS in discovering other HR-MSs nearby when there is no infrastructure stations.

# 4. Text Proposal for IEEE 802.16n AWD

Xxx

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n Document (XXX --- document number)]

17.2.2.5 HR-MS Discovery for Direct Communication without Infrastructure

When HR-MS cannot receive any BS preamble from any infrastructure station or an HR-MS that is associated with an infrastructure station, and HR-MS direct communication without infrastructure is permitted by device configuration, then HR-MSs are allowed to transmit network discovery signals to the network.

When HR-MS sends out network discovery messages, to avoid collision with other HR-MSs, it should follow a random-back off mechanism as follows:

1) A back-off timer shall be started.

2) When the timer is timeout, HR-MS should sense the channel for the presence of preambles first. If no preambles detected, then the HR-MS should transmit the discovery message. If a preamble has been detected, then node should hold the transmission and restart the timer.

3) HR-MS should get the value for the duration of back-off from a window, for example, from a window of [wmin, wmax], the size of window can be adjusted based on the traffic of networks. The value of Wmin and Wmax are TBD.

The network discovery message shall take the following format: a frame preamble shall be transmitted first followed by control and discovery information. The control information includes FCH, DL-MAP. UL-MAP shall be omitted. Discovery information should follow the DL-MAP. A data packet may be transmitted as part of the discovery information.

When HR-MS sends out network discovery messages, to avoid collision with other HR-MSs, it should follow a random-back off mechanism as follows:

1) A back-off timer shall be started.

2) When the timer is timeout, HR-MS should sense the channel for the presence of preambles first. If no preambles detected, then the HR-MS should transmit the discovery  message. If a preamble has been detected, then node should hold the transmission and  restart the timer.

3) HR-MS should get the value for the duration of back-off from a window, for example, from a window of [wmin, wmax], the size of window can be adjusted based on the traffic of  networks. The value of Wmin and Wmax are TBD.

For the FCH, it takes the same format as defined in 8.4.4.4.

For the DL-MAP, the DCD messages transmitted in the PHY Synchronization Field shall set the value of frame duration code to 255. The value indicates that the message is not from a BS, it is from an HR-MS for the discovery purpose.  DCD count shall be set to zero. The base station ID shall be set to the MAC address of the current HR-MS.

17.2.2.5.1 DC_DISCOV_Message

The discovery message follows the DL-MAP and shall take the following encoding format:

Table xxx: DC discovery message encodings

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DC_DISCOV_Message() { | — | — |
| Length | 16 | The length of the message |
| NBR Count | 8 | Number of neighboring HR-MSs |
| for(i=0;i<n;i++){ |  |  |
| DC_DISCOV_IE(); |  |  |
| } |  |  |
| } |  |  |

MAC Address

MAC address is the 48 bit address assigned to the HR-MS device.  It shall be used as unique identity of the HR-MS in network discovery.

NBR Count

The value indicates the number of neighboring HR-MSs that the current HR-MS discovered via the neighbor discovery process.

DC_DISCOV_IE

Various information such as name of the HR-MS, MAC address of the neighboring node, invitation for communication etc is contained in the IEs.

17.2.2.5.2 Encoding of DC_DISCOV_IEs

The IEs contained in discovery message has a common encoding format as follows:

Table xxx: DC discovery IE encodings

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DC_DISCOV_IE() { | — | — |
| Type | 8 | — |
| Length | 8 | The length of data contained in the value field |
| Value | variable |  |
| } |  |  |

A few type of IE has been defined in table xxx [number TBD].

Table xxx: DC discovery IE types.

| Type | Name |
| --- | --- |
| 0x01 | DC_DISCOV_NODE_NAME |
| 0x02 | DC_DISCOV_NBR_ADDR |
| 0x03 | DC_DISCOV_INVITE |
| 0x04 | DC_DISCOV_INVITE_ACCEPT |
| 0x05 | DC_DISCOV_INVITE_REJECT |
| 0x06 – 0xfe | Reserved |
| 0xff | DC_DISCOV_DATA |

17.2.2.5.2.1 DC_DISCOV_NODE_NAME

The node name is an ASCII string. The maximum length is 16 bytes.

Table xxx: DC HR-MS Name

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x01 | 1 – 16 | A name given by the user of HR-MS |

17.2.2.5.2.2 DC_DISCOV_NBR_ADDR

It contains MAC addresses of neighboring HR-MSs discovered by the current HR-MS. Each MAC address takes six bytes. Multiple MAC addresses can be transmitted in the same DC_DISCOV_NBR_ADDR IE.

Table xxx: DC Neighbor Address IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x02 | variable | MAC Address of the HR-MSs |

17.2.2.5.2.3 DC_DISCOV_INVITE

The IE contains MAC address of the HR-MS that the current HR-MS want to setup connections. Multiple MAC addresses can be contained in the IE.

Table xxx: DC Invitation IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x03 | variable | MAC address of the invited HR-MS |

17.2.2.5.2.4 DC_DISCOV_INVITE_ACCEPT

The current HR-MS decided to accept the invitation. It intends to join the HR-MS network once the HR-MS become a coordinator.

Table xxx: DC Accept IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x04 | 6 | MAC address |

The MAC address belongs to the HR-MS who sends out a DC_DISCOV_INVITE_ACCEPT message

17.2.2.5.2.4 DC_DISCOV_INVITE_REJECT

The current HR-MS rejects the invitation from the HR-MS. The IE contains the MAC address of the HR-MS who sends out a DC_DISCOV_INVITE message. It indicates that the current HR-MS declines to communicate with the other HR-MS.

Table xxx: DC Reject IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x05 | 6 | MAC address of the inviting HR-MS |

17.2.2.5.2.6 DC_DISCOV_DATA

A short data packet is contained in the IE. The interpretation of the data is up to application.

Table xxx: DC Data IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0xff | 1 – 255 | First 6 bytes is the MAC address of intended HR-MS and follows by the data from upper layer. |

8.4.5.2 Frame duration codes

Table 320—OFDMA frame duration (Tf ms) codes

| Code (N) | Frame duration (ms) | Frames per second |
| --- | --- | --- |
| 0 | Reserved | N/A |
| 1 | 2 | 500 |
| 2 | 2.5 | 400 |
| 3 | 4 | 250 |
| 4 | 5 | 200 |
| 5 | 8 | 125 |
| 6 | 10 | 100 |
| 7 | 12.5 | 80 |
| 8 | 20 | 50 |
| 9-2554 | Reserved |  |
| 255 | Infinity | 0 |

The code 255 is used for HR-MS direct communication without infrastructure station only.

17.3.2.5 HR-MS Discovery for Direct Communication without Infrastructure

When HR-MS cannot receive any BS preamble from any infrastructure station or an HR-MS that is associated with an infrastructure station, and HR-MS direct communication without infrastructure is permitted by device configuration, then HR-MSs are allowed to transmit network discovery signals to the network.

When HR-MS sends out network discovery messages, to avoid collision with other HR-MSs, it should follow a random-back off mechanism as follows:

1) A back-off timer shall be started.

2) When the timer is timeout, HR-MS should sense the channel for the presence of  preambles first. If no preambles detected, then the HR-MS should transmit the discovery message. If a preamble has been detected, then node should hold the transmission and restart the timer.

3) HR-MS should get the value for the duration of back-off from a window, for example,  from a window of [wmin, wmax], the size of window can be adjusted based on the traffic of  networks. The value of Wmin and Wmax are TBD.

The network discovery message shall take following format: frame preambles, PA-Preamble and SA-Preamble shall be transmitted first, and then followed by the discovery information.

When HR-MS sends out network discovery messages, to avoid collision with other HR-MSs, it should follow a random-back off mechanism as follows:

1) A back-off timer shall be started.

2) When the timer is timeout, HR-MS should sense the channel for the presence of preambles first. If no preambles detected, then the HR-MS should transmit the discovery message. If a preamble has been detected, then node should hold the transmission and restart the timer.

3) HR-MS should get the value for the duration of back-off from a window, for example, from a window of [wmin, wmax], the size of window can be adjusted based on the traffic of networks. The value of Wmin and Wmax are TBD.

Based on the preamble pattern, HR-MS knows the signals are from a BS or from HR-MSs. The discovery message shall be transmitted after the SA-Preamble and use radio resource specified by SA-Preamble.  The radio resource is TBD.

17.3.2.5.1 DC_DISCOV_Message

The discovery message shall take the following encoding format:

Table xxx: DC discovery message encodings

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DC_DISCOV_Message() { | — | — |
| MAC Address | 48 | MAC address of the device |
| Length | 16 | The length of the message |
| NBR Count | 8 | Number of neighboring HR-MSs |
| for(i=0;i<n;i++){ |  |  |
| DC_DISCOV_IE(); |  |  |
| } |  |  |
| } |  |  |

MAC Address

MAC address is the 48 bit address assigned to the HR-MS device.  It shall be used as unique identity of the HR-MS in network discovery.

NBR Count

The value indicates the number of neighboring HR-MSs that the current HR-MS discovered via the neighbor discovery process.

DC_DISCOV_IE

Various information such as name of the HR-MS, MAC address of the neighboring node, invitation for communication etc is contained in the IEs.

17.3.2.5.2 Encoding of DC_DISCOV_IEs

The IEs contained in discovery message has a common encoding format as follows:

Table xxx: DC discovery IE encodings

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DC_DISCOV_IE() { | — | — |
| Type | 8 | — |
| Length | 8 | The length of data contained in the value field |
| Value | variable |  |
| } |  |  |

A few type of IE has been defined in table [number TBD]

Table xxx: DC discovery IE types.

| Type | Name |
| --- | --- |
| 0x01 | DC_DISCOV_NODE_NAME |
| 0x02 | DC_DISCOV_NBR_ADDR |
| 0x03 | DC_DISCOV_INVITE |
| 0x04 | DC_DISCOV_INVITE_ACCEPT |
| 0x05 | DC_DISCOV_INVITE_REJECT |
| 0x06 – 0xfe | Reserved |
| 0xff | DC_DISCOV_DATA |

17.3.2.5.2.1 DC_DISCOV_NODE_NAME

The node name is an ASCII string. The maximum length is 16 bytes.

Table xxx: DC HR-MS Name

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x01 | 1 – 16 | A name given by the user of HR-MS |

17.3.2.5.2.2 DC_DISCOV_NBR_ADDR

It contains MAC addresses of neighboring HR-MSs discovered by the current HR-MS. Each MAC address takes six bytes. Multiple MAC addresses can be transmitted in the same DC_DISCOV_NBR_ADDR IE.

Table xxx: DC Neighbor Address IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x02 | variable | MAC Address of the HR-MSs |

17.3.2.5.2.3 DC_DISCOV_INVITE

The IE contains MAC address of the HR-MS that the current HR-MS want to setup connections. Multiple MAC addresses can be contained in the IE.

Table xxx: DC Invitation IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x03 | variable | MAC address of the invited HR-MS |

17.3.2.5.2.4 DC_DISCOV_INVITE_ACCEPT

The current HR-MS decided to accept the invitation. It intends to join the HR-MS network once the HR-MS become an HR-BS.

Table xxx: DC Accept IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x04 | 6 | MAC address of the inviting HR-MS |

The MAC address belongs to the HR-MS who sends out a DC_DISCOV_INVITE_ACCEPT message

17.3.2.5.2.4 DC_DISCOV_INVITE_REJECT

The IE contains the MAC address of the HR-MS who sends out a DC_DISCOV_INVITE_ACCEPT message and the current HR-MS reject the invitation. It intends to not join the HR-MS network when the HR-MS become an HR-BS.

Table xxx: DC Reject IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0x05 | 6 | MAC address of the inviting HR-MS |

17.3.2.5.2.6 DC_DISCOV_DATA

A short data packet is contained in the IE. The interpretation of the data is up to application.

Table xxx: DC Data IE

| Type (1 byte) | Length (1 byte) | Value (variable length) |
| --- | --- | --- |
| 0xff | 1 – 255 | First 6 bytes is the MAC address of intended receving HR-MS and followed by data packets from upper layer |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 201.