| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A Procedure to Setup HR-MS to HR-MS Direct Communication Link with Supporting from Infrastructure Stations |  |
| Date Submitted | 2011-07-11 |  |
| Source(s) | Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2256 E-mail: hwang@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this proposal, we proposed a procedure for HR-networks in setting up HR-MS direct communication in HR-BS or HR-RS appears in the network. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# A Procedure to Setup HR-MS Direct Communication with Supporting from Infrastructure Station

Haiguang Wang, Hoang Anh Tuan, Jaya Shankar,

Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

Supporting the HR-MS to HR-MS direct communication is required by IEEE 802.16n network. The HR-MS direct communication can happen with our without the supporting form infrastructure nodes. In this contribution, we focus on the use cases of two HR-MS either direct under the same HR-BS or the same HR-RS as figure 1 shows.

Figure 1: Use cases of HR-MS to HR-MS direct communication with infrastructure stations

When receive DSA-REQ from HR-MS, the HR-BS/RS shall look through the HR-MS neighbor table maintained at the HR-BS and knows the possibility of whether a direct communication exists between the source and destination. If the two HR-MS are neighbors, that means a link may exist. However, the HR-BS has to consider other factors such as the link quality between source and destination HR-MS.

# 2. Summary

In this contribution, we provide a procedure for 802.16n HR-MS in setting up direct communication link with the help from infrastructure stations.

# 4. Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n Document (XXX --- document number)]

17.2.2.4 HR-MS Direct Communication with Infrastructure Stations

HR-BS shall check DSA_REQ messages received from HR-MS and determine whether HR-MS direct communication can be adopted for a flow. If a direct communication is feasible between the source and destination HR-MSs, then the HR-BS shall help the source and destination HR-MSs setting up a direct communication link through DSA signaling.

HR-BS knows the possibility of setting up a direct communication between two HR-MSs by checking the HR-MS neighbor tables. If the two nodes are neighbor, HR-MS may schedule the two HR-MSs to do fine channel measurement and determine whether a direct communication link should be setup or not.

Before a service flow can be conveyed over the link between source and destination HR-MS, a direct communication link shall be setup first if it has not been setup yet.

17.3.2.4 HR-MS Discovery for Direct Communication without Infrastructure

HR-BS shall check DSA_REQ messages received from HR-MS and determine whether HR-MS direct communication can be adopted for a flow. If a direct communication is feasible between the source and destination HR-MSs, then the HR-BS shall help the source and destination HR-MSs setting up a direct communication link through DSA signaling.

HR-BS knows the possibility of setting up a direct communication between two HR-MSs by checking the HR-MS neighbor tables. If the two nodes are neighbor, HR-MS may schedule the two HR-MSs to do fine channel measurement and determine whether a direct communication link should be setup or not.

Before a service flow can be conveyed over the link between source and destination HR-MS, a direct communication link shall be setup first if it has not been setup yet.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.