| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | # Signaling for Flow Setting up over HR-MS Direct Communication Link |  |
| Date Submitted | 2011-07-11 |  |
| Source(s) | Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2256 E-mail: hwang@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this proposal, we proposed a procedure for HR-networks in setting up HR-MS direct communication in HR-BS or HR-RS appears in the network. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Signaling for Flow Setting up over HR-MS Direct Communication Link

Haiguang Wang, Hoang Anh Tuan, Jaya Shankar,

Shoukang Zheng, Yeow Wai Leong, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

HR-MS to HR-MS direct communication is new feature for IEEE 802.16 networks. It will be defined in the IEEE 802.16n amendment.  This feature will significantly improve the capability of HR-MS in various harsh environment and increase network throughput when local traffic dominates the network.

In 802.16n System Requirement Document [1] section 6.1.3.1, it says that “HR-MS shall provide direct communication i.e. the origination and termination of the data are at the HR-MS. Association establishment procedure of an HR-MS to another HR-MS shall be supported.”.

Since the requirement does not set constraint on whether HR-MSs have associated to an infrastructure stations, in this contribution, we assume an HR-BS is presented in the network and the HR-MSs involved in the direct communication are associated to the same HR-BS or HR-RS.

# 2. HR-MS to HR-MS Direct Communication

Supporting the HR-MS to HR-MS direct communication is required by IEEE 802.16n network. The HR-MS direct communication can happen with our without the supporting form infrastructure nodes. In this contribution, we focus on the use cases of two HR-MS either direct under the same HR-BS or the same HR-RS as figure 1 shows. We propose a signaling procedure for set up direct communication between the source and destination..

Figure 1: Use cases of HR-MS to HR-MS direct communication with infrastructure stations

# 3. Summary

In this contribution, we provide a procedure for 802.16n HR-MS in setting up direct communication link with the help from infrastructure stations.

# 4. Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n Document (XXX --- document number)]

17.2.2.4 HR-MS Direct Communication with Infrastructure Stations

17.2.2.4.1 Direct communication service flow creation

After a direct communication link has been setup between the source and destination HR-MS, the source HR-MS can setup flows over the direct communication link.

A direct communication setup protocol is illustrated in Figure xxx and described in detail in 16.3.2.1.2.

When HR-BS receives DSA-REQ from HR-MS, it checks whether a direct communication can be setup between the source and destination. If direct communication can be setup, HR-BS sends back a DSX_RVD to source HR-MS with indication of direct communication link. If direct communication is possible, the HR-BS holds on the transmission of DSA-RSP to source HR-MS until it finishes DSA negotiation with the destination HR-MS.

Figure XXX: The establishment of direct communication between HR-MS

17.2.2.4.2 Dynamic Service Flow Modification and Deletion

In addition to the methods presented in 17.2.2.4.1 for creating service flows, protocols are defined for modifying and deleting service flows; see x.x.x.x.x and x.x.x.x.x.

The modification of parameters of a service flow over direct communication link also involves both the source and destination HR-MS.

If the modification is initiated by one HR-MS, then if and only if the HR-BS and the other HR-MS agree with the modification, then the modification can be applied. If the modification is initiated by the HR-BS, then if and only if both HR-MSs agrees with the change then the modification can be applied.

17.2.2.4. 3 Dynamic Service Addition

When HR-BS receives a DSA_REQ from an HR-MS and find that direct communication can be setup between the source and destination HR-MSs by checking the source and destination addresses after testing the integrity. If direct communication can be setup, the Confirmation Code in DSX_RVD message shall be set to direct-comm-setup so that the source HR-MS knows that a direct communication link is going to be setup between the source and destination HR-MSs. Else, the HR-BS processes the DSA_REQ as a normal request.

HR-BS creates a flow_id based on the QOS requirement in the DSA_REQ.

Before sending DSA_RSP back to the source HR-MS, the HR-BS sends a DSA_REQ to the destination HR-MS first with the flow_id created for direct communication.

The HR-BS sends DSA_REQ to the destination HR-MS with flow_id and indication of direct communication.

Table XXX – DSA Process for the direct communication

| Source SS | Destination SS |  | BS |
| --- | --- | --- | --- |
| New service flow needed |  |  |  |
| Check if resources are available |  |  |  |
| Send DSA-REQ Set Timers T7 and T14 |  | ---DSA-REQ--> | Receive DSA-REQ |
|  |  |  | DSA-REQ integrity valid |
| Timer T14 Stops; If direct communication is feasible, Timer T7 stops; Set Timer T74. |  |  | Check whether source and destination SS can support direct communication. |
|  |  | <-- DSX-RVD-- | Check whether source and destination are neighbours. |
|  |  |  | Check whether SS is authorized for Service |
|  |  |  | Check whether service flow QoS can be Supported |
|  |  |  | Create SFID |
|  | Receive DSA-REQ | <-- DSA-REQ-- | Send DSA-REQ Set Timer T7 |
|  | Confirm that SS can support service flow |  |  |
|  | Add DL SFID (if present) |  |  |
|  | Enable reception on any new DL service flow |  |  |
|  | Send DSA-RSP | ---DSA-RSP--> | Receive DSA-RSP Timer T7 Stops |
|  |  |  | Enable transmission (DL) or reception (UL) of data on new service flow |
|  | Receive DSA-ACK | <-- DSA-ACK-- | Send DSA-ACK |
|  | Enable transmission on new UL service flow |  |  |
| Receive DSA-RSP Timer T74 Stops |  | <--DSA-RSP--- | Send DSA-RSP |
| If ActiveQoSParamSet is non-null, Enable transmission or reception of data on new service flow |  |  |  |
| Send DSA-ACK |  | ---DSA-ACK--> | Receive DSA-ACK |
|  |  |  | If DL ActiveQoSParamSet is non-null, Enable transmission of data on new DL service flow |

17.2.2.4.4 Dynamic Service Modification

With direct communication, the data flow is uni-direction, from source to destination HR-MS. Only source HR-MS and HR-BS are allowed to modify the service flow parameters.

Source HR-MS initiated Modification

| Source HR-MS | Destination HR-MS |  | HR-BS |
| --- | --- | --- | --- |
| Service flow requires modifying |  |  |  |
| Send DSC-REQ Set Timers T74 and T14 |  | --------- DSC-REQ ----> | Receive DSC-REQ |
| Timer T14 Stops |  | <------ DSX-RVD ------- | DSC-REQ integrity valid |
|  |  |  | Validate Request |
|  | Receive DSC-REQ | <------ DSC-REQ ------- | Set Timers T7 |
|  | Validate request |  |  |
|  | Modify service flow |  |  |
|  | Send DSC-RSP | --------- DSC-RSP ----> | Receive DSC-RSP Timer T7 Stops |
|  |  |  | Increase Channel Bandwidth if Required |
|  | Receive DSC-ACK | <------ DSC-ACK ------- | Send DSC-ACK |
| Receive DSC-RSP Timer T74 Stops |  | <------ DSC-RSP -------- | Send DSC-RSP |
| Reduce Payload Bandwidth if Required |  |  |  |
| Send DSC-ACK |  | --------- DSC-ACK ----> | Decrease channel Bandwidth if Required |
| Increase Payload Bandwidth if Required |  |  |  |

HR-BS Initiated

HR-BS can do the modification over DSC transaction with source and destination HR-MSs separately but with certain order.

In case bandwidth allocated to the direct communication link will be increased, the HR-BS should finish the transaction with destination HR-MS first and followed by the DSC transaction with source HR-MS.

| Source HR-MS | Destination HR-MS |  | HR-BS |
| --- | --- | --- | --- |
|  |  |  | Service flow requires modifying |
|  | Receive DSC-REQ | <------ DSC-REQ ------- | Send DSC-REQ Set Timers T74 and T14 |
|  | Validate request |  |  |
|  | Modify service flow |  |  |
|  | Send DSC-RSP | --------- DSC-RSP ----> | Receive DSC-RSP Timer T7 Stops |
|  | Receive DSC-ACK | <------ DSC-ACK ------- | Send DSC-ACK |
| Receive DSC-REQ |  | <------ DSC-REQ ------- | Send DSC-REQ Set Timers T74 and T14 |
| Validate request |  |  |  |
| Modify service flow |  |  |  |
| Send DSC-RSP |  | --------- DSC-RSP ----> | Receive DSC-RSP Timer T7 Stops |
|  |  |  | Increase Channel Bandwidth if Required |
| Receive DSC-ACK |  | <------ DSC-ACK ------- | Send DSC-ACK |
| Increase Payload Bandwidth if Required |  |  |  |

In case bandwidth allocated to the direct communication link will be reduced, the HR-BS should finish the transaction with source HR-MS first and followed by the DSC transaction with destination HR-MS.

| Source HR-MS | Destination HR-MS |  | HR-BS |
| --- | --- | --- | --- |
|  |  |  | Service flow requires modifying |
| Receive DSC-REQ |  | <------ DSC-REQ ------- | Send DSC-REQ Set Timers T74 and T14 |
| Validate request |  |  |  |
| Modify service flow |  |  |  |
| Reduce Payload Bandwidth if Required |  |  |  |
| Send DSC-RSP |  | --------- DSC-RSP ----> | Receive DSC-RSP Timer T7 Stops |
| Receive DSC-ACK |  | <------ DSC-ACK ------- | Send DSC-ACK |
|  | Receive DSC-REQ | <------ DSC-REQ ------- | Send DSC-REQ Set Timers T74 and T14 |
|  | Validate request |  |  |
|  | Modify service flow |  |  |
|  | Send DSC-RSP | --------- DSC-RSP ----> | Receive DSC-RSP Timer T7 Stops |
| Receive DSC-ACK |  | <------ DSC-ACK ------- | Send DSC-ACK |
|  |  |  | Decrease channel Bandwidth if Required |

17.2.2.4.5 Dynamic Service Deletion

An HR-MS wishing to delete a service flow over HR-MS direct communication link generates a delete request to the HR-BS using a DSD-REQ message. The HR-BS finds the other HR-MS that also associates to the service flow first and then removes the service flow. After that, the HR-BS generates a response using a DSD-RSP message to the HR-MS who sends DSD_REQ. At the same time, the HR-BS also generates DSD-REQ to the other HR-MS who also associates to this service flow. The HR-MS receives the DSD-REQ removes the service flow and send back a DSD-RSP to the HR-BS. This process is illustrated in Table XXX.

Table XXX: DSD initiated by HR-MS for service flow over direct communication link

| HR-MS | HR-MS |  | HR-BS |
| --- | --- | --- | --- |
| Service flow no longer needed |  |  |  |
| Delete service flow |  |  |  |
| Send DSD-REQ |  | ---DSD-REQ--> | Receive DSD-REQ |
|  |  |  | Verify HR-MS is service flow “owner” |
|  |  |  | Determine associated HR-MS for this service flow |
|  |  |  | Delete service flow |
|  |  | <---DSD-RSP--- | Send DSD-RSP |
| Receive DSD-RSP | Receive DSD-REQ | <---DSD-REQ--- | Send DSD-REQ |
|  | Delete service flow |  |  |
|  | Send DSD-RSP | ---DSD-RSP---> | Receive DSD-RSP |

A BS wishing to delete a dynamic service flow over HR-MS direct communication link generates two delete requests to the associated HR-MSs respectively using a DSD-REQs. The HR-MSs removes the service flows and generate responses using a DSD-RSPs. This process is illustrated in Table xxx.

Table XXX: DSD initiated by HR-BS for service flow over direct communication link

| HR-MS | HR-MS |  | HR-BS |
| --- | --- | --- | --- |
|  |  |  | Service flow no longer needed |
|  |  |  | Delete service flow |
|  |  |  | Determine associated SS for this service flow |
| Receive DSD-REQ |  | <---DSD-REQ--- | Send DSD-REQ |
| Delete service flow | Receive DSD-REQ | <---DSD-REQ--- | Send DSD-REQ |
| Send DSD-RSP | Delete service flow | ---DSD-RSP---> | Receive DSD-RSP |
|  | Send DSD-RSP | ---DSD-RSP---> | Receive DSD-RSP |

17.2.2.4.6 Management Messages

17.2.2.4.6.1 DSA_REQ

When HR-BS establishes direct communication between the source and destination HR-MSs, the DSA_REQ from HR-BS to the destination HR-MS shall contain a TLV that indicates of direct communication link setting up.

The TLV for direct communication is defined in 11.13.46.

17.2.2.4.6.2 DSA_RSP

When HR-BS establishes service flow over direct communication between the source and destination HR-MSs setting up direct communication, the DSA_RSP from HR-BS to the source HR-MS shall indicate by a TLV that a direct communication link should be used for the coming flow.

17.2.2.4.6.3 DSA_ACK

After receiving the DSA_ACK from the destination HR-MS, the HR-BS shall send DSA_RSP to the source HR-MS.

17.2.2.4.6.4 DSX_RVD

When setting up a direct communication link between source and destination HR-MSs, HR-BS should set the confirmation code in DSX_RVD to direct-com-setup as defined in table 607.

10. 1 Global values

Table 554—Parameters and constants

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| SS | T74 | Wait for DSA/DSC Acknowledge timeout in case the flow runs over a direct communication link | — | — | 600 ms |

11.13 Service flow management encodings

Table 606—Service flow encodings

| Type | Parameter |
| --- | --- |
| 58 | Direct Communication |

Table 607—CC values

| CC | Status |
| --- | --- |
| 19 | direct-comm-setup |

11.13.46

The value of this field specifies that the flow specified in this DSA_REQ will be transmitted over a direct communication link.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 145.58 | 1 | 0 | DSA_REQ |

17.3.2.4 HR-MS Direct Communication with Infrastructure

After a direct communication link has been setup between the source and destination HR-MS, the source HR-MS can setup flows over the direct communication link.

A direct communication setup protocol is illustrated in Figure xxx and described in detail in 17.3.2.1.2.

Figure XXX: The establishment of direct communication between HR-MS

When receive AAI-DSA-REQ from HR-MS, if the BS already setup a direct communication link between the source and destination HR-MS and it intends to setup the flow over the direct communication link, then the HR-MS shall send a AAI-DSA_RSP to source HR-MS and indicate that it is setup a flow over direct communication. At the same time, the HR-BS shall send AAI-DSA_REQ to the destination HR-MS with an indication of the direct communication flag. The destination HR-MS shall send back a AAI-DSA_RSP with indication of accept/reject of direct communication. The HR-BS sends a AAI-DSA_ACK back to the destination HR-MS. The HR-BS shall send a AAI-DSA-RSP to the source HR-MS with indication of accept/reject of flow setup with indication of type. If direct communication setup is rejected, the flow shall be setup on the uplink in a normal way.

17.3.2.4.1 Dynamic Service Flow Modification and Deletion

When HR-BS initiates the service flow deletion and the target flow is over a direct communication link, it should send AAI-DSD to the two HR-MS respectively. When source/destination HR-MS initiates the service flow deletion and the target flow is over a direct communication link, HR-BS should also send a AAI-DSD to the destination/source HR-MS also.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.