| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Frame Structure for Talk-around Direct Communications |  |
| Date Submitted | 2011-07-19 |  |
| Source(s) | Young-Ho Jung, Jihoon Choi Korea Aerospace University Sungcheol Chang, Eunkyung Kim, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: jihoon@kau.ac.kr, yhjung@kau.ac.kr scchang@etri.re.kr |
| Re: | Call for Comments on the 802.16n AWD |  |
| Abstract | This provides AWD text proposals for frame structure of talk-around direct communications |  |
| Purpose | To be discussed and adopted by 802.16 TGn |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Frame Structure for Talk-around Direct Communications

Young-Ho Jung, Jihoon Choi

Korea Aerospace University

Sungcheol Chang, Eunkyung Kim, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

# 1. Introductions

To support direct communication in IEEE 802.16n, some dedicated resources can be assigned as described in [1]. By dedicatedly reserving a part of communication resources for infra-structure communications, the reserved resources are used to provide multiple pairs of direct communication in OFDMA (orthogonal frequency division multiple access) manner with contention based transmission.

In this contribution, we propose the frame structure for talk-around direct communications in IEEE 802.16n, and define physical channels for talk-around direct communications. We propose the separate proposals for details of physical channels for talk-around direct communications in [2] and [3].

## 2. Frame structure for talk-around direct communications

For talk-around direct communication, two types of infra-structure communication resources are dedicatedly assigned:

- Common Direct Mode Zone: set of PRUs with fixed size and positions which are commonly assigned to all cells (must be CRU)

- Cell Specific Direct Mode Zone: additionally assigned direct-mode resource blocks independently assigned by each HR-BS (CRU or DRU) and information to receive cell specific direct mode region can be obtained from common direct mode zone

Figure 1 shows an example of talk-around direct communication resource allocation. In the figure, some part of uplink infra structure resources are assigned for common direct mode zone, and some part of uplink and downlink infra structure communication resources are assigned for cell specific direct mode zone. By assigning the same physical resources for common direct mode zone the following benefits can be obtained:

- Reduce the overhead of control channels to transmit allocation information of resources for direct-mode communication

- Reduce the computation complexity and power to obtain synchronization for direct communication, specially for the HR-MSs in the outside of HR-BS coverage

Figure 1. An example of talk-around direct communication resource allocation

In [4] we proposed to assign four PRUs with highest PRU indices for each bandwidth option for common direct mode zone. These four PRUs are always assigned for sub-band based PRUs except of DSAC = 0 or USAC = 0, and because the sub-band based PRUs are always assigned for CRUs, these four PRUs can be commonly assigned for direct communication zone for all cells without effecting the resource permutation of infra-structure communication resources.

As discussed in [1], if some of uplink resources are utilized for direct communication, the main victims of mutual interference between direct communication and cellular communication are HR-MSs, however, if some of downlink resources are utilized for direct communication, the main victims of mutual interference between direct communication and cellular communication are cellular mode MSs. Therefore we propose to assign the uplink part of PRUs with highest PRU indices for each bandwidth option for common direct mode zone. The downlink parts of these PRUs can be preferentially assigned for cell specific direct mode region when some of downlink resources are assigned for cell specific direct mode region.

Logical frame of common direct mode zone is composed by collecting all resources of common direct mode zone in a superframe as shown in Figure 2. In the example, there are three uplink subframes for each 5msec frame. Frame structure of cell specific direct mode zone is extension of common direct mode frame structure and it will be proposed in the next meeting.

Figure 2. An example of common direct mode logical frame construction

There are three physical channels for talk-around direct communication:

- Synchronization Channel (Sync-CH)

- Dedicated Channel(Ded-CH)

- Supplementary Channel(Sup-CH)

The first subframe of the common direct mode frame is occupied by synchronization channel. All the HR-MSs receives the synchronization signal on the Synchronization channel except HR-MSs transmitting the synchronization signals. The HR-MSs are synchronized to the received synchronization signal if the signal timing has priority to HR-MS’s synchronization timing itself. The details of timing priority is FFS. Some HR-MSs sends the synchronization signal on the Synchronization channel at selected subframes. HR-MS selects its slots for sending synchronization timing in distributed way. The details of how to select is FFS. The synchronization channel is composed of two parts: synchronization channel preamble part and synchronization channel message part. The synchronization channel preamble part is used for acquiring time and frequency synchronization, and synchronization channel preamble part is used for transmitting some information including frame structure information, hop count, transmitter ID et. al. The detailed design of synchronization channel is discussed in [2].

Resources excluding the first subframe assigned for synchronization channel are assigned for dedicated channels and supplementary channels. A Dedicated channel is a resource to send direct communication packets for two HR-MSs or a group of HR-MSs. An HR-MS sends a packet on one or more than one subchannels of dedicated channel and the other HR-MSs receives the packet on it. If two HR-MSs and a group of HR-MSs are involved, the transmissions are unicast and multicast, respectively. How to configure the resource of dedicated channel is discussed in [3]. The resources for dedicated channel is divided into small size sub-blocks (mRB: mini-Resource Block), as shown in Figure 3. One mRB is composed of 6 subcarriers-by-6 OFDM symbols, and a dedicated subchannel is composed of a collection of nine mRBs distributed across the entire frequency region.

Figure 3. An example of common direct mode zone resource segmentation and construction of mRB

A supplementary sub-channel is one-to-one mapped with each dedicated sub-channel. By using the supplementary sub-channel, the following MAC messages, PHY signalings and short feedback messages related with the corresponding dedicated subchannel are transmitted.

- MAC messages: RTS, CTS, MCS information, ranging response et. al.

- PHY signalings: periodic ranging sequence, sounding signal et. al.

- Short feedback messages: ACK, NACK, CQI, CSI, RI (rank information) et. al.

An mRB for supplementary sub-channel is divided into two half-mRB as shown in Figure 3, and a half mRB is composed of 3 subcarriers-by- 5 OFDM symbols. Since all HR-MSs should listen the supplementary channels, to obtain Tx. And Rx. Switching time, no signal is transmitted in the last OFDM symbol of supplementary channel (6th OFDM symbol). The details of supplementary channel design are discussed in [3].

# 4. References

- IEEE C802.16n-11/0051r2, “Dedicated resources allocation for direct communications in IEEE 802.16n,” March 2011.

- IEEE C802.16n-11/0131, “Synchronization channels for talk-around direct communications,” July 2011.

- IEEE C802.16n-11/0132, “Dedicated channels and supplementary channels for talk-around direct communications,” July 2011.

# 5. Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy1: Adapt the following change in Section 17.3.2.6 in the 802.16n AWD]

17.3.2.6 Talk-around Direct Communication

[note: This contribution provides text proposals for the following sections:

17.3.2.6.2 Physical layer

17.3.2.6.2.1 Frame structure

17.3.2.6.2.2 Physical structure

17.3.2.6.2.3 Control structure]

17.3.2.6.2.1 Frame structure

For talk-around direct communication, two types of infra-structure communication resources are dedicatedly assigned:

- Common Direct Mode Zone (CDMZ): set of PRUs with fixed size and positions which are commonly assigned to all cells (must be CRU)

- Cell Specific Direct Mode Zone (CSDMZ): additionally assigned direct-mode resource blocks independently assigned by each HR-BS (CRU or DRU) and information to receive cell specific direct mode region can be obtained from common direct mode zone

Figure xxx shows an example of talk-around direct communication resource allocation. In the figure, some part of uplink infra structure resources are assigned for common direct mode zone, and some part of uplink and downlink infra structure communication resources are assigned for cell specific direct mode zone. By assigning the same physical resources for common direct mode zone the following benefits can be obtained:

- Reduce the overhead of control channels to transmit allocation information of resources for direct-mode communication

- Reduce the computation complexity and power to obtain synchronization for direct communication, specially for the HR-MSs in the outside of HR-BS coverage

Figure xxx. An example of talk-around direct communication resource allocation

The highest four PRUs of uplink resources are assigned for Common Direct Mode Zone (CDMZ). For FFT size = 512, PRU 20, 21, 22, 23 are assigned for CDMZ, for FFT size = 1024, PRU 44, 45, 46, 47 are assigned for CDMZ, and for FFT size = 2048, PRU 92, 93, 94, 95 are assigned for CDMZ. The resources for Cell Specific Direct Mode Zone (CSDMZ) are determined by each HR-BS independently and the assignment information is transmitted in the CDMZ.

Logical frame of common direct mode zone is composed by collecting all resources of CDMZ in a superframe as shown in Figure yyy. In the example, there are three uplink subframes for each 5msec frame. Frame structure of cell specific direct mode zone is extension of common direct mode frame structure and the details are FFS.

Figure yyy. An example of common direct mode logical frame construction

There are three physical channels for talk-around direct communication:

- Synchronization Channel (Sync-CH)

- Dedicated Channel(Ded-CH)

- Supplementary Channel(Sup-CH)

The first subframe of the common direct mode frame is occupied by synchronization channel. All the HR-MSs receives the synchronization signal on the Synchronization channel except HR-MSs transmitting the synchronization signals. The HR-MSs are synchronized to the received synchronization signal if the signal timing has priority to HR-MS’s synchronization timing itself. The details of timing priority is FFS. Some HR-MSs sends the synchronization signal on the Synchronization channel at selected subframes. HR-MS selects its slots for sending synchronization timing in distributed way. The details of how to select is FFS. The synchronization channel is composed of two parts: synchronization channel preamble part and synchronization message part. The synchronization channel preamble part is used for acquiring time and frequency synchronization, and synchronization sequence part is used for transmitting some information including frame structure information, hop count, transmitter ID et. al. The detailed design of synchronization channel is described in 17.3.2.6.2.3.1

Resources excluding the first subframe assigned for synchronization channel are assigned for dedicated channels and supplementary channels. A Dedicated channel is a resource to send direct communication packets for two HR-MSs or a group of HR-MSs. An HR-MS sends a packet on one or more than one subchannels of dedicated channel and the other HR-MSs receives the packet on it. If two HR-MSs and a group of HR-MSs are involved, the transmissions are unicast and multicast, respectively. How to configure the resource of dedicated channel is discussed in described in 17.3.2.6.2.3.2. The resources for dedicated channel is divided into small size sub-blocks (mRB: mini-Resource Block), as shown in Figure zzz. One mRB is composed of 6 subcarriers-by-6 OFDM symbols, and there are 12 mRBs for each subframe (4PRU/ 1/3 PRU = 12). A dedicated subchannel is composed of a collection of 9 mRBs distributed across the entire frequency region in the slot.

Figure zzz. An example of common direct mode zone resource segmentation and construction of mRB

A supplementary sub-channel is one-to-one mapped with each dedicated sub-channel. By using the supplementary sub-channel, the following MAC messages, PHY signalings and short feedback messages related with the corresponding dedicated subchannel are transmitted.

- MAC messages: RTS, CTS for corresponding dedicated subchannel, MCS information, ranging response et. al.

- PHY signalings: periodic ranging sequence, sounding signal et. al.

- Short feedback messages: ACK, NACK, CQI, CSI, RI (rank information) et. al.

An mRB for supplementary sub-channel is divided into two half-mRB as shown in Figure zzz, and a half mRB is composed of 3 subcarriers-by- 5 OFDM symbols. Since all HR-MSs should listen the supplementary channels, to obtain Tx. And Rx. Switching time, no signal is transmitted in the last OFDM symbol of supplementary channel (6th OFDM symbol). The details of supplementary channel design are discussed in described in 17.3.2.6.2.3.3.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]