| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Dedicated Channel and Supplementary Channel for Talk-around Direct Communications |  |
| Date Submitted | 2011-07-19 |  |
| Source(s) | Young-Ho Jung, Jihoon Choi Korea Aerospace University Sungcheol Chang, Eunkyung Kim, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: jihoon@kau.ac.kr, yhjung@kau.ac.kr scchang@etri.re.kr |
| Re: | Call for Comments on the 802.16n AWD |  |
| Abstract | This provides AWD text proposals for dedicated channel and supplementary channel structure of talk-around direct communications |  |
| Purpose | To be discussed and adopted by 802.16 TGn |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Dedicated Channel and Supplementary Channel for Talk-around Direct Communications

Young-Ho Jung, Jihoon Choi,

Korea Aerospace University

Sungcheol Chang, Eunkyung Kim, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

# 1. Introductions

To support direct communication in IEEE 802.16n, some dedicated resources can be assigned as described in [1]. By dedicatedly reserving a part of communication resources for infra-structure communications, the reserved resources are used to provide multiple pairs of direct communication in OFDMA (orthogonal frequency division multiple access) manner with contention based transmission. We propose the frame structure for talk-around direct communication in [2], and there are three types of physical channels utilized for talk-around direct communications: synchronization channel, dedicated channel and supplementary channel.

In this contribution, we propose the details of dedicated channel and supplementary channel structure for talk-around direct communications in IEEE 802.16n, and the details of synchronization channel are described in [3].

## 2. Structure of Dedicated Channel

Figure 1. An example of common direct mode zone resource segmentation and construction of mRB

Logical frame of common direct mode zone is composed by collecting all resources of common direct mode zone in a superframe as shown in Figure 1. In the example, there are three uplink subframes for each 5msec frame. Resources excluding the first subframe assigned for synchronization channel are assigned for dedicated channels and supplementary channels. The resources for dedicated channel is divided into small size sub-blocks (mRB: mini-Resource Block), as shown in the figure. One mRB is composed of 6 subcarriers-by-6 OFDM symbols, and a dedicated sub-channel is composed of a collection of nine mRBs distributed across the entire frequency region in the same slot. There are two slots for dedicated sub-channels. The subframes in the first and second frame are assigned for slot 1, and the subframes in the third and fourth frame are assigned for slot 2. For the case the number of uplink subframes is two and five, the first subframe of the third frame is allocated for both slot 1 and slot 2 simultaneously. Table 1 summarizes the number of dedicated sub-channels for the various number of uplink subframes in a frame.

Table 1. The number of dedicated sub-channels according to the number of uplink subframes in a frame

| The number of uplink subframes in a 5msec frame | The number of dedicated sub-channel in the slot 1 | The number of dedicated sub-channel in the slot 2 | Total number of dedicated sub-channels |
| --- | --- | --- | --- |
| 2 | 3 | 3 | 6 |
| 3 | 5 | 6 | 11 |
| 4 | 8 | 9 | 17 |
| 5 | 11 | 11 | 22 |

Figure 2 ~ Figure 5 show the frame structure of talk-around direct communication channels. In the figures, each block indicates mRB, and the number inside each mRB is the index of the dedicated sub-channel. For example, the dedicated sub-channel 1 is composed of nine mRBs in subframe in slot 1.

Figure 2. Frame structure of talk-around direct communication when the number of uplink subframe per 5msec frame is two. In the figure, each block indicates mRB, and the number inside each mRB is the index of the dedicated sub-channel. For example, the dedicated sub-channel 1 is composed of nine mRBs in subframe two and four (in blue color). Ci is the i-th supplementary sub-channel corresponding to the i-th dedicated sub-channel

Figure 3. Frame structure of talk-around direct communication when the number of uplink subframe per 5msec frame is three. In the figure, each block indicates mRB, and the number inside each mRB is the index of the dedicated sub-channel. For example, the dedicated sub-channel 1 is composed of nine mRBs in subframe two, three, five and six (in blue color). Ci is the i-th supplementary sub-channel corresponding to the i-th dedicated sub-channel

Figure 4. Frame structure of talk-around direct communication when the number of uplink subframe per 5msec frame is four. In the figure, each block indicates mRB, and the number inside each mRB is the index of the dedicated sub-channel. For example, the dedicated sub-channel 1 is composed of nine mRBs in subframe two, three, four, six, seven and eight (in blue color). Ci is the i-th supplementary sub-channel corresponding to the i-th dedicated sub-channel

Figure 5. Frame structure of talk-around direct communication when the number of uplink subframe per 5msec frame is five. In the figure, each block indicates mRB, and the numbser inside each mRB is the index of the dedicated sub-channel. For example, the dedicated sub-channel 1 is composed of nine mRBs in subframe two, three, four, five, seven, eight, nine and ten (in blue color). Ci is the i-th supplementary sub-channel corresponding to the i-th dedicated sub-channel

## 2. Structure of supplementary Channel

There are two subframes to transmit supplementary channel in a super frame. One subframe for supplementary channel is located in the first uplink subframe of the second frame, and the other subframe for supplementary channel is located in the first uplink subframe of the fourth frame. A supplementary sub-channel is one-to-one mapped with each dedicated sub-channel. By using the supplementary sub-channel, the following MAC messages, PHY signalings and short feedback messages related with the corresponding dedicated sub-channel are transmitted.

- MAC messages: RTS, CTS, MCS information, ranging response et. al.

- PHY signalings: periodic ranging sequence, sounding signal et. al.

- Short feedback messages: ACK, NACK, CQI, CSI, RI (rank information) et. al.

As shown in Figure 2 ~ Figure 5, Ci is the i-th supplementary sub-channel corresponding to the i-th dedicated sub-channel. A supplementary sub-channel is composed of two sub-blocks distributed in the frequency domain. Since all HR-MSs should listen the supplementary channels, to obtain Tx. And Rx. Switching time, no signal is transmitted in the last OFDM symbol of supplementary channel (6-th OFDM symbol). The sub-block of supplementary sub-channel is composed of 3 subcarriers-by- 5 OFDM symbols. The supplementary sub-channels corresponding to the dedicated sub-channels in slot 1 are located in slot 2, and the supplementary sub-channels corresponding to the dedicated sub-channels in slot 2 are located in slot 1. By assigning a dedicated sub-channel and the corresponding supplementary sub-channel in a cross way, the setup time of communication link and the retransmission latency can be minimized. For example, if an HR-MS transmits a packet by using the dedicated sub-channel 1 in slot 1, because the corresponding supplementary sub-channel is located in slot 2, the receiving HR-MS can transmit ACK/NACK signal by using the supplementary sub-channel in the same superframe, and the retransmission packet can be transmitted in the next superframe.

# 4. References

- IEEE C802.16n-11/0051r2, “Dedicated resources allocation for direct communications in IEEE 802.16n,” March 2011.

- IEEE C802.16n-11/0130, “Frame structure for talk-around direct communications,” July 2011.

- IEEE C802.16n-11/0131, “Synchronization channels for talk-around direct communications,” July 2011.

# 5. Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy1: Adapt the following change in Section 17.3.2.6 in the 802.16n AWD]

17.3.2.6 Talk-around Direct Communication

[note: This contribution provides text proposals for the following sections:

17.3.2.6.2 Physical layer

17.3.2.6.2.1 Frame structure

17.3.2.6.2.2 Physical structure

17.3.2.6.2.3 Control structure]

17.3.2.6.2.2.3.2 Dedicated Channel

Resources excluding the first subframe assigned for synchronization channel are assigned for dedicated channels and supplementary channels. A dedicated sub-channel is composed of a collection of nine mRBs distributed across the entire frequency region in the same slot. There are two slots for dedicated sub-channels. The subframes in the first and second frame are assigned for slot 1, and the subframes in the third and fourth frame are assigned for slot 2. For the case the number of uplink subframes is two and five, the first subframe of the third frame is allocated for both slot 1 and slot 2 simultaneously. Table xxx summarizes the number of dedicated sub-channels for the various number of uplink subframes in a frame.

Table xxx. The number of dedicated sub-channels according to the number of uplink subframes in a frame

| The number of uplink subframes in a 5msec frame | The number of dedicated sub-channel in the slot 1 | The number of dedicated sub-channel in the slot 2 | Total number of dedicated sub-channels |
| --- | --- | --- | --- |
| 2 | 3 | 3 | 6 |
| 3 | 5 | 6 | 11 |
| 4 | 8 | 9 | 17 |
| 5 | 11 | 11 | 22 |

Figure xx1 ~ Figure xx4 show the frame structure of talk-around direct communication channels. In the figures, each block indicates mRB, and the number inside each mRB is the index of the dedicated sub-channel. For example, the dedicated sub-channel 1 is composed of nine mRBs in subframe in slot 1.

Figure xx1. Frame structure of talk-around direct communication when the number of uplink subframe per 5msec frame is two. In the figure, each block indicates mRB, and the number inside each mRB is the index of the dedicated sub-channel. For example, the dedicated sub-channel 1 is composed of nine mRBs in subframe two and four (in blue color). Ci is the i-th supplementary sub-channel corresponding to the i-th dedicated sub-channel

Figure xx2. Frame structure of talk-around direct communication when the number of uplink subframe per 5msec frame is three. In the figure, each block indicates mRB, and the number inside each mRB is the index of the dedicated sub-channel. For example, the dedicated sub-channel 1 is composed of nine mRBs in subframe two, three, five and six (in blue color). Ci is the i-th supplementary sub-channel corresponding to the i-th dedicated sub-channel

Figure xx3. Frame structure of talk-around direct communication when the number of uplink subframe per 5msec frame is four. In the figure, each block indicates mRB, and the number inside each mRB is the index of the dedicated sub-channel. For example, the dedicated sub-channel 1 is composed of nine mRBs in subframe two, three, four, six, seven and eight (in blue color). Ci is the i-th supplementary sub-channel corresponding to the i-th dedicated sub-channel

Figure xx4. Frame structure of talk-around direct communication when the number of uplink subframe per 5msec frame is five. In the figure, each block indicates mRB, and the number inside each mRB is the index of the dedicated sub-channel. For example, the dedicated sub-channel 1 is composed of nine mRBs in subframe two, three, four, five, seven, eight, nine and ten (in blue color). Ci is the i-th supplementary sub-channel corresponding to the i-th dedicated sub-channel

17.3.2.6.2.2.3.3 Supplementary Channel

There are two subframes to transmit supplementary channel in a super frame. One subframe for supplementary channel is located in the first uplink subframe of the second frame, and the other subframe for supplementary channel is located in the first uplink subframe of the fourth frame. A supplementary sub-channel is one-to-one mapped with each dedicated sub-channel. By using the supplementary sub-channel, the following MAC messages, PHY signalings and short feedback messages related with the corresponding dedicated sub-channel are transmitted.

- MAC messages: RTS, CTS, MCS information, ranging response et. al.

- PHY signalings: periodic ranging sequence, sounding signal et. al.

- Short feedback messages: ACK, NACK, CQI, CSI, RI (rank information) et. al.

As shown in Figure 2 ~ Figure 5, Ci is the i-th supplementary sub-channel corresponding to the i-th dedicated sub-channel. A supplementary sub-channel is composed of two sub-blocks distributed in the frequency domain. Since all HR-MSs should listen the supplementary channels, to obtain Tx. And Rx. Switching time, no signal is transmitted in the last OFDM symbol of supplementary channel (6-th OFDM symbol). The sub-block of supplementary sub-channel is composed of 3 subcarriers-by- 5 OFDM symbols. The supplementary sub-channels corresponding to the dedicated sub-channels in slot 1 are located in slot 2, and the supplementary sub-channels corresponding to the dedicated sub-channels in slot 2 are located in slot 1. By assigning a dedicated sub-channel and the corresponding supplementary sub-channel in a cross way, the setup time of communication link and the retransmission latency can be minimized. For example, if an HR-MS transmits a packet by using the dedicated sub-channel 1 in slot 1, because the corresponding supplementary sub-channel is located in slot 2, the receiving HR-MS can transmit ACK/NACK signal by using the supplementary sub-channel in the same superframe, and the retransmission packet can be transmitted in the next superframe.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]