| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Procedures for HR-MS mode change to HR-BS initiated by HR-MS itself |  |
| Date Submitted | 2011-07-11 |  |
| Source(s) | Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: woniks@etri.re.kr ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-11/0008,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Multicast connection management clarification |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Procedures for HR-MS mode change to HR-BS initiated by HR-MS itself

Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

# Introductions

IEEE 802.16n AWD describes BS function for HR-MS in Section 17.3.1.3. According to the current AWD, a mode switch of an HR-MS to HR-BS may be initiated by either the HR-MS itself or a super-ordinate HR-BS of the HR-MS. However, the current AWD doesn’t provide detailed description on mode change procedures.

In this contribution, we propose the more detailed description on the procedure for HR-MS mode change initiated by the HR-MS itself.

If a superordinate HR-BS is break down without notice, subordinate HR-MSs that are capable of role change to HR-BS may initiate the mode change to HR-BS at the same time. In this case, adjacent HR-MSs initiating the mode change to HR-BS may interfere with one another because they start to transmit DL preambles simultaneously.

For solving the problem, an HR-MS trying to perform a role change to HR-BS is needed to start the mode change after a random backoff timer is expired. The HR-MS randomly selects a value for random backoff timer from a backoff window. The size of the backoff window may be adjusted based on the some status information of the HR-MS such as battery level. How to perform random backoff is TBD.

After completion of mode switch, the HR-MS acting as HR-BS may request mode change to one of its subordinate HR-MSs in order to hand HR-BS role over. In this case, the new HR-BS follows the procedure for HR-MS mode change directed by a superordinate HR-BS.

# References

- IEEE 802.16n-10/0048r1, “802.16n System Requirements Document including SARM annex,” March 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16m-09/0034r3, “IEEE 802.16m System Description Document (SDD),” June 2010.

- IEEE 802.16n-11/0009r1, “802.16n Amendment Working Draft,” May 2011.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Add the following text into Section 17.2.1 in the 802.16n AWD]

## 17.2.1 Multi-mode operation

[Insert the text in 17.2.1.3 as follows:]

###### 17.2.1.3 Base station function for HR-MS

An HR-MS may operate as an HR-BS to provide connectivity for itself and other HR-MSs. During basic capability negotiation within network entry, an HR-MS that is capable of role change to HR-BS shall report such capability to the super-ordinate HR-BS/HR-RS.

While operating as an HR-BS, the station may maintain certain HR-MS functionalities. A mode switch to HR-BS may be initiated by the HR-MS itself or may be directed by the superordinate HR-BS of the HR-MS.

If the HR-MS acting as HR-BS does not connect to the backbone network, it follows standalone network procedure defined in 17.2.4.

###### 17.2.1.3.x Mode change to HR-BS initiated by HR-MS itself

In the case of a mode switch to HR-BS initiated by an HR-MS itself, an HR-MS may initiate a mode switch to HR-BS after a random backoff timer to avoid potential collision among adjacent HR-MSs trying to perform a mode switch to HR-BS at the same time. How to perform random backoff is TBD.

After completion of mode switch, the HR-MS acting as HR-BS may request mode change to one of its subordinate HR-MSs in order to hand HR-BS role over. In this case, the new HR-BS follows the procedure for HR-MS mode change directed by a superordinate HR-BS as described in 17.2.1.3.xx.