| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Procedures for HR-MS mode change to HR-BS initiated by HR-MS itself |  |
| Date Submitted | 2011-07-19 |  |
| Source(s) | Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI Liru Lu (Alina), Ming-Tuo Zhou, Xin Zhang, Vinh Dien Hoang, Masayuki Oodo, Hiroshi Harada NICT | E-mail: woniks@etri.re.kr ekkim@etri.re.kr scchang@etri.re.kr liru@nict.com.sg |
| Re: | “IEEE 802.16n-11/0008,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Multicast connection management clarification |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Procedures for HR-MS mode change to HR-BS initiated by HR-MS itself

Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

Liru Lu (Alina), Ming-Tuo Zhou, Xin Zhang, Vinh Dien Hoang, Masayuki Oodo, Hiroshi Harada

NICT

# Introductions

IEEE 802.16n AWD describes BS function for HR-MS in Section 17.3.1.3. According to the current AWD, a mode switch of an HR-MS to HR-BS may be initiated by either the HR-MS itself or a super-ordinate HR-BS of the HR-MS. However, the current AWD doesn’t provide detailed description on mode change procedures.

In this contribution, we propose more detailed description on the procedures for HR-MS mode change initiated by the HR-MS itself aw wells as for HR-MS mode change directed by a superordinate HR-BS of the HR-MS respectively.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Add the following text into Section 17.3.1 in the 802.16n AWD]

## 17.3.1 Multi-mode operation

[Insert the text in 17.3.1.3 as follows:]

###### 17.3.1.3 Base station function for HR-MS

An HR-MS may operate as an HR-BS to provide connectivity for itself and other HR-MSs. During basic capability negotiation within network entry, an HR-MS that is capable of role change to HR-BS shall report such capability to the super-ordinate HR-BS/HR-RS.

While operating as an HR-BS, the station may maintain certain HR-MS functionalities. A mode switch to HR-BS may be initiated by the HR-MS itself or may be directed by the superordinate HR-BS of the HR-MS.

The HR-MS may start operating as an HR-BS in a Proactive operation or a Reactive operation. For proactive operation, the mode switch is directed by the superordinate HR-BS of the HR-MS; In reactive operation, the mode switch is initiated by the HR-MS itself.

###### 17.3.1.3.x Proactive Operation

A superordinate HR-BS may select a target HR-MS among its subordinate HR-MSs which are capable of role changing to HR-BS, according to the measured signal power at HR-BS and/or subordinate HR-MS’ status information such as the battery level. The subordinate HR-MS capable of role changing to HR-BS may report its status information to the superordinate HR-BS via MM-STAT-REP MAC control message and/or AMS Battery Level Report header as described in 16.2.2.1.3.5. The triggering condition for reporting status information may be configured by the superordinate HR-BS.

After selecting the target HR-MS, the superordinate HR-BS requests the target HR-MS to change its mode to HR-BS by exchanging HRBS-REQ/RSP message. If the target HR-MS accepts the request from the superordinate HR-BS to change the mode to HR-BS, the superordinate HR-BS may transmit HRBS-CONFIG-CMD message to request the target HR-MS to set the configuration parameters and the trigger conditions for operating as HR-BS.

###### 17.3.1.3.xx Reactive Operation

The HR-MSs which are capable of role changing to HR-BS may contend for operating at BS mode when the superordinate HR-BS fails. The HR-MSs may initiate a mode switch to HR-BS after expiration of a random backoff timer to avoid potential collision among adjacent HR-MSs trying to perform a mode switch to HR-BS at the same time.

After completion of mode switch, the HR-MS acting as HR-BS may request mode change to one of its subordinate HR-MSs in order to hand HR-BS role over. In this case, it follows the procedure for Proactive operation as described in 17.3.1.3.x.

[Remedy2: Add the following text into Section 17.2.1 in the 802.16n AWD]

## 17.2.1 Multi-mode operation

[Insert the text in 17.2.1.3 as follows:]

###### 17.2.1.3 Base station function for HR-MS

An HR-MS may operate as an HR-BS to provide connectivity for itself and other HR-MSs. During basic capability negotiation within network entry, an HR-MS that is capable of role change to HR-BS shall report such capability to the super-ordinate HR-BS/HR-RS.

While operating as an HR-BS, the station may maintain certain HR-MS functionalities. A mode switch to HR-BS may be initiated by the HR-MS itself or may be directed by the superordinate HR-BS of the HR-MS.

The HR-MS may start operating as an HR-BS in a Proactive operation or a Reactive operation. For proactive operation, the mode switch is directed by the superordinate HR-BS of the HR-MS; In reactive operation, the mode switch is initiated by the HR-MS itself.

###### 17.2.1.3.x Proactive Operation

A superordinate HR-BS may select a target HR-MS among its subordinate HR-MSs which are capable of role changing to HR-BS, according to the measured signal power at HR-BS and/or subordinate HR-MS’ status information such as the battery level. The subordinate HR-MS capable of role changing to HR-BS may report its status information to the superordinate HR-BS via MM-STAT-REP MAC control message. The triggering condition for reporting status information may be configured by the superordinate HR-BS.

After selecting the target HR-MS, the superordinate HR-BS requests the target HR-MS to change its mode to HR-BS by exchanging HRBS-REQ/RSP message. If the target HR-MS accepts the request from the superordinate HR-BS to change the mode to HR-BS, the superordinate HR-BS may transmit HRBS-CONFIG-CMD message to request the target HR-MS to set the configuration parameters and the trigger conditions for operating as HR-BS.

###### 17.2.1.3.xx Reactive Operation

The HR-MSs which are capable of role changing to HR-BS may contend for operating at BS mode when the superordinate HR-BS fails. The HR-MSs may initiate a mode switch to HR-BS after expiration of a random backoff timer to avoid potential collision among adjacent HR-MSs trying to perform a mode switch to HR-BS at the same time.

After completion of mode switch, the HR-MS acting as HR-BS may request mode change to one of its subordinate HR-MSs in order to hand HR-BS role over. In this case, it follows the procedure for Proactive operation as described in 17.2.1.3.x.