| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Procedures for HR-MS mode change to HR-BS directed by superordinate HR-BS |  |
| Date Submitted | 2011-07-11 |  |
| Source(s) | Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Jaesun Cha, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: woniks@etri.re.kr ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-11/0008,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Multicast connection management clarification |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Procedures for HR-MS mode change to HR-BS directed by superordinate HR-BS

Won-Ik Kim, Eunkyung Kim, Sungkyung Kim, Sungcheol Chang, Jaesun Cha, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

# Introductions

IEEE 802.16n AWD describes BS function for HR-MS in Section 17.3.1.3. According to the current AWD, a mode switch of an HR-MS to HR-BS may be initiated by either the HR-MS itself or a super-ordinate HR-BS of the HR-MS. However, the current AWD doesn’t provide detailed description on mode change procedures.

In this contribution, we propose the more detailed description on the procedure for HR-MS mode change directed by a superordinate HR-BS of the HR-MS.

In the case that a superordinate HR-BS is responsible for managing mode change of its subordinate HR-MSs, the superordinate HR-BS shall select a target HR-MS which can operates optimally as an HR-BS before requesting one of its subordinate HR-MS to start the mode change to HR-BS. Criteria for selection of a target HR-MS may include basic capabilities of HR-MSs, interference to neighbor stations and/or status information of HR-MSs such as battery level, etc. For this, all the HR-MSs that are capable of role change to HR-BS may report their status information to the superordinate HR-BS via MM-STAT-REP MAC control message and/or AMS Battery Level Report header defined in IEEE 802.16m. The triggering condition for status report may be transmitted by the superordinate HR-BS.

After selecting a target HR-MS, the superordinate HR-BS may request the selected target HR-MS to change its mode to HR-BS and/or a previous HR-MS acting as HR-BS to terminate HR-BS role by exchanging HRBS-REQ/RSP message. And, it may transmit HRBS-CONFIG-CMD message to request the target HR-MS to set the configuration parameters for operating as HR-BS. The format of MAC control message exchanged for mode change operation is TBD.

# References

- IEEE 802.16n-10/0048r1, “802.16n System Requirements Document including SARM annex,” March 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16m-09/0034r3, “IEEE 802.16m System Description Document (SDD),” June 2010.

- IEEE 802.16n-11/0009r1, “802.16n Amendment Working Draft,” May 2011.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Add the following text into Section 17.2.1 in the 802.16n AWD]

## 17.2.1 Multi-mode operation

[Insert the text in 17.2.1.3 as follows:]

###### 17.2.1.3 Base station function for HR-MS

An HR-MS may operate as an HR-BS to provide connectivity for itself and other HR-MSs. During basic capability negotiation within network entry, an HR-MS that is capable of role change to HR-BS shall report such capability to the super-ordinate HR-BS/HR-RS.

While operating as an HR-BS, the station may maintain certain HR-MS functionalities. A mode switch to HR-BS may be initiated by the HR-MS itself or may be directed by the superordinate HR-BS of the HR-MS.

###### 17.2.1.3.xx Mode change to HR-BS directed by superordinate HR-BS

Before a superordinate HR-BS selects a target HR-MS which will operate as HR-BS, all the HR-MSs that are capable of role change to HR-BS may report their status information to the superordinate HR-BS via MM-STAT-REP MAC control message and/or AMS Battery Level Report header as described in 16.2.2.1.3.5. The triggering condition for status report may be transmitted by the superordinate HR-BS.

Based on the status report from the HR-MSs, the superordinate HR-BS may select a target HR-MS. It may request the selected target HR-MS to change its mode to HR-BS and/or a previous HR-MS acting as HR-BS to terminate HR-BS role by exchanging HRBS-REQ/RSP message. If the target HR-MS accepts the request from the superordinate HR-BS to change the mode to HR-BS, the superordinate HR-BS may transmit HRBS-CONFIG-CMD message to request the target HR-MS to set the configuration parameters for operating as HR-BS.