| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Security for Multicast in IEEE 802.16n |  |
| Date Submitted | 2011-07-11 |  |
| Source(s) | Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-11/0008,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Security supporting multicast in IEEE 802.16n |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Security for Multicast in IEEE 802.16n

Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

# Introductions

IEEE 802.16n AWD [5] describes optimized MAC protocol for unicast and multicast transmission based on the 802.16n SRD [1].

This document provides the detail description of security operation for multicast operation for the 802.16n focusing on the multicast key management.

# Key management in WirelessMAN-OFDMA AAI

WirelessMAN-OFDMA Advanced System uses the PKM protocol to achieve:

• Transparent exchange of authentication and authorization messages

• Key agreement

• Security material exchange

PKM protocol provides mutual authentication and establishes shared secret between AMS and ABS. The shared secret is then used to exchange or drive other keying material. This two-tiered mechanism allows the frequent traffic key refreshing without incurring the overhead of computation intensive operations.

Figure 1 and Figure 2 show the process to calculate the AK yielding an MSK and the unicast key hierarchy starting from AK based on PMKv3, respectively.

As shown Figure 1 and Figure 2, all PKMv3 keys (excluding MSK) are derived directly/indirectly from the MSK.

Figure 1 – AK from PMK (Pairwise Master Key (PMK) is derived from the MSK and this PMK is used to derive the Authorization Key (AK).)

Figure 2 – CMAC key and TEK derivation from AK (AK is used to derive Traffic Encryption Key (TEK) and Cipher-based Message Authentication Code (CMAC) key.)

# Multicast Key Management in IEEE 802.16n

Similar to unicast key management, multicast traffic can be encrypted using multicast specific key management based on PMKv3 as described in Figure 3. As shown Figure 3, Multicast CMAC (MCMAC) key and Multicast TEK (MTEK) are derived from Multicast AK (MAK). MAK is a shared key among an HR-BS and a group of HR-MSs in an HR multicast group and the MAK is encrypted and transmitted to each HR-MS using unicast security key.

Figure 3 – MCMAC Key and MTEK derivation from MAK

# Multicast Security Association (MSA)

In addition to sharing between ABS and AMS for unicast, some shared security association (i.e., Multicast Security Association; MSA) is proposed. MSA is an SA for the multicast transport/control flow and it provides keying material. Security key related to parameter to support multicast and the context is secured till the key expires.

# References

- IEEE 802.16n-10/0048r1, “802.16n System Requirements Document including SARM annex,” March 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16m-09/0034r3, “IEEE 802.16m System Description Document (SDD),” June 2010.

- IEEE 802.16n-11/0009r1, “802.16n Amendment Working Draft,” May 2011.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Add the following text in Section 16.2.5.2.2 Security in the 802.16n AWD]

[Change section 16.2.5.2.2 as indicated:]

###### 16.2.5.2.2 SA Management

A security association (SA) is the set of information required for secure communication between ABS and AMS. SA is shared between ABS and its client AMS across the AAI network. SA is identified using an SA identifier (SAID). The SA is applied to the respective unicast flows. AAI supports unicast static SA only and SAs are mapped one-by-one to cryptographic methods. (See Table 764.)

SA is used to provide keying material for unicast transport/control flows. Once an SA is mapped to an unicast transport flow, the SA is applied to all the data exchanged within the unicast transport flow. Multiple flows may be mapped to the same SA. The indication to the receiver that the MAC PDU is encrypted or not is indicated by the FID 0x1 and 0x0 in AGMH respectively for unicast control flows, and indicated by the SA that is associated to FID in AGMH and SPMH for unicast transport flows.

The Flow ID in the AGMH is used to indicate whether the PDU contains a control message encrypted based on security level. Whether each control message is encrypted or not is decided based on the security level with which the message is associated (see Table 683).

If authorization is performed successfully, SAID 0x01 is applied to flows for confidentiality and integrity, and SAID 0x02 for confidentiality only. In addition, for secure multicast service, SAID 0x03 is applied to flows for confidentiality and integrity. SAID 0x01 shall be applied to control flows as defined in Table 680. However, SAID 0x02 can be applied to transport flows only if the AMS and ABS decide to create an unprotected transport flow, the Null SAID (i.e., SAID 0x00) is used as the target SAID. SAID 0x03 can be applied to secure multicast transport flow (see Table 764.)

Table 764 – SA mapping with protection level

| Name | Name of SA | Characteristics | Usage |
| --- | --- | --- | --- |
| 0x00 | Null SA | Neither confidentiality nor integrity protection | For non-protected transport flow. |
| 0x01 | Primary SA | Confidentiality & integrity protection(i.e., AES-CCM mode is applied) | Encryption for unicast control/ transport flow. |
| 0x02 |  | Confidentiality protection only(i.e., AES-CTR mode is applied) | Encryption for unicast transport flow |
| 0x03 | Multicast SA | Confidentiality & integrity protection | Encryption for multicast transport flow |
| 0x030x04-0xFF |  | Reserved |  |

Using PKM protocol, AMS shares the SAs’ keying material with ABS. An SA contains keying material that is used to protect unicast flows (see SA context in 16.2.5.4.4).

16.2.5.2.2.1 Mapping of flows to SAs

The following rules for mapping flows to SAs apply:

a) The unicast transport flows shall be mapped to an SA.

b) The multicast or broadcast transport flows shall be mapped to Null SA.

c) The encrypted unicast control flows shall be mapped to the Primary SA.

d) The non-encrypted unicast control flows shall not be mapped to any SA.

e) The broadcast control flows shall not be mapped to any SA.

f) The secure multicast transport flows shall be mapped to any multicast SA.

The actual mapping is achieved by including the SAID of an SA in the DSA-xxx messages together with the FID.

Control messages which the Primary SA is applied to are predetermined according to the control message protection level depending on each control message type and its usage. Even if non-encrypted unicast control flows shall not be mapped to any SA, CMAC-based integrity protection can be applied per control message according the control message protection level (see 16.2.5.3.3).

[Remedy2: Add the following text into Section 17.3.10.2 Security in the 802.16n AWD]

17.3.10.2 Security Procedure for Multicast Operation

PKMv3 as described in 16.2.5.2 provides HR-stations with strong protection from theft of service by encrypting connections between HR-MSs and HR-BSs.

PKMv3 also shall provide HR-stations with strong protection from theft of service by encrypting multicast connections between HR-MSs and HR-BSs, as defined in this subsection.

If a DL multicast connection is to be encrypted, each HR-MS participating in the connection shall have an additional security association (SA) (i.e., multicast SA), allowing that connection to be encrypted using keys that are independent of those used for other encrypted transmissions between HR-MSs and the HR-BS.

Similar to unicast key management, multicast traffic can be encrypted using multicast specific key management based on PMKv3 as described in Figure aaa. Multicast CMAC (MCMAC) key and Multicast TEK (MTEK) are derived from Multicast AK (MAK). MAK is a shared key among an HR-BS and a group of HR-MSs in an HR multicast group and the MAK is encrypted and transmitted to each HR-MS using unicast security key.

Figure aaa – MCMAC Key and MTEK derivation from MAK

Shared security association (i.e., Multicast Security Association; MSA) is an SA for the multicast transport/control flow and it provides keying material. Security key related to parameter to support multicast and the context is secured till the key expires.

###### 17.3.10.2.1 Security context for multicast communication

The multicast security context is a set of parameters linked to a key in each hierarchy that defines the scope while the key usage is considered to be secure.

Examples of these parameters are key lifetime and counters ensuring the same encryption will not be used more than once. When the context of the key expires, a new key should be obtained to continue working. The purpose of this sub clause is to define the context that belongs to each key, how it is obtained and the scope of its usage.

17.3.10.2.1.1 MAK context

The MAK context includes all parameters associated with the MAK. This context is created whenever a new MAK is derived.

This context shall be deleted whenever the MAK is no longer valid or used.

The MAK context is described in Table bbb.

Table bbb – The MAK context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| MAK | 160 | Shared by HR-MSs in a multicast group |
| MAK Lifetime | 32 | MAK Lifetime |
| MAKID | 64 | Identifies the authorization key. |
| MAK_COUNT | 16 | A value used to derive the MCMAC key and MTEK |
| MCMAC_KEY_D | 128 | The key which is used for signing DL MAC control messages. |
| MCMAC_PN_D | 24 | Used to avoid DL replay attack on the control connection before this expires, reauthorization is needed. The initial value of MCMAC_PN_D is zero and the value of MCMAC_PN_D is reset to zero whenever MAK_COUNT is increased. |
| Next available counter_MTEK | 16 | The counter value to be used in next MTEK derivation, after derivation this is increased by 1. |

17.3.10.2.1.2 MSA context

The MSA context is the set of parameters managed by each MSA in order to ensure MTEK management and usage in secure way.

The MSA context holds MTEK context and additional information that belongs to the MSA itself.

17.3.10.2.1.2.1MTEK context

The MTEK context includes all relevant parameters of a single MTEK and is described in Table ccc.

Table ccc – The MTEK context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| MTEK | 128 | Key used for encryption or decryption of MAC PDUs from FIDs associated with the corresponding MSA |
| MEKS | 2 | Encryption key sequence number |
| COUNTER_MTEK | 16 | The counter value used to derive this MTEK |
| MTEK lifetime | 32 | MTEK lifetime |
| MTEK_PN_D | 22 | The PN used for encrypting DL packets. After each MAC PDU transmission, the value shall be increased by 1. (0x000000-0x1FFFFF) |
| PN Window Size | As negotiated in key agreement | The receiver shall track the PNs received inside PN window |

17.3.10.2.1.2.2 MSA context

The MSA context is described in Table ddd.

Table ddd – The MSA context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| MSAID | 8 | The identifier of this MSA, which describes the applied en/ decryption method and MTEK contexts. |
| MTEKDLE context | Sizeof(MTEK Context) | MTEK context used for downlink encryption and decryption. |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]